3GPP TSG-SA WG6 Meeting #68	S6-253266

Göteborg, Sweden, 25th – 29th August 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	Updates to Solution#6 on AIMLE Service Performance Information Collection

Spec:	3GPP TR 23.700-83 v0.2.0

Agenda item:	9.4

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

Solution #6 introduces enhancements to performance monitoring. In this solution, the ML model performance monitoring procedure is enhanced by adding IEs to monitoring request. This pCR proposes to update the solution #6 by including the option for the AIMLE server to collect AIMLE service performance information via A-DCCF.

2. Reason for Change

To improve solution #6 by including the option of the AIMLE server to collect AIMLE service performance information via A-DCCF.

3. Conclusions

This paper proposes to update the solution #6 on AIMLE Service performance information collection.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

* * * First Change * * * *

## 7.6	Solution #6: Enhancements to performance monitoring

### 7.6.1	Solution description

#### 7.6.1.1	Enhancements to ML model performance monitoring

The ML model performance monitoring procedure as specified in clause 8.22 of 3GPP TS 23.482 [2] is enhanced as follows (new text in bold italics):

### 8.22.2 Procedure Pre-conditions: 1. One or more AIMLE services (at the AIMLE server or clients) using the given ML model are ongoing. Figure 8.22.2-1: ML model performance monitoring 1. The VAL server sends an ML model performance monitoring subscription request to AIMLE server, requesting to assist in monitoring the ML model performance. This request consists of ML model information and optionally the AIML service information, etc. The request also includes baseline validation dataset information which is the baseline reference for evaluation of HFL model after each round of parameter aggregation. 2. The AIMLE server checks whether the VAL server is authorized to perform the ML model performance monitoring request. 3. If the VAL server is authorized, AIMLE server returns the success response, otherwise a failure response indication the reason for failure. 4. The AIMLE server identifies the AIMLE services which are utilizing the requested ML model. Such AIMLE services can be an ML model training service or an HFL service at the AIMLE server or clients. The identification of the AIMLE services may be performed via fetching ML model information from the ML repository using ML model management procedure as in clause 8.11.3. Then, the AIMLE server then starts monitoring the AIMLE service performance (e.g. accuracy, a KPI or QoS metric related to the AIML operation). This step includes receiving information from one or more AIMLE clients performing an operation based on the target ML model (e.g., based on step 7 of procedure in clause 8.12.2 or step 6 of procedure in clause 8.15.2), with an expected or experienced deviation of the required performance of the AIMLE service. In order to get the information, the request is enhanced with following IEs: Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. Aggregated AI/ML model parameters; M; Information about the AI/ML model and model parameters for use in HFL training.. Performance Metrics; M; The performance metrics that are used to evaluate the performance of model (e.g. accuracy, latency). each AIMLE client evaluates the performance of the received aggregated global model on the local data based on the received Aggregated AI/ML model parameters and Performance metrics. Each AIMLE client sends the performance report of aggregated global model on the local data as the response message to AILMLE Server. The performance report response message includes information as described in the following table. The response is enhanced with following IEss: Information element; Status; Description. AIMLE Client ID; M; The identifier of the AIMLE Client.. AIMLE service ID; M; The service identifier for the AIMLE HFL training operation.. Performance records; M; The evaluation performance records (e.g. on accuracy, on latency) of received aggregated global model from AIMLE Server on local data.. 5. The AIMLE server detects an expected ML model degradation (e.g., model drift, data drift) based on the deviation of the performance of the AIMLE service as indicated in step 4 . 6. The AIMLE server based on the expected ML model degradation, it may also indicate and execute a trigger action to ensure meeting the AIMLE service requirement. Further, AIMLE Server sends a configuration request to increase training collaboration to only AIMLE Clients that have degraded performance records. Such trigger action may be either an adaptation of the AIMLE service, such as training of a new ML model for the AIMLE by the same or a different AIMLE client, or re-training of the ML model by the same or different AIMLE client; or termination of the AIMLE service and initiating a new AIMLE service with a new ML model. NOTE: If this action involves re-selecting an AIMLE client for the AIMLE service, this is based on the procedure defined in clause 8.9. 7. The AIMLE server notifies the VAL server on the expected ML model degradation and if requested the triggered adaptation of the AIMLE service. Alternatively, the AIMLE server sends requests to A-DCCF for collecting AIMLE service performance information. The procedure for AIMLE Server to request data collection via A-DCCF are as specified in clause 8.12 of 3GPP TS 23.436 [4].

# *** End of Changes ***