3GPP TSG-SA WG6 Meeting #68	S6-253270

Göteborg, Sweden, 25th – 29th August 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	New Key Issue on Enhance Existing Application Enablement Layer Services to Support Energy Saving

Spec:	3GPP 23.700-44 v0.0.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

This pCR proposes new Key Issue for FS_EnergySys_Ph2_APP.

2. Reason for Change

The New SID for study on Application Enablement to support Energy Saving Phase 2 (SP-250871) was approved in the SA#108 meeting. According to the approved SID, the objective of the study is to identify and specify enhancements at application enablement layer to enable support for energy saving, which include

- Study functional enhancements and possible architectural enhancements to application enablers/services to support energy saving, including:

  - potential enhancements to NSCE services for usage/configuration of dedicated network slice to verticals.

  - potential enhancements to the application enablement layer (e.g. SEALDD, AIMLE, LM, ADAE) to support energy saving for the network resources used by applications.

- Identify potential solutions, including the information flows and application enablement APIs satisfying the architectural requirements and enhancements identified in bullet#1.

NOTE:	The study of application enablement for energy saving will consider the underlying 3GPP system’s existing capabilities such as leveraging the energy consumption information provided by the Core Network (e.g. EIF defined in TS 23.501), network slicing management and energy measurement information exposed by Management system (e.g. introduced in TS 28.552 and TS 28.554).

Therefore, a new Key Issue is proposed for the study of the objective 1-b) on enhance existing application enablement layer services to support energy saving for the network resources used by applications.

3. Conclusions

This paper proposes a new Key Issue is proposed for the study of the objective 1-b) on enhance existing application enablement layer services to support energy saving for the network resources used by applications.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.0.0.

* * * First Change * * * *

## 5.X	Key Issue #X: Enhance Existing Application Enablement Layer Services to Support Energy Saving

### 5.X.1	Description

The new SID "study on Application Enablement to support Energy Saving Phase 2" lists the objective of the study include:

- Potential enhancements to the application enablement layer (e.g. SEALDD, AIMLE, LM, ADAE) to support energy saving for the network resources used by applications.

SA6 developed a lot of energy consuming solutions that impact operator domain (e.g. SEAL enablers). These services require to be energy efficient and provide energy saving capabilities in order to satisfy the request from consumer, as well as support SA6 solutions in the exposure ecosystem landscape. Examples and potential aspects in scope of SA6 include (but not limited to) :

-	Data delivery via SEALDD server and client consumes energy. For example, enhancements to the SEALDD server on adjustment of data delivery configuration with consideration of energy contains is needed.

-	There is energy-related consideration in the Rel-19 AIMLAPP work, but without solution on how to realize energy saving or efficiency. For example, AIMLE services for supporting FL, energy consumed at the participants (e.g., VAL server/client, AIMLE server/client) for performing ML model training and/or intermediate result delivery with energy constraints. Enhancements to AIMLE services, e.g., FL member selection, can be considered for the energy requirements.

-	LM client reports location information consumes energy. LM service on location information collection can be adjusted, for example, adjustment of LCS QoS to reduce energy consumption and/or switch LM client location reporting to network location reporting to saving energy at the UE side.

-	There are existing ADAE analytics on energy efficiency, for example, DN Energy Efficiency analytics, can be used for supporting energy saving of the network resources in some applications. To support wider range of applications, potential enhancements to the ADAE services may needed, e.g., enhance existing ADAE analytics and/or introduce new analytics.

Besides the examples listed above, there may other existing SEAL enablers and services can be enhanced to support energy saving for the network resources used by various applications. Thus, to enhance existing application enablement layer services for supporting energy saving, the following aspects need be studied :

-	Which existing application enablement layer services need be enhanced to support energy saving of the resources used by applications, including.

-	Whether and how to enhance the identified existing application enablement layer services for energy saving.

### 5.X.2	Open Issues

To enhance existing application enablement layer services to support energy saving, the key issue will study:

-	Study and identify which existing application enablement layer services need be enhanced to support energy saving of the resources used by applications, including

-	study which existing application enablement layer services can be enhanced for supporting energy saving

-	identify the use cases and requirements for the enhancements of the application enablement layer services

-	Study whether and how to enhance the identified existing application enablement layer services for energy saving

* * * End of Changes * * * *