3GPP TSG-SA WG6 Meeting #68	S6-253271

Göteborg, Sweden, 25th – 29th August 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	New Key Issue on Leverage Existing 3GPP System’s Capabilities to Application Enablement for Energy Saving

Spec:	3GPP 23.700-44 v0.0.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

This pCR proposes new Key Issue for FS_EnergySys_Ph2_APP.

2. Reason for Change

The New SID for study on Application Enablement to support Energy Saving Phase 2 (SP-250871) was approved in the SA#108 meeting. According to the approved SID, the objective of the study is to identify and specify enhancements at application enablement layer to enable support for energy saving, which include

- Study functional enhancements and possible architectural enhancements to application enablers/services to support energy saving, including:

  - potential enhancements to NSCE services for usage/configuration of dedicated network slice to verticals.

  - potential enhancements to the application enablement layer (e.g. SEALDD, AIMLE, LM, ADAE) to support energy saving for the network resources used by applications.

- Identify potential solutions, including the information flows and application enablement APIs satisfying the architectural requirements and enhancements identified in bullet#1.

NOTE:	The study of application enablement for energy saving will consider the underlying 3GPP system’s existing capabilities such as leveraging the energy consumption information provided by the Core Network (e.g. EIF defined in TS 23.501), network slicing management and energy measurement information exposed by Management system (e.g. introduced in TS 28.552 and TS 28.554).

Therefore, a new Key Issue is proposed for the study of application enablement for energy saving by leveraging existing 3GPP system’s capabilities.

3. Conclusions

This paper proposes a new Key Issue is proposed for the study of application enablement for energy saving by leveraging existing 3GPP system’s capabilities.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.0.0.

* * * First Change * * * *

## 5.Y	Key Issue #Y: Leverage Existing 3GPP System’s Capabilities to Application Enablement for Energy Saving

### 5.Y.1	Description

The new SID "study on Application Enablement to support Energy Saving Phase 2" mentions leveraging existing 3GPP system’s capabilities to support application enablement for energy saving:

- The study of application enablement for energy saving will consider the underlying 3GPP system’s existing capabilities such as leveraging the energy consumption information provided by the Core Network (e.g. EIF defined in TS 23.501), network slicing management and energy measurement information exposed by Management system (e.g. introduced in TS 28.552 and TS 28.554).

In SA2, Release-19 study on 5GS Enhancement for Energy Efficiency and Energy Saving is completed with solutions. SA2 focus on enhancements to the 5G System that includes framework for data volume information collection, network energy consumption collection, calculation, and exposure. A new network function, Energy Information Function (EIF), is introduced (TS 23.501, TS 23.502) to enable a consumer (AF/NEF or 5GC NF) to subscribe for energy related information of required granularities (UE, S-NSSAI, PDU Session and/or Service Data Flow).

The work on the energy efficiency topics of 5G in SA5 started from Release-16 and continues until now in Release-19 work. As defined in TS 28.310, TS 28.552, and TS 28.554, etc., SA5 focus on defining use cases, requirements, and solutions for the measurement of the energy efficiency of NG-RAN, 5G Core and network slicing and for the optimization of the energy savings.

These existing capabilities introduced in SA2 and SA5 can be considered to support application enablement for energy saving. For example,

-	Energy consumption information per UE/service data flow provided by EIF can be used by SEAL servers (e.g., AIMLE server, SEALDD server, LM server) to determine operations (e.g., switch location reporting entity, change AIMLE members for performing an AIML task) and/or adjust SEAL services (e.g., adjust data delivery timeslot) for saving energy of the relevant network resources used for the corresponding applications.

-	The capability on network slicing management and energy measurement information exposed by Management system can be considered by SEAL servers (e.g., NSCE server) to adjust configurations on dedicate network slices to saving energy of the relevant network resources used for the corresponding applications.

Thus, to support application enablement for energy saving by leveraging existing 3GPP system’s capabilities, the following aspects need be studied:

-	Whether and how existing capabilities provided by the core network can be used for application enablement layer services to support energy saving.

-	Whether and how existing capabilities provided by management system can be used for application enablement layer services to support energy saving.

### 5.Y.1	Open Issues

To support application enablement for energy saving by leveraging existing 3GPP system’s capabilities, the key issue will study:

-	Study whether and how existing capabilities provided by the core network can be used for application enablement layer services to support energy saving

-	Study whether and how existing capabilities provided by management system can be used for application enablement layer services to support energy saving

* * * End of Changes * * * *