3GPP TSG-SA WG6 Meeting #68	S6-253288

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0062 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements for spatial anchor state event management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-APP |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The proposal provides solution to address the ENs. There are two ENs which are related to each other: EN#1 Clause 8.3.1.2 has Editor's Note: How SAn sever handles the “auto delete” of the spatial anchor on expiry is FFS. EN#2 And clause 8.3.4 has Editor's Note: How the SAn client handles expiration of Spatial anchor is FFS. In the clause 8.3.5.2.1 the discovery event provides the state changes event. This event enables requestor to receive notifications about any state change related event. For EN#1: As per the table Table 7.3.2.1-1: Spatial anchor profile - if the indication is not provided, the SAn server deletes the inactive spatial anchor. It is important for the SAn server to provide a notification regarding the state inactive state change and the spatial anchor would be deleted. The SAn server deleting the spatial anchor would disrupt the service and negatively impact the QoE of the users. Therefore it is necessary to provide notifications to the consumers or requestor before SAn handles the auto deletion of spatial anchors. If the requestor is authorized, may update the state and change the spatial anchor to active because of reasons like ongoing usage of the spatial anchor. The notifications enables the consumers to handle the impacts of the deletion of the spatial anchor in application logic. It informs the consumers that such spatial anchors would no longer be available. For EN#2: It is necessary to provide the notification to the consumers or requestor before the actual expiration of SA to provide a grace period so that consumer or requestor can determine the handling of expiration of the spatial anchor. For example, the SAn client on receiving early notification, would provide a notification to the user(application) that SA would no longer be available due to inactive state. This enables consumers(SAn client) to better handle and avoid sudden disruption as inactive anchors would no longer be discoverable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix for Ens in clause 8.3.1.2 and 8.3.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ENs will not be resolved and handling of expiration of SA is essential. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.5.1, 8.3.5.2, 8.3.5.2.1, 8.3.5.2.2, 8.3.5.3.1, 8.3.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.3.5	Spatial anchor subscription

#### 8.3.5.1	General

Spatial anchor subscription enables a SAn client or VAL server to subscribe with the SAn server to receive notifications related to spatial anchors.

#### 8.3.5.2	Procedures

##### 8.3.5.2.1	Subscribe

Figure 8.3.5.2.1-1 illustrates the spatial anchor subscription procedure.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

Figure 8.3.5.2.1-1: Spatial anchor subscription

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor subscribe request to the SAn server. The request includes a requestor identifier, security credentials, a notification endpoint, and event information for which the requestor is subscribing. The request may include an expiration time.

a.	The requestor may include the "discovery" event type to indicate the SAn server to notify the requestor when the SAn server detects events related to the management of spatial anchors (e.g., creation, update, deletion, spatial anchor state changes (e.g. active, inactive)) according to discovery event filters and spatial anchor filters provided in the request. If the requestor is a SAn client, the request includes a VAL client identifier . For the spatial anchor state changes monitoring notification, the discovery event filter include spatial anchor expiration offset time which is used by the SAn server to send the notification at a time earlier than the actual spatial anchor expiration time by the offset.

The request may include a early spatial anchor inactive notification indication in the spatial anchor. Based on this indication, the SAn server monitors the state changes of the spatial anchors that are active and would be inactive on expiry and sends a notification at a time earlier than the expiration time of the spatial anchors by offset time. The offset time can be configured as per the internal policy of the SAn server, if not included in the request by consumers.

b.	The VAL server may include the "UE with spatial anchor interest" event type to indicate the SAn server to notify the VAL server when the SAn server detects UE(s) with interest in specific spatial anchors according to the spatial anchor of interest event filters and spatial anchor filters provided in the request.

c.	The requestor may include the "spatial anchor in range" event type to indicate the SAn server to notify the requestor when the SAn server detects that spatial anchors become in range or out of range of the subscriber according to range event filters provided in the request. Range event filters can include a range for detecting spatial anchors, an indication to enable persistent search, the pose of the subscriber. If the requestor is a SAn client, the spatial anchor subscribe request includes a VAL client identifier.

2.	Upon receiving the request from the requestor, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server creates the subscription and stores the subscription information.

a.	If the subscribed event is for "discovery", the SAn server starts monitoring spatial anchors management events according to the discovery event filters and spatial anchor filters.

b.	If the subscribed event is for "UE with spatial anchor interest", the SAn server starts monitoring spatial anchors of interest according to the UE with spatial anchor interest event filters and spatial anchor filters. If the spatial anchors of interest have been discovered by a UE, the SAn server can obtain the UE location information from the 5GS or the SEAL Location Management service. Additionally, the SAn server can consider data analytics, such as UE mobility prediction. The SAn server may monitor the distance between UEs and the spatial anchor of interest to determine whether the UEs enters, exits or is within range of the spatial anchors of interest.

c.	If the subscribed event is for "spatial anchor in range" and the requestor is a SAn client, the SAn server starts monitoring the corresponding UE location and direction corresponding to the pose information according to the range event filters and spatial anchor filters. The UE location information can be obtained by using capabilities of the 5GS or the SEAL Location Management service. Additionally, the SAn server can consider data analytics, such as UE mobility prediction. The SAn server monitors the distance between UEs and the spatial anchor of interest to determine whether the spatial anchors become in-range or out-of-range of the UE. If persistent search is enabled, the SAn server monitors location of the user (or UE) and pose of the user, along with a direction of movement, and discovers a list of spatial anchors which becomes in-range or out of range of the user device based on the location of the user device, the pose of the user and in the user's field of view. For spatial anchors associated with trackable objects, the SAn server can determine the in-range or out-of-range of the UE with the current position of the objects which are being tracked by SEAL SM server. Additionally, SAn server can consider trackable object mobility prediction which is provided by SEAL SM server based on the data analytics for position, direction and velocity of the object.

NOTE:	How to do trackable object mobility prediction and estimate the duration of in-range object tracking is implementation specific and out of scope.

Editor's Note: Spatial map management procedures to request object tracking need to be provided.

3.	The SAn server sends a spatial anchor subscribe response to the requestor. If the SAn server has created the subscription, the response includes an indication of success, the subscription identifier and may include an expiration time; to maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the spatial anchor subscription expires. If the SAn server has not created the subscription, the response includes an indication of failure and may include a reason for failure.

* * * Next Change * * * *

##### 8.3.5.2.2	Notify

Figure 8.3.5.2.2-1 illustrates the spatial anchor notify operation between the SAn server and a SAn client or VAL server.

Pre-conditions:

1.	The SAn client or VAL server has subscribed for spatial anchor notifications with the SAn server;

Figure 8.3.5.2.2-1: Spatial anchor notification

1.	An event occurs at the SAn server that satisfies trigger conditions for notifying a subscriber (e.g., SAn client or VAL server) according to subscribed events and discovery filters provided for the events.

2.	The SAn server sends a spatial anchor notification to the requestor indicating the event. The notification includes a subscription identity and includes event-specific information. The SAn server includes the spatial anchors in the notification if their state is active.

a.	If the subscribed event is for "discovery", the notification includes the list of determined spatial anchors. For the state changes monitoring events for spatial anchors which are active and will be inactive as per the expriy time period, the  SAn server sends the Transient Inactive state Spatial anchor list in the notification at a time earlier than the actual spatial anchor expiration time by the offset (e.g., send the notification at 100 seconds before the expiry time of the spatial anchor).

Based on the early spatial anchor inactive notification indication, the SAn server monitors the state changes of the spatial anchors that are active and would be inactive on expiry and sends a notification at a time earlier than the expiration time of the spatial anchors by offset time. If the spatial anchor expiration offset time is not provided in the request, then the SAn server can use its pre-configured spatial anchor expiration offset time.

The notification message may include the Auto delete spatial anchor list to indicate list of spatial anchors that are inactive and will be deleted and the inactivity spatial anchor expiry timer. The inactivity spatial anchor delete timer indicates the inactivity timer, the expiry of which initiates the deletion of inactive spatial anchors by the SAn server.

b.	If the subscribed event is for "UE with spatial anchor interest", the notification includes information about UE with spatial anchor interest (e.g., UE identifier, spatial anchor identifier(s), range from the spatial anchor(s), entering or exiting the range, etc.).

c.	If the subscribed event is for "spatial anchor in range", the notification includes the list of determined spatial anchors that become in range, out of range of the subscriber.

* * * Next Change * * * *

#### 8.3.5.3	Information flows

##### 8.3.5.3.1	Spatial anchor subscribe request

Table 8.3.5.3.1-1 describes information elements for the spatial anchor subscribe request from the SAn client or VAL server to the SAn server.

Table 8.3.5.3.1-1: Spatial anchor subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL client ID | O | The identifier of the VAL client when the requestor is a SAn client. |
| Notification Target Address | M | Notification target address (e.g. URL) where the notifications should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription. |
| Event type | M | The type of event for which the requestor is subscribing. Supported event types include: - "discovery" - "UE with spatial anchor interest" - "spatial anchor in range" |
| Persistent Search (NOTE) | O | Indicates to enable persistent search. |
| Event detection filters | O | Event type specific detection filters. |
| > Discovery event filters | O | Event filters applicable to the "discovery" event type. The event filter is considered by the SAn server for triggering the notification and can include management events (e.g., creation, update, deletion, state changes). The filter for state changes from active to inactive state may include a spatial anchor expiration offset time to trigger notification ahead of expiration time. It can include an early inactive notification indication to trigger an early notification for the spatial anchors that are active and will be inactive on spatial anchor expiration time. |
| > UE with spatial anchor interest event filters | O | Event filters applicable to “UE with spatial anchor interest” event type. The event filter is considered by the SAn server for triggering the notification and can include an indication for detecting UEs that have discovered spatial anchors, that are within range or become out of range of spatial anchors, or that enter or leave the range of a spatial anchor, and a value for the range. |
| > Range event filters | O | Event filters applicable to “spatial anchor in range” event type. The event filter is considered by the SAn server for triggering the notification and can include an indication for spatial anchors becoming in range of the UE, out of range of the UE, or both and a value for the range. |
| >> current location | O | Current location of the user and direction. |
| >> pose | O | Current pose of the user |
| >> application specific range | M | Specifies range within which the spatial anchors are monitored to be in range or out of range of the subscriber's field of view. |
| Spatial anchor filters | O | The characteristics of spatial anchors that the SAn server considers for triggering a notification. |
| > Spatial anchor ID list | O | Identifiers of the spatial anchors of interest. |
| > Service ID list | O | Identifiers of the associated application services of interest. |
| > Area of interest | O | The area of interest (e.g., topological and/or geographical). |
| NOTE: This IE can be included with "spatial anchor in range" event. |  |  |

##### 8.3.5.3.3	Spatial anchor notification

Table 8.3.5.3.3-1 describes information elements for the spatial anchor notification sent by the SAn server.

Table 8.3.5.3.3-1: Spatial anchor notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| Event type | M | The type of event notification. Supported event types include: - "discovery" - "UE with spatial anchor interest" - "spatial anchor in range" |
| Detected event | O | Identifier of the event that was detected and that triggered the server to send a notification (e.g., spatial anchors becoming in range of the UE or out of range of the UE; or, UE entering the range of a spatial anchor or leaving the range of a spatial anchor; or, discovery events like spatial anchors state changes, create, update). |
| Spatial anchor list | O | List of discovered spatial anchors (as detailed in spatial anchor discovery response). Applicable for "discovery" and "spatial anchor in range" events. |
| Transient Inactive state Spatial anchor list | O | List of spatial anchors that are going to inactive. Applicable for state changes discovery event for early inactiven notification by offset of the spatial anchors which are active and will be inactive . |
| Auto delete spatial anchor list | O | List of spatial anchors information that are inactive and will be deleted by the SAn server. Applicable for "discovery" events |
| > Inactive spatial anchor ID | M | Indicates the spatial anchor ID |
| > Inactivity spatial anchor expiry timer | M | For the spatial anchors which have Expiry handling indication set in their profile, indicates the time until the spatial anchor deletion by the SAn server. |
| UE with spatial anchor interest list | O | List of UE with spatial anchor interest. Applicable for "UE with spatial anchor interest" and "spatial anchor in range" events. |
| > UE ID | O | Identifier of the UE with spatial anchor interest. |
| > Spatial anchor ID | O | Identifier of the spatial anchor of interest. |
| > Spatial anchor range | O | The spatial anchor range. |

* * * End of Changes * * * *