3GPP TSG-SA WG6 Meeting #68	S6-253313

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	Huawei, Hisilicon

Title:	Key Issue 1 on AIoT Data Management

Spec:	3GPP TS / TR 23.700-26 v0.0.0

Agenda item:	9.8

Document for:	Approval

Contact:	Cuili Ge @huawei.com

1. Introduction

Multiple industries have requirements on AIoT data. If each industry application implements the requirements on their own, it may hinder the widely marketing adoption of AIoT. In fact, some requirements are comman to multiple industries. To reduce the complexity of industry-specific adaptations, it is proposed to introduce the enabler service addresses common needs of AIoT data by building common and essential capabilities.

2. Reason for Change

AIoT services provide raw data of EPC, Reader/Region and time, which has significant gaps in semantics, granularity and business context from industry applications, making it unable to be directly used.. The  data mapping and processing between the application semantics and AIoT data  is missing.

3. Conclusions

AIoT services has significant gaps in semantics, granularity and business context, compared to the requirements of industry applications.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.0.0.

* * * First Change * * * *

AIoT services provide the raw data including device ID, reader/region information and timestamp, etc, which is mainly the unerlying expression about "which tag was seen by which reader/region at what time". However, the upper-layer applications require high-level, semantic-rich information such as "which business object, at what time, in which logical location, experienced what meaningful business event, and associated with which business attributes". For example, typical AIoT data is about  "Tag X was seen at Reader Y at Time Z ", while application requirement is about "the Product SKU123 has just been received through the Gate 1 at Warehouse 1".

So, the AIoT data and application requement has significant gaps in semantics, granularity and business context:

1)	Physical location vs logical location: AIoT provides location information by the reader id, but the application needs a logically significant business position, such as warehouse zone A, workstation 3, entry checkpoint1.

2)	Device id vs business object: AIoT identifies the tag(EPC) attached to the item, but the application needs to get the information of the item(product, asset, pallet).

3)	AIoT event vs business event: AIoT services provide independent, atomic event(Tag X was seen at Reader Y at Time Z), but the application usually needs the aggregated state or meaningful event inferred from multiple instantaneous events, for example:

-	Business event: A packet containing 5 items has been shipped. (Confirm that all five product tags are read at a specific location and time and are associated with the same shipping order.)-	Counting: There are 25 items on the shelf. (Summarize the number of unique tags which is associated to some item, read within a specific area.)

-	State: Pallet a is moved to production line. (Check the tag’s physical location is accurately changed from reader A to reader B)

For the downlink datastream, the AIoT core network provides inventory service API. It requires the upper layer to specify the network resources (service area) for executing inventory instructions. However, the application layer is only interested in the applicatoin layer items and business area information. There is a semantic gap between the business area information and the physical area information on the network side. Direct integration and use require a significant amount of adaptation development work, which is not conducive to batch replication.

Open issues:

1.	How to bridge the gap between the AIoT raw data and the application requirements in semantics, mapping the AIoT data and the business data.2.	How to build the model or rule to transfer the AIoT event to the business event, so that the aggregated state or meaningful event can be consumed by upper applications such as ERP(enterprise resource planning system), MES(Management Execution System) or WMS(warehousing management system).3.	How to address the semantic translation issue between application layer business tasks and AIoT inventory commands.