3GPP TSG-SA WG6 Meeting #68	S6-253500

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253128, 1366, 1138)

Source:	Telefonica, Apple, Fogus, UMA

Title:	Pseudo-CR CAPIF_Ph4 Key Issue Certificate unavailability

Spec:	3GPP TR 23.700-43

Agenda item:	9.11

Document for:	Agreement

Contact:	Walter Featherstone, w_featherstone@apple.com

1. Introduction

The CAPIF phase 4 SID includes an objective: Support for new CAPIF enhancements identified from SDOs/industry forums such as GSMA, ETSI OpenCAPIF, etc.

2. Reason for Change

This paper proposes a Key Issue based on findings from ETSI OpenCAPIF, specifically relating to certificate expiry/unavailability.

A call flow illustrating the certificate expiry issue in the case API Provider deregistration and API Invoker offboarding is illustrated below in Figure 1.

Figure 1: Flow showing API Provider & API Invoker with expired Certificate

The steps relating to Figure 1 are described below:

1. API Provider registers AMF, APF and AEF into CCF.

2. API Provider entities certificates expire/become unavailable

3. API Provider tries to update AMF/APF/AEF from CCF, but operation fails as Certificate is not valid

4. API Invoker onboards to the CCF

5. API Invoker entities certificate expires/become unavailable

6. API Invoker tries to update at the CCF, but the operation fails as Certificate is not valid

7. API Invoker onboarding expTime is reached if it was set during onboarding

8. CCF deletes API Invoker after expTime is reached if it was set

These situations leave API Invoker and API Providers information in CCF that is invalid and can lead to inconsistences (eg CCF returining information about AEFs that are not valid anymore, as API Providers registrations do not expire).

3. Conclusions

n/a.

4. Proposal

It is proposed to agree the following key issue for inclusion in 3GPP TR 23.700-43.

* * * First Change * * * *

## X.x	Key issue #X: Certificate unavailability

### X.x.1	Description

As described in clause 6.1 of 3GPP TS 33.122 [x], a prerequisite to onboarding an API invoker to CAPIF, an API invoker is required to obtain onboarding enrolment information from the API provider domain (see step 1 of the referenced clause). This information is necessary to enable the API invoker to authenticate with the CAPIF core function (CCF) and establish secure communication over which to perform the onboarding process. The enrolment information must include details of the CCF (i.e., its address in the form of a URI and root certificate authority (CA) certificate) and includes an onboarding credential (i.e., an access token). With this information the API invoker can request to onboard to the CCF (clause 8.1 of 3GPP TS 23.222 [y]) and, if successful, receives its onboarding information in the response, which also includes the CCF assigned API invoker ID. A key attribute of the onboarding information response is the certificate assigned to the API invoker, which is signed e.g., by the CCF CA. API providers are also assigned signed certificates when registering to the CCF.

The problem identified is that an entity (API Invoker or API Provider) can no longer manage their resources at the CCF if they lose access to a valid certificate e.g., certificate expiry). This means for example:

- The API Provider cannot update/delete/create new AEFs, APFs or update/delete AMF.

- The API Invoker entity cannot perform any opreation with CCF (update/delete onboarding information, request security context, …).

### X.x.2	Open issues

This key issue will study approaches to managing entities (i.e., API Invoker or API Provider) without a valid CCF issued and signed certificate, specifically how such entities can be deleted from the CCF including associated resources to the entities like Security_Context associated to API Invokers and API resources published by APFs served by AEFs.

NOTE: Coordination will be required with SA3 for the identificationof the  aspects of certificate lifecycle management. that can affect the CAPIF framework

* * * End of Changes * * * *