3GPP TSG-SA WG6 Meeting #68	S6-253524

Gothenburg, Sweden 25th – 29th August 2025	(Revision of S6-253113)

Source:	Huawei, Hisilicon

Title:	Key issues: utilizing sensing results for tracking dynamic UAVs

Spec:	3GPP TR 23.700-15 v0.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	Lei Chen (chenlei79@huawei.com)

1. Introduction

This contribution proposes a key issue for how to assist tracking dynamic UAVs by utilizaing sensing results.

2. Reason for Change

This contribution is to disscuss potential key issues of utilizing sensing results for UAV.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15..

* * * First Change * * * *

## X.X Key issue #x: utilizing sensing result for tracking UAVs

## x.x.1 Description

3GPP SA1 has specified sensing service requirement in TS 22.137 [x]. According to this spec, the 5G system shall be able to provide sensing service to detect, and/or track one or more objects (e.g., UAVs, birds) and the environment around the object(s).

3GPP SA6 has developed the application enabler (i.e., UAE) to support UAS which specified in 3GPP TS 23.255 [y]. In section 7.8, an UAE service of tracking dynamic UAVs in an application defined area relative to a host UAV is specified. In this service, the UAE layer needs to provide the dynamic information (i.e. other UAVs’ location information) to the application layer and/or the host UAV. Currently, this service is performed by invoking location management service in SEAL layer. The location management service provided by SEAL layer coud localize an UAV with equiping UE module, but it is impossible to obtain the location information of an UAV without equiping UE module.

If the UAE service of tracking dynamic UAVs could use sensing results, its function could be enhanced signaturely. In addition, sensing results also can be used to track UAV with UE modle and the effiency will be higher than positioning way because sensing can track many UAVs by one time while positioning has to be performed per UE. So, it would make sense to stduy how to utilize related sensing results to enhance the track dynamic UAVs service.

## x.x.2 Open issues

Based on the above analysis, the following open issue need to be studied:

- How to enhance UAE layer to support tracking UAVs by utilizing sensing results.

# *** Next Change ***

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]		3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x] 3GPP TS 22.137, “Service requirements for Integrated Sensing and Communication; Stage 1”.

[y] 3GPP TS 23.255, “Application layer support for Uncrewed Aerial System (UAS); Functional architecture and information flows”.

# *** End of Changes ***