3GPP TSG-SA WG6 Meeting #68	S6-253526

Gothenburg, Sweden 25th – 29th Aug 2025	(revision of S6-253281)

Source:	CATT

Title:	New KI on enhancement of UAV service utilizing sensing results

Spec:	3GPP TR 23.700-15 v0.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes a new KI on enhancement of UAV service utilizing sensing results.

2. Reason for Change

SA6 has developed the application enabler (i.e. UAE) to support UAS specified in 3GPP TS 23.255 [23255] in which communications between UAVs, monitoring of UAV location deviation, reporting of UAV events, etc. have been studied. However, how to support the new service requirements (e.g. UAV detection, illegal invasion, collision avoidance and trajectory tracking) stated in 3GPP SA1 in the application enabled layer have not been studied and should be investigated based on current UAS application enabled layer.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-15 v0.0.0.

* * * First Change * * * *

5	Key issues

5.x	Key issue #x: Enhance UAV service utilizing sensing results

5.x.1	Description

The sensing related use cases about UAVs have been specified in 3GPP TR 22.837[22837] including UAV detection, illegal invasion, collision avoidance, etc. And it also specified the service requirements in the 3GPP TS 22.137 [22137] to provide the sensing service to implement the use cases relevant to the UAVs. Such as “The 5G system shall be able to provide sensing service to detect, and/or track one or more objects (e.g., UAVs, birds) and the environment around the object(s).”

In Rel-19, the application enabler (i.e. UAE) has been developed to support UAS which specified in 3GPP TS 23.255 [23255] in which communications between UAVs, monitoring of UAV location deviation and reporting of UAV events have been studied. Especially for Detect And Avoid (DAA) service which is to detect the potential collicsion between UAVs, is a key service of UAS to accomplish the safe and effient UAV management. However, the DAA service must depends on the PC5 communication and the PC5 communication is not large-scale commercially used in UAV industry and even some UAVs are not equiped with 3GPP UE module that cannot utilize the defined 3GPP service (e.g. location)..

So how to support or optimize the new service requirements (e.g. UAV detection, illegal invasion, collision avoidance) is still under investigation in the application enabled layer based on current UAS application enabler and need further study.

5.x.2	Open Issues

Based on the above analysis, the following open issues need to be studied:

-	How to enhance the UAS application enabler to support UAV detection utilizing the obtained sensing results.

-	How to enhance the UAS application enabler to support illegal invasion for UAVs utilizing the obtained sensing results.

-	How to enhance the UAS application enabler to support collision avoidance for UAVs utilizing the obtained sensing results.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[22837]	3GPP TR 22.837: “Feasibility Study on Integrated Sensing and Communication".

[23255]	3GPP TS 23.255: "Application layer support for Uncrewed Aerial System (UAS); Functional architecture and information flows ".

* * * End of Change * * * *