3GPP TSG-SA WG6 Meeting #68	S6-253603

Stor-Göteborg, Sweden 25th – 29th August 2025                                       was S6-253181

Source:	Nokia

Title:	Pseudo-CR on updating Technical Solution 10 procedures and information flow

Spec:	3GPP TR 23.700-35 v1.0.0

Agenda item:	9.3

Document for:	Approval

Contact:	Rajesh Babu Natarajan rajesh_babu.natarajan@nokia.com

- Introduction

Technical Solution # 10 was proposed where a single NRM service for network service QoS control for applications. The proposed solution is not complete, since the procedure and information flows need to be added.

- Reason for Change

Technical Solution #10 is updated to include procedure and information flows.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 v1.0.0.

* * * First Change * * * *

##### 8.1.10.1.1	Procedures

This subclause describes the procedure for QoS control service creation/updation/deletion based on VAL server application QoS requirements.

Figure 8.1.10.1.1-1 illustrates the procedure for the QoS control service.

Figure 8.1.10.1.1-1: QoS control service procedure

- The VAL server triggers SS_ NRM_QoS_Service_Create/Update/Delete request to the NRM server to create/update/delete network resources for one or more users belonging to one or more VAL services based on the application QoS requirements. This includes target QoS requirements (e.g. Data rate, Delay characteristics and Packet loss) for the corresponding VAL UEs or group of VAL UEs or VAL services.

- The NRM server shall authorize the request and evaluates the input required for Nnef_AFSessionWithQoS API trigger.

NOTE:	If the NRM is considered to be trusted by the operator, the NRM can use the Npcf_PolicyAuthorization_Create/Update/Delete request message to interact directly with 5GC (i.e. PCF) to request/update/delete resources.

3.	The NRM server shall trigger Nnef_AFSessionWithQoS_Create/Update/Delete request towards 5GC (via NEF) to request/update/delete resources for the VAL services.

4.	The NRM server shall provide SS_NRM_QoS_Service_Create/Update/Delete response to the VAL server with the result. This will include information either per VAL UE or per set of VAL UEs or VAL services, as indicated by the request of the VAL server in step 1.

* * * Next Change * * * *

##### 8.1.10.1.2	Information flows

###### 8.1.10.1.2.1	SS_NRM_QoS_Service_Create request

Table 8.1.10.1.2.1-1 describes the information flow for QoS service create request from the VAL server to the NRM server.

Table 8.1.10.1.2.1-1: SS_NRM_QoS_Service_Create request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Application Server address | M | The endpoint address of the Application Server, where the Application traffic flows. |
| List of VAL UE IDs or VAL user IDs | O (see NOTE) | List consisting of one or more VAL UE IDs and/or VAL user IDs for whom the network resource creation is requested. |
| VAL group ID | O (see NOTE) | The VAL group ID for whom the network resource creation is requested. |
| Target QoS requirement | M | The target QoS requirement corresponds to the VAL service QoS requirements as applied for a UE or group of UEs (E.g. Data rate, Delay characteristics, Packet loss) and the corresponding Application traffic flow. |
| NOTE: Either of the information elements should be present. |  |  |

###### 8.1.10.1.2.2	SS_NRM_QoS_Service_Create response

Table 8.1.10.1.2.2-1 describes the information flow for QoS service create response from the NRM server to the VAL server.

Table 8.1.10.1.2.2-1: SS_NRM_QoS_Service_Create response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the creation of the network resource with the underlying network. |

###### 8.1.10.1.2.3	SS_NRM_QoS_Service_Update request

Table 8.1.10.1.2.3-1 describes the information flow for QoS service update request from the VAL server to the NRM server.

Table 8.1.10.1.2.3-1: SS_NRM_QoS_Service_Update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Application Server address | M | The endpoint address of the Application Server, where the Application traffic flows. |
| List of VAL UE IDs or VAL user IDs | O (see NOTE) | List consisting of one or more VAL UE IDs and/or VAL user IDs. |
| VAL group ID | O (see NOTE) | The VAL group ID. |
| Target QoS requirement | O | The target QoS requirement corresponds to the VAL service QoS requirements as applied for a UE or group of UEs (E.g. Data rate, Delay characteristics, Packet loss) and the corresponding Application traffic flow. |
| NOTE: Either of the information elements should be present. |  |  |

###### 8.1.10.1.2.4	SS_NRM_QoS_Service_Update response

Table 8.1.10.1.2.4-1 describes the information flow for QoS service update response from the NRM server to the VAL server.

Table 8.1.10.1.2.4-1: SS_NRM_QoS_Service_Update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the update of network resource with the underlying network. |

###### 8.1.10.1.2.5	SS_NRM_QoS_Service_Delete request

Table 8.1.10.1.2.5-1 describes the information flow for QoS service delete request from the VAL server to the NRM server.

Table 8.1.10.1.2.5-1: SS_NRM_QoS_Service_Delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Application Server address | M | The endpoint address of the Application Server, where the Application traffic flows. |
| List of VAL UE IDs or VAL user IDs | O (see NOTE) | List consisting of one or more VAL UE IDs and/or VAL user IDs. |
| VAL group ID | O (see NOTE) | The VAL group ID. |
| NOTE: Either of the information elements should be present. |  |  |

###### 8.1.10.1.2.6	SS_NRM_QoS_Service_Delete response

Table 8.1.10.1.2.6-1 describes the information flow for QoS service delete response from the NRM server to the VAL server.

Table 8.1.10.1.2.6-1: SS_NRM_QoS_Service_Delete response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the deletion of network resource with the underlying network. |

* * * End of Change * * * *