3GPP TSG-SA WG6 Meeting #68	S6-253640

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253313)

Source:	Huawei, Hisilicon

Title:	Key Issue 1 on AIoT Data Management

Spec:	3GPP TS / TR 23.700-26 v0.0.0

Agenda item:	9.8

Document for:	Approval

Contact:	Cuili Ge @huawei.com

1. Introduction

Multiple industries have requirements on AIoT data. If each industry application implements the requirements on their own, it may hinder the widely marketing adoption of AIoT. In fact, some requirements are comman to multiple industries. To reduce the complexity of industry-specific adaptations, it is proposed to introduce the enabler service addresses common needs of AIoT data by building common and essential capabilities.

2. Reason for Change

AIoT services provide inventory result data of EPC, Reader/Region and time, which has significant gaps in semantics, granularity and business context from industry applications, making it unable to be directly used.. The  data mapping and processing between the application semantics and AIoT data  is missing.

3. Conclusions

AIoT services has significant gaps in semantics, granularity and business context, compared to the requirements of industry applications.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.0.0.

* * * First Change * * * *

AIoT inventory service provides the inventory result data only including device ID . However, the upper-layer applications require high-level, semantic-rich information such as "which business object, at what time, in which logical location, experienced what meaningful business event, and associated with which business attributes". For example, typical AIoT inventory result data is about "Device ID list: ID1, ID2, ID3…… ", while application requirement is about "the Product SKU123 has just been received through the Gate 1 at Warehouse 1".

So, the AIoT data and application requement has significant gaps in semantics, granularity and business context:

1)	Physical location vs logical location: Currently the external target area for inventory is normally e.g., a civic address or shapes, but the application needs a logically significant business position, such as warehouse zone A, workstation 3, entry checkpoint1.

2)	Device id vs business object: AIoT inventory service provided by SA2 only identifies the tag(EPC) attached to the item, but the application needs to get the information of the item(product, asset, pallet).

3)	Network inventory event vs business event: The 5GS AIoT inventory services provide inventory result only with Device ID list, but the application usually needs to aggregate device ID and other information like target service area, timestamp, and even aggreate the inventory result from multiple times inventory to generate a meaningful event, for example:

-	Business event: A packet containing 5 items has been shipped. (Confirm that all five product tags are read at a specific location and time and are associated with the same shipping order.)

-	Counting: There are 25 items on the shelf. (Summarize the number of unique tags which is associated to some item, read within a specific target area.)

-	State: Pallet a is moved to production line. (Check the tag’s physical location is accurately changed from target area A to target area B)

For the downlink datastream, the 5G core network provides inventory service API. It requires the upper layer to specify the network resources (service area) for executing inventory instructions. However, the application layer is only interested in the applicatoin layer items and business area information. There is a semantic gap between the business area information and the physical area information on the network side. Direct integration and use the network reported data require a significant amount of adaptation development work for the AIoT application, which is not conducive to promoting the 3GPP AIoT marketing adoptation.

Open issues:

1.	How to bridge the gap between the inventory result data and the application requirements in semantics.

2.	How to build the model or rule to convert the inventory result data to the business event, which can be directly consumed by upper applications such as ERP(enterprise resource planning system), MES(Management Execution System) or WMS(warehousing management system).

3.	How to address the semantic translation issue between application layer business tasks and AIoT inventory commands.

4.	How to achieve the security and privacy of the data provided by and sent to AIoT applications?