3GPP TSG-SA WG6 Meeting #68	S6-253681

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253332)

Source:	Lenovo

Title:	AIMLAPP architecture enhancements

Spec:	3GPP TR 23.700-83 v0.3.0

Agenda item:	9.4

Document for:	Approval

Contact:	Manos Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper provides architecture enhancements for AIMLAPP considering roaming and multi-operator scenarios.

2. Reason for Change

This paper provides architecture enhancements for AIMLAPP considering roaming and multi-operator scenarios.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

* * * First Change * * * *

# 6	Enhanced architecture for enabling AI/ML services

## 6.1	General

## 6.2	Enhanced AI/ML enablement functional model

### 6.2.1	AI/ML enablement functional model for multi-operator services

In this clause the architecture enhancements relate to the support for multi-operator services. There are two cases for the multi-operator service support:

- An AIMLE server connected to AIMLE clients over different PLMNs

- Each AIMLE server connected to AIMLE clients of single PLMN, whereas the AIMLE server interact each other via AIML-E.

In the first scenario, the AIML service (VAL service, ASP service) runs in a certain service area, which can be an edge or cloud service area or geographical area, where UEs connected to different PLMNs are present. Such service can be considered multi-operator service. VAL UEs are assumed to have AIMLE clients installed and active and connect to AIMLE server via different PLMN.

In Figure 6.2.1-1, the VAL UE#1 is connected to PLMN#1 and VAL UE#2 to PLMN#2. Both UEs are connected to the same AIMLE server. In this scenario, different VAL server may connect to UE#1 VAL client and different VAL server to UE#2; however, both VAL servers are connected to the same AIMLE server.

Figure 6.2.1-1 architecture for multi-operator support with common AIMLE server

In the second scenario, as depicted in Figure 6.2.1-2, the VAL UEs are assumed to have AIMLE clients installed and active and connect to different AIMLE servers provided by different MNOs or trusted 3rd parties of the MNOs.

In Figure 6.2.1-2, each AIMLE server connects to each PLMN. The VAL UE 1 is connected to PLMN1 and AIMLE server 1 and VAL UE 2 is connected to PLMN 2 and AIMLE server 2.

Figure 6.2.1-2 architecture for multi-operator support with distributed AIMLE servers

Editor’s Note 1: It is FFS whether and in which scenarios the VAL server has service agreements with one or both 3GPP network system operators in multi-operator scenarios.

Editor’s Note 2: The cardinality aspects of ML repositories and the requirement for new interface among them is FFS based on solutions’ progress.

### 6.2.2	AI/ML enablement functional model for roaming scenarios

This clause describes the AIMLAPP architecture for roaming UEs. This architecture is aligned with network roaming architecture, since it re-uses existing network roaming principles and describes the AIML service continuity while VAL UEs are roaming; hence how AIMLE spans across HPLMN and VPLMN is discussed. Such architecture applies to both roaming models (LBO and HR roaming architectures).

In Figure 6.2.2-1, the AIMLAPP architecture for enabling AI/ML services for roaming scenarios is showcased. In this architecture, AIML-UU is the interface between the AIMLE client and the AIMLE server, which can be over HPLMN or VPLMN which VAL UE is roaming. AIML-E supports the interaction among AIMLE servers deployed by different MNOs, whereas the MLR-E is the new interface supporting the interaction among repositories for aligning the information on the ML models and ML/FL members to allow the continuity of the AIML service while the VAL UE is roaming.

Figure 6.2.2-1 architecture for roaming support

Editor’s Note: The requirement for a new interface among ML repositories of different PLMNs in roaming scenario is FFS.

### 6.2.3	AI/ML enablement functional model for federation

Editor’s Note: The enhancements for federation support are FFS.

## 6.3	Functional Elements

The functional elements corresponding to the architecture will be presented in this clause.

## 6.4	Reference Points

The reference points corresponding to the architecture will be presented in this clause.

* * * Next Change * * * *