3GPP TSG-SA WG6 Meeting #68	S6-253750

Gothenburg, Sweden, 25th – 29th August 2025	(revision of S6-252222, 252405, 252525, 253312,253620)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on new solution of broadcast data delivery service to KI#1

Spec:	3GPP TR 23.700-52 0.2.0

Agenda item:	9.7

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This pCR is proposed a new solution to KI#1.

2. Reason for Change

In this solution, the SEALDD server will handle the broadcast delivery service request and interacts with the 5GS and eMBMS accordingly. And the SEALDD server act as the media ingress node towards the 3GPP network broadcast user plane functions (e.g., UPF(s), BM-SC).

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-52 v0.2.0.

* * * First Change * * * *

6	Solutions

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #x | X |  |  |  |
| Sol #2 |  |  |  |  |

6.x	Solution #x: SEALDD enabled broadcast data delivery

6.x.1	Solution description

6.x.1.1	General

In this solution, the SEALDD server provides new data delivery services to deliver the same content (e.g., application signalling or application user plane data) from a single source entity to all the recipients in a certain area. The SEALDD server provides:

- SEALDD broadcast data delivery service. It is used for the application that provides the same content to all the recipients in a certain area. It requires the support of the broadcast 5MBS session and/or eMBMS at the network and UE.

In general, the broadcast delivery services require the VAL server to provide its service requirements e.g., service area, start and end time of the delivery, QoS requirements. The SEALDD server provides its own address as the ingress address for the VAL server to send the DL data. During the data delivery service update with service area update, more MBS session may be created and additional MB-UPF addresses are returned to the SEALDD server. However, SEALDD server hides the multiple network user plane addresses towards the VAL server and the ingress address keep the same. For broadcast data delivery service, the SEALDD server determines to create both eMBMS bearer and MBS session based on the service area.

In this solution, the SEALDD server interacts with the 3GPP network to manage the eMBMS bearer and 5MBS broadcast session. Also the SEALDD client interacts with SEALDD server to handle the session/service announcement, listening status report. The SEALDD-UU utilizes the multicast/broadcast functions of NRM-UU to support this broadcast data delivery service.

The SEALDD server assumes the functionality of NRM server. The SEALDD client assumes the functionality of NRM client.

The SEALDD-UU-C is responsible for establishing unicast connection signalling for SEALDD data delivery. The NRM-UU is responsible for interactions between SEALDD server (being bundled with the function of the NRM server) and SEALDD client (being bundled with the function of the NRM client) for MBS session as defined in TS 23.434(MBS session announcement, UEjoin session acknowledgement, MBS status listening status report, MBS deannoucnement message,etc)

This SEALDD-UU includes the NRM-UU reference point as described in 3GPP TS 23.434 [23.434] regarding the MBS session control interactions between the SEALDD server and the UE.

Figure 6.x.1.1 -1 shows the high-level functional model for this service.

Figure 6.x.1.1-1: Broadcast delivery service functional model

6.x.1.2	Broadcast data delivery service

6.x.1.2.1	Broadcast data delivery service create

Figure 6.x.1.2.1-1 illustrates the high-level procedure of creating a broadcast data delivery service for the VAL server DL data delivery.

Figure 6.x.1.2.1-1: Broadcast delivery service create

- The VAL server invokes the Sdd_BroadcastDelivery create service to distribute its DL content to all the recipients within the service area. The data delivery requirements e.g., service area, start and end time of the delivery, QoS requirements are included. The endpoint of a VAL server which ingresses the DL data may be also provided.

NOTE 1：	This VAL server ingresses the DL data can be a different VAL server than the one in step 1.

2.	The SEALDD server determines to create a broadcast flow. The SEALDD server allocates the broadcast flow ID, the source multicast address and port used for DL data delivery to the recipients.

3.	The SEALDD server activates the eMBMS bearer as described in 3GPP TS 23.247 [23.247], and/or create the broadcast MBS session as described in 3GPP TS 23.247 [23.247] for this data delivery service and obtains the TMGI(s) from 3GPP network.

4.	The SEALDD server allocates an ingress address for the VAL server to send its DL data. The SEALDD also allocates a data delivery service ID which is used to update and delete the created data delivery service. The SEALDD server returns the Sdd_BroadcastDelivery create response with the allocated ingress address to the VAL server.

5.	The VAL server provision the service information including the SEALDD server address, data delivery service ID to the VAL client.

6. The VAL client sends the service request to the SEALDD client. The SEALDD server address, data delivery service ID are included.

7.	The SEALDD client sends the connection request (data delivery service ID) to the SEALDD server.

8.	The SEALDD server determines and returns the media related information including TMGI, broadcast flow ID, the source multicast address and port, protocols used for the DL data delivery based on the data delivery service ID.

9.	The SEALDD server (being bundled with the function of the NRM server) sends the MBS service announcement with TMGI of the broadcast MBS session to the SEALDD client  (being bundled with the function of the NRM client) via the NRM-UU as described in step 3 clause 14.3.4A.2.2 in 3GPP TS 23.434 [23.434].

10.	The UE starts to lisening the broadcast channel as inticated by the TMGI to receive the DL data as described in 3GPP TS 23.247 [23.247]..

11.	The SEALDD client (being bundled with the function of the NRM client) sends the MBS listening status report to the SEALDD server (being bundled with the function of the NRM server) via the NRM-UU as as described in step 3 clause 14.3.4A.2.2 in 3GPP TS 23.434 [23.434]. This message may be sent multiple time due the MBS reception quality changes.

12.	The VAL server starts to send DL data to the allocated ingress address. The SEALDD server encapsulates the received DL data within the broadcast flow packet with the source multicast address and further delivers the encapsulated packet(s) via the established eMBMS bearer and/or the broadcast MBS session.

NOTE 2:	The VAL server can start to send data to the SEALDD server after step 5. And the SEALDD server can start to send DL data to the network even before step 7. Then the UE can immediately receive the data after step 9.

6.x.1.2.2	Broadcast data delivery service update

Figure 6.x.1.2.2-1 illustrates the high-level procedure of updating a broadcast data delivery service for the VAL server DL data delivery.

Pre-condition:

1.	The VAL server has created a broadcast delivery service and obtained the its data delivery service ID as described in clause 6.x.1.2.1. And there is ongoing DL data is being delivered via this service.

Figure 6.x.1.2.2-1: Broadcast delivery service update

1.	The VAL server invokes the Sdd_broadcast data delivery update service. The data delivery service ID and updated parameters, e.g., service area or QoS requirements are included.

2.	The SEALDD server initiates the eMBMS bearer update and/or the broadcast MBS session update accordingly as described in 3GPP TS 23.247 [23.247]. If the service areas are updated and new broadcast MBS session(s) is created, addition MB-UPF ingress address(es) may be also returned. The SEALDD server associates it with the allocated ingress address towards the VAL server.

3.	The SEALDD server returns the Sdd_BroadcastDelivery update response indicating the success of update.

4.	The VAL server continues to send DL data to the allocated ingress address. And the SEALDD further delivers the DL via the updated eMBMS bearer and/or the broadcast MBS session.

6.x.1.2.3	Broadcast data delivery service delete

Figure 6.x.1.2.3-1 illustrates the high-level procedure of deleting a broadcast data delivery service for the VAL server DL data delivery.

Pre-condition:

1.	The VAL server has created a broadcast delivery service and obtained the its data delivery service ID as described in clause 6.x.1.2.1. And there is ongoing DL data is being delivered via this service.

Figure 6.x.1.2.3-1: Broadcast delivery service delete

1.	The VAL server no longer requires the broadcast delivery service. It invokes the Sdd_broadcast data delivery delete service. The data delivery service ID is included.

2.	The SEALDD server performs the eMBMS bearer delete and/or the broadcast MBS session release accordingly and interacts with the SEALDD client as described in clause 3GPP TS 23.247 [23.247].

4.	The SEALDD server returns the Sdd_BroadcastDelivery delete response indicating the success of delete.

6.x.2	Architecture Impacts

There are no architecture impacts.

6.x.3	Corresponding services

This solution introduces a new SEALDD data delivery service.

6.x.4	Solution evaluation

This solution provides a new data delivery service, i.e., broadcast data delivery service, where the VAL server only sends one copy of data to the SEALDD server, and the SEALDD server send it to all the recipients within the service area. During the service area update, the VAL server is not bothered by creating new MBS sessions with additional ingress addresses.