3GPP TSG-SA WG6 Meeting #69	S6-254030

Wuhan, P.R.China, 13th – 17th October 2025	(revision of S6-xxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0068 | rev |  | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fix for Spatial map position in clause 8.3.1& 8.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In Table 7.3.2.1-1, the Description of Spatial map position is “Position information based on spatial map. It includes the identifier of the Spatial map and the corresponding localization information.” - In Table 8.3.1.3.1-1 & 8.3.2.3.1-1, the Spatial map position IE is the second sub-IE of Position information, the Description of Position information is “Position information of the spatial anchor”, the Description of Spatial map position is “Position information of the spatial anchor within the spatial map.”, the Description of Position (first sub-IE of Position information) is “The 3D geographic position of the spatial anchor in space (x, y and z coordinates)”. It is wrong for the Description of Spatial map position, it should be “Position information based on spatial map”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Table 8.3.1.3.1-1 & 8.3.2.3.1-1, change the Description of Spatial map position which follow Table 7.3.2.1-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Keep wrong description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.3.1, 8.3.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

8.3.1.3.1	Spatial anchor create request

Table 8.3.1.3.1-1 describes information elements for the spatial anchor create request from the SAn client or VAL server to the SAn server.

Table 8.3.1.3.1-1: Spatial anchor create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL service information | M | Information about the application service associated with the spatial anchor. |
| > Service ID | M | The identifier of the application service. |
| > Service endpoint | M | Endpoint information (e.g., URI, FQDN, IP address) used to communicate with the application service. |
| > Service connection information | O | The connection information (e.g., DNN, DNAI(s)) for establishing connectivity to the application service. |
| List of spatial anchors | M | List of spatial anchors to be created. |
| > Position information | M | Position information of the spatial anchor |
| >> Position (NOTE) | O | The 3D geographic position of the spatial anchor in space (x, y and z coordinates). |
| >> Spatial map position (NOTE) | O | Position information based on spatial map. It includes the identifier of the Spatial map and the corresponding localization information. |
| > Application specific Spatial anchor information | O | A container to contain application specific information. |
| > Service area | O | The service area where the spatial anchor is accessible. |
| > Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover the spatial anchor. |
| > Spatial anchor resource identifer | M | The identifier to associate the resource information with the created spatial anchors. |
| NOTE: One of the IEs shall be present. |  |  |

# * * * * Next change * * * *

8.3.2.3.1	Spatial anchor update request

Table 8.3.2.3.1-1 describes information elements for the spatial anchor update request from the SAn client or VAL server to the SAn server.

Table 8.3.2.3.1-1: Spatial anchor update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| > VAL service information | O | Information about the application service associated with the spatial anchor. |
| >> Service ID | O | The identifier of the application service. |
| >> Service endpoint | O | Endpoint information (e.g., URI, FQDN, IP address) used to communicate with the application service. |
| >> Service connection information | O | The connection information (e.g., DNN, DNAI(s)) for establishing connectivity to the application service. |
| List of spatial anchors | M | List of spatial anchors to be updated. |
| > Spatial anchor ID | M | Identifier of the spatial anchor to be updated. |
| > Position information | O | Position information of the spatial anchor |
| >> Position (NOTE) | O | The 3D geographic position of the spatial anchor in space (x, y and z coordinates). |
| >> Spatial map position (NOTE) | O | Position information based on spatial map. It includes the identifier of the Spatial map and the corresponding localization information. |
| > Application specific Spatial anchor information | O | A container to contain application specific information. |
| > Service area | O | The service area where the spatial anchor is accessible. |
| > Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover the spatial anchor. |
| NOTE: If Position information is included, at least one of the IEs shall be present. |  |  |

# * * * * End of changes * * * *