3GPP TSG-SA WG6 Meeting #69                                                                           	S6-254038

Wuhan, P.R.China, 13th – 17th October 2025	(revision of S6-253xxx)

Source:	Huawei, Hisilicon

Title:	Conclusion of the technical gap#1

Spec:	3GPP TR 23.700-35 v2.0.0

Agenda item:	9.3

Document for:	Approval

Contact:	Yang Yanmei yangyanmei@huawei.com

1. Introduction

This contribution is to propose conclusion for the technical gap#1.

2. Reason for Change

2.1 Concept clarification

It should be noted that SEAL services and SEAL APIs is not the exact same concept. SEAL API invoking is just some http messages to request certain SEAL service.

From customer perspective, what they buy from providers are SEAL services, rather than the API messages. From service provider perspective, SEAL services are the exact the products they intent to offer to customers. Stage 2 mainly focus on SEAL services design. It is up to stage 3 how design APIs to enable APP developers to implement the application software for requesting SEAL services. The mapping between APIs with SEAL services is also up to stage 3.  One SEAL API can be used request different SEAL services by including service indication in the API messages.

2.1 Solutions comparation

Solution 6 propose to design different NRM resource adaption services, while it is still possible for stage 3 to design single API to support those services using certain attributes for services indication. Solution 7 propose using one single API to request different resource for different traffic flow types (e.g., audio, video, haptic). Although it is not explicitly mentioned, the essential idea is already using different traffic types to indicate different NRM resource adaption services being requested. The only difference with solution 6 is that criterial of NRM resource adaption services 1) Solution 6 suggest to design the service based on the QoS type required by different traffic type 2) Solution 7 suggest to using traffic type directly.

Solution 10 propose to define single resource adaption services for any type traffics. The key difference with solution 6 and 7 is whether to include indication on resource adaption service type (either based on traffic type or required resource type).

Since network resource costs have big difference for different traffic types, e.g., URLLC may requires double resources than normal traffic, it allow different charge policy for different traffic type. SLA negotiation should happen offline solution 10 does not support online service type indication.

|  | Number of NRM Resource adaption service | API implementation impact if using single API | Business impact |
| --- | --- | --- | --- |
| Solution 10 | One | NONE | More usable for offline negotiation on SLA (Negotiation on the QoS requirement and charging policy based on exact QoS value) |
| Solution 6 | Multiple -based on QoS requirement type required by traffic type (URLLC, high bandwidth, low latency) …. | Additional parameter Required Resource service type (potential value URLLC, high bandwidth, low latency) to the API request message | Allow online subscription based on Required Resource service type (Price of each type of Required Resource service can be published on the marketing place and allow online subscription) |
| Solution 7 | Multiple -based on QoS requirement type of the traffic | Additional parameter traffic type (potential value audio, video, haptics) | Allow online subscription based on traffic type (Price of each type of traffic can be published on the marketing place and allow online subscription) |
| Summary | - Different operator may have different business policy about how sell their network resource, either offline or online, same price for all QoS type, or different prices on different types (based on traffic type or required resource type associated with traffic type). Standard should allow such a flexible for operator to design their business mode. |  |  |

2.3 Conclusion

It is suggest to merge solution 6 and solution 10 and solution 7 based on the following principle:

- Allow both single or multiple resource adaption services to meet with different business case in the standard from service design perpective. It is up to operator decision whether to support one of them or both business mode.

- Allow single API to support multiple resource adaption services, using a new paramer service type with different values to activiate the request QoS resource (traffic required resource based, or traffic type based).

3. Proposal

It is proposed to agree the CR to 3GPP TR 23.700-35 v20.0.0 in TDoc S6-254039.