3GPP TSG-SA WG6 Meeting #69	S6-254046

Wuhan, P.R.China, 13th – 17th October 2025	(revision of S6-253xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0055 | rev | -- | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The clarification on the FL member registration/deregistration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI-20 |  |  |  |  |  | Date: |  |  | 2025-10-6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are 3 type of FL member including VAL server, AI/ML server and AI/ML client. The FL member registration is required and the precondition of any FL member being selected to participate certain FL task. The whole registration procedure should cover two phases: 1.Registration authorization phase: A FL member makes registration request to get authorizated by AI/ML server. 2.Registration information storage/update phase: AI/ML server update the FL member information to ML repository. However, the following issues existed in current TS. - Only registration information storage/update phase is described for AI/ML server and VAL server type of FL member in section 8.4. Registration authorization phase for these types of FL member is missing. - Only registration authorization phase is described for AI/ML client in 8.7. The registration information update phase for AI/ML client is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Rephrase the wording in section 8.4, to make it clear the procedure is for registration information update phase and clarify the re-use of AI/ML client registration authorization phase for VAL server and AI/ML server type of FL member. - Calrify the FL member information update in 8.4 can be re-used for AI/ML client type of FL member in section 8.7. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TR is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.4, section 8.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.4	Support for FL member registration

8.4.1	General

This clause provides the procedures for the registration, registration update, and deregistration of candidate FL members in the ML repository which serves as service registry for the FL members undertaking a task related to the ML model lifecycle, such as ML model local training. Candidate FL members can be application layer entities at the server side (e.g., VAL server, AIMLE server), which can potentially be selected as FL clients or FL server for a particular ASP /vertical requirement, or at the client side (e.g., AIMLE client), which can potentially be selected as FL clients. If VAL server or the AIMLE client is the candidate FL member, it registers indirectly via AIMLE server to the ML repository.

8.4.2	Procedure on FL member registration information storage

Figure 8.4.2-1 illustrates the procedure where the registration of a candidate FL member happens via the ML repository, serving as AIML service registry.

Figure 8.4.2-1: Procedure for registration on FL member registry

0. The candidate FL member (e.g., VAL server or AIMLE client, or another AIMLE server) sends an FL member registration request to AIMLE server and gets authorization based on section 8.7. When the candidate FL member is VAL server or another AIMLE server, it should perform the same function as AIMLE client in the related procedure defined in section 8.7.

1.	AIMLE server sends an FL member registration information update request to the ML repository which acts as the AIML service information registry.

2.	The ML repository validates the received request and generates the identity and other security related information for all the FL members listed in the request.

3.	The ML repository sends the generated information in the FL member registration information update response message to the AIMLE server.

8.4.3	Procedure on FL member registration information update

Figure 8.4.3-1 illustrates the procedure where the registration update of a candidate FL member happens via the ML repository.

Figure 8.4.3-1: Procedure for registration update

0. The candidate FL member (e.g., VAL server or AIMLE client, or another AIMLE server) sends an FL member registration update/deregistration request to AIMLE server and gets authorization based on section 8.7.2.3 or section 8.7.2.4.

1.	The candidate FL member (VAL server via AIMLE server or AIMLE Server) sends an FL member registration update request to the ML repository acting as AIML service registry for updating the registration of the FL member.

2.	The ML repository validates the received update request and generates the identity and other security related information for the FL member.

3.	The ML repository sends the generated information in the FL member registration update response message to the candidate FL member.

The procedure for AIMLE client to update its registration to ML repository as FL member is introduced in clause 8.7.2.3.

8.4.4	Information flows

8.4.4.1	General

The following information flows are specified for supporting the FL member registration information storage and registration information update based on clause 8.4.2 and clause 8.4.3.

8.4.4.2	FL member registration information storage request

Table 8.4.4.2-1 describes information elements for the FL member registration information storage request AIMLE server to the ML repository/registry.

Table 8.4.4.2-1: FL member registration information storage request

| Information element | Status | Description |
| --- | --- | --- |
| Security information | M | Information for ML repository to validate the registration information storage request. |
| FL member list | M | The information of registered FL members, which are authorized as candidate FL member |
| >FL member ID | M (NOTE) | The identifier of the candidate FL member. This can be the VAL server ID, the EAS ID, the AIMLE server ID, AIMLE client ID based on which entity is the candidate FL member. |
| >FL member supported AI/ML role | M (NOTE) | The supported AI/ML role of FL member which can be used as FL server or FL client. |
| >FL member capabilities | O (NOTE) | FL member capability information (e.g., ML application type, allowed resource usage level). |
| >FL member velocity | O | Indicates the FL member velocity (e.g., mobile or static). |
| >FL member Location information | O | Indicates the location information of the UE (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses). |
| >FL member Availability schedule | O (NOTE) | Availability schedule of the FL member for a certain FL member. |
| >Supported ML model ID list | O | The list of ML model IDs for which the FL member can be used. |
| >Area of Interest | O | The area of interest (i.e., location co-ordinates) for which the registration applies. |
| >Time validity | O | The time validity for the registration of the FL member. |
| NOTE: At least one of these shall be present. |  |  |

8.4.4.3	FL member registration information storage response

Table 8.4.4.3-1 describes information elements for the FL member registration information storage response to AIMLE server from the ML repository/registry.

Table 8.4.4.3-1: FL member registration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the registration request (positive or negative acknowledgement). |

8.4.4.4	FL member registration information update request

Table 8.4.4.4-1 describes information elements for the FL member registration information update request from AIMLE server to the ML repository/registry.

Table 8.4.4.4-1: FL member registration information update request

| Information element | Status | Description |
| --- | --- | --- |
| Security information | M | Information for ML repository to validate the registration information update request |
| FL member list to be updated | M | The information FL member to be deleted due to deregistration or to be added due to registration update |
| >FL member ID | M | The identifier of the candidate FL member for which the update applies. This can be the VAL server ID, the EAS ID, the AIMLE server ID based on which entity is the candidate FL member. |
| >Updated registration parameters | O | The updates of the registration parameters if registration information is updated |
| >> FL member supported AI/ML role | O (see NOTE) | The updated supported AI/ML role of FL member which can be FL server, FL client. |
| >> FL member Availability schedule | O (see NOTE) | Updated Availability schedule of the FL member. |
| >> supported ML model ID list | O (see NOTE) | The updated list of ML model IDs for which the FL member can be used |
| > >area of interest | O (see NOTE) | The updated area of interest for the registration update. |
| > >time validity | O (see NOTE) | The update time validity for the FL member registration update. |
| >de-registration parameter | O | The indication of FL member ID being deregistered. |
| NOTE: At least one of these information elements shall be present. |  |  |

8.4.4.5	FL member registration update response

Table 8.4.4.5-1 describes information elements for the FL member registration information update response to AIMLE server from the ML repository/registry.

Table 8.4.4.5-1: FL member registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the registration update request (positive or negative acknowledgement). |

* * * Next Change * * * *

8.7AIMLE client registration

8.7.1	General

Prior to participation in AI/ML operations, AI/ML capable UEs register to an AIMLE server and provide an AIMLE client profile and optionally a list of supported services. The AIMLE server uses information provided by the AIMLE client to discover and select suitable AIMLE clients for requested AI/ML operations.

The following clauses specify procedures, information flows, and APIs for AIMLE client registration.

The procedure in this 8.7.2 and signalling flow 8.7.3 can be used for the FL member (including AIMLE client, AIMLE server, VAL) registration to AIML sever as required by section 8.4.

8.7.2	Procedures

8.7.2.1	General

The following are supported for AIMLE client registration:

-	AIMLE client registration procedure;

- 	AIMLE client registration update procedure; and

-	AIMLE client de-registration procedure

8.7.2.2	AIMLE client registration

Pre-conditions:

1.	The AIMLE client has been pre-configured or has discovered the address (e.g., URI) of the AIMLE server.

2.	The AIMLE client has been pre-configured with an AIMLE client profile.

Figure 8.7.2.2-1: AIMLE client registration

1.	The AIMLE client sends an AIMLE client registration request to the AIMLE server, the registration request includes information as described in Table 8.7.3.2-1. The AIMLE client indicates in the registration request its AI/ML capabilities such as supported ML model types and supported AI/ML operations, supported AIMLE client task capabilities with compute and task performance capabilities to assist with performing AIMLE client discovery and AIMLE client selection.

2.	The AIMLE server validates the registration request and performs an authentication and authorization check to determine if the AIMLE client is permitted to register to the AIMLE server and participate in AI/ML operations. Upon successful authorization, the AIMLE server saves the context of the AIMLE client registration in the ML repository.

3.	The AIMLE server returns an AIMLE client registration response to the AIMLE client with the status of the request.

8.7.2.3	AIMLE client registration update

Pre-conditions:

1.	The AIMLE client has already registered with the AIMLE Server.

Figure 8.7.2.3-1: AIMLE client registration update

1.	The AIMLE client sends an AIMLE client registration update request to the AIMLE server, the registration update shall include registration identifier, updated list of supported services and updated AIMLE client profile as described in Table 8.7.3.4-1. The AIML client removes services from the list of services identified by Service ID(s) when it ceases to provide a service and adds to the service when it starts to provide a service.

2.	The AIMLE server validates the registration update and performs an authentication and authorization check. Upon successful authorization, the AIMLE server updates the context of the AIMLE client registration in the ML repository.

3.	The AIMLE server returns an AIMLE client registration update response to the AIMLE client with the status of the update.

8.7.2.4	AIMLE client de-registration

Pre-conditions:

1.	The AIMLE client has already registered with the AIMLE Server.

Figure 8.7.2.4-1: AIMLE client de-registration

1.	The AIMLE client sends an AIMLE client de-registration request to the AIMLE server. The request shall include registration identifier as described in Table 8.7.3.6-1.

2.	The AIMLE server validates the deregistration request and performs an authentication and authorization check to determine whether the AIMLE client can deregister from AI/ML operations. Upon successful authorization, the AIMLE server removes the context of the AIMLE client registration from the ML repository.

3.	The AIMLE server returns an AIMLE client de-registration response to the AIMLE client with the status of the request.

8.7.3	Information flows

8.7.3.1	General

The following information flows are specified for AIMLE client registration:

-	AIMLE client registration request and response;

-	AIMLE client registration update request and response; and

-	AIMLE client de-registration request and response

8.7.3.2	AIMLE client registration request

Table 8.7.3.2-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration request.

Table 8.7.3.2-1: AIMLE client registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of supported profiles | M | Supported AIML client profile(s). For each client profile provided in the list, the supported service information is provided. |
| > AIMLE client profile | M | Information about the capability of the AIMLE client to support AI/ML operations for the VAL service ID as detailed in Table 8.7.3.2-2. |
| > List of supported services | M | List of VAL service IDs with corresponding permissions. |
| > VAL service ID | M | The identifier of the VAL service. |
| > Service permission level | O | Service permission level (e.g., premium resource usage, standard resource usage, limited resource usage). |

Table 8.7.3.2-2: AIMLE client profile

| Information element | Status | Description |
| --- | --- | --- |
| Supported AI/ML model types | O | AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks). |
| Supported AI/ML operations | M | AI/ML operations supported by the AIMLE client such as ML model training, model transfer, model inference, model offload, model split, continue perform intermediate AI/ML operation/task). |
| AIMLE client time schedule configurations | O | Indicates the availability schedule of the AIMLE client for the AIML service, e.g., the AIMLE client is (not) available to participate in the AIML operations (e.g., ML model training) in the given time slot(s) and/or day(s) of the week. |
| AIMLE client location configurations | O | Indicates the location-based configurations of the AIMLE client for the AIML service, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID. |
| AIMLE client capabilities | M | AIMLE client capability information (e.g., ML application type, allowed resource usage level). |
| Dataset availability | O | Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers. |
| Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g., raw data), supported data processing capabilities (e.g., processed data), and supported exploratory data analysis functions. |
| AIMLE client task capability | O See NOTE | Indicates the AIML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., green task, energy-efficient, low costs) |
| NOTE: The Green and Energy-efficient task performance may not be applicable to a UE. |  |  |

8.7.3.3	AIMLE client registration response

Table 8.7.3.3-1 shows the AIMLE client registration response sent by the AIMLE server to the AIMLE client.

Table 8.7.3.3-1: AIMLE client registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the registration was successful. |
| > Registration ID | M | The identifier of the registration. |
| > Expiration time | O | Indicates the expiration time of the updated registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated registration never expires. |
| Failure response | O (NOTE) | Indicates that the registration request failed. |
| > Cause | O | Indicates the cause of registration request failure |
| NOTE: One of the IEs shall be present. |  |  |

8.7.3.4	AIMLE client registration update request

Table 8.7.3.4-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration update request.

Table 8.7.3.4-1: AIMLE client registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | Identifier of the existing registration for which the update request applies. |
| AIMLE client profile | O | Update information about the capability of the AIMLE client to support AI/ML operations as detailed in Table 8.7.3.2-2. |
| List of supported services | O | Update to supported service information. Services identified by their VAL Service ID are either removed from the list when the AIMLE client ceases to provide a service and added when the AIMLE client starts to provide a service. |
| > VAL service ID | M | The identifier of the VAL service. |
| > Service permission level | O | Service permission level (e.g., premium resource usage, standard resource usage, limited resource usage). |

8.7.3.5	AIMLE client registration update response

Table 8.7.3.5-1 shows the AIMLE client de-registration update response sent by the AIMLE server to the AIMLE client.

Table 8.7.3.5-1: AIMLE client registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the registration update request was successful. |
| > Expiration time | O | Indicates the expiration time of the updated registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated registration never expires. |
| Failure response | O (NOTE) | Indicates that the registration update request failed. |
| > Cause | O | Indicates the cause of registration update request failure |
| NOTE: One of the IEs shall be present. |  |  |

8.7.3.6	AIMLE client de-registration request

Table 8.7.3.6-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client de-registration request.

Table 8.7.3.6-1: AIMLE client de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | Identifier of the existing registration for which the update request applies. |

8.7.3.7	AIMLE client de-registration response

Table 8.7.3.7-1 shows the AIMLE client de-registration response sent by the AIMLE server to the AIMLE client.

Table 8.7.3.7-1: AIMLE client de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the de-registration request was successful. |
| Failure response | O (NOTE) | Indicates that the de-registration request failed. |
| > Cause | O | Indicates the cause of de-registration request failure |
| NOTE: One of the IEs shall be present. |  |  |

* * * End of Changes * * * *