3GPP TSG-SA WG6 Meeting #69	S6-254051

Wuhan, China 13th – 17th August 2025

Source:	Pengcheng Laboratory

Title:	Pseudo-CR on Updates to Solution#2

Spec:	3GPP TR 23.700-51 v0.3.0

Agenda item:	9.5

Document for:	Approval

Contact:	Yang Li, liy24@pcl.ac.cn

1. Introduction

This solution enhances Solution #2 by introducing the VAL service category into the SEALDD-enabled data transmission quality guarantee request message in Step 1.​

2. Reason for Change

The current solution estimates required network transmission parameters (such as bandwidth, periodicity, etc.) based on media configuration information (e.g., resolution, FPS, GOP). However, identical media configurations can entail fundamentally different network requirements across various types of XR services. Consider a 2K/60FPS video stream as an example:

- In a VR service, where it functions as part of an immersive panoramic video, the demand is for extremely high and stable downlink bandwidth, while tolerating relatively higher latency.

- In an AR service, where it serves as a real-time pass through video crucial for precise alignment of virtual objects with the real world without drift, there are stringent requirements for uplink bandwidth and ultra-low latency.

- In a CG service, where it represents interactive gameplay, there is extreme sensitivity to bidirectional latency (particularly downlink) and network jitter, as any delay directly impacts operational feedback.

Explicitly specifying the VAL Service Category provides the network with crucial business semantics. This enables the system to transcend mere bitstream transmission, understand the ultimate purpose of the data flow, and consequently deliver the most targeted assurance strategies for diverse service experiences.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51 v0.3.0.

* * * First Change * * * *

#### 5.3.2.1	SEALDD enabled data transmission quality guarantee based on traffic characteristics

Figure 5.3.2.1-1: SEALDD enabled data transmission quality guarantee based on traffic characteristics

1.	VAL server sends request for SEALDD enabled data transmission quality guarantee based on traffic characteristics. The request may include the VAL service category (e.g., VR, CG, and AR) and related media configuration information (i.e., Video: Resolution, encoded method (e.g., Compression Ratio, Color Encoding Model (bits per pixel, bpp)), Frames Per Second (FPS), group of pictures (GOP); Audio: sampling rate, bit depth, number of channels; Haptic: sampling rate, etc.).

2a.	SEALDD server calculates the traffic characteristics based on received VAL service category and related media configuration information.

2b.	If the received media information is not enough to calculate the traffic characteristics, the SEALDD server request analytics or prediction of user plane performance (i.e. average/maximum traffic rate) from NWDAF (or via NEF) as defined in TS 23.288[9], clause 6.14.4.

NOTE: the Bitrate calculation based on traffic rate is implementation based, depending on specific transmission protocol.

3a.	Based on the traffic characteristics from step2, SEALDD server could do the transmission enhancement by request the rate control based on the bitrate. The traffic characteristics could be used to calculate the suggested traffic transmission bandwidth to the SEALDD client, as defined in TS 23.433[6], clause 9.8.2.1, step7.

3b.	SEALDD server request AF Session with required QoS as defined in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3] can be reused to provision the protocol description and the calculated Traffic Periodicity, bitrate.

* * * End of Change * * * *