3GPP TSG-SA WG6 Meeting #69	S6-254064

Wuhan, China 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0070 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SM data source triggering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-APP |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The SM data source discovery procedure currently triggers the SM data sources to immediately report data to the SM data source consumer. In some cases, it is required or preferable to trigger the data reporting independently of discovery. Enhancements are proposed to enable triggering SM data reporting after discovery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - - Enhancements to SM data source discovery to indicate deferred SM data reporting. - - New procedure to trigger the SM data reporting for discovered data sources. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Use cases requiring defered SM data reporting cannot be realized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.5.2, 9.5.5.3.1, 9.5.5.3.2, 9.5.5.3.3, 9.5.x (new), 9.5.x.1 (new), 9.5.x.2 (new), 9.5.x.3 (new), 9.5.x.3.1 (new), 9.5.x.3.2 (new), 9.5.x.3.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.5.5.2	Procedure

Figure 9.5.5.2-1 depicts the procedure for an authorized VAL server to discover SM data source(s).

Pre-conditions:

1.	The VAL server has received information (e.g. URI, IP address) related to the SEAL SM server;

2.	The VAL server has received security credentials authorizing it to communicate with the SEAL SM server.

Figure 9.5.5.2-1: SM data source discovery procedure

1.	The requestor (e.g, VAL server or SEAL server) sends a SM data source discovery request to the SEAL SM server. The request includes a requestor ID, security credentials, and may include discovery filters, endpoint information for receiving spatial mapping information from discovered SM data sources, and a deferred reporting indication. Discovery filters inlcude the area of interest (e.g., three-dimensional area, localization information, availability, data source update interval, mobility information) and requirements on spatial mapping information as defined in clause 9.5.5.3.1.

NOTE 1:	How the requestor determines the requested area of interest is implementation specific.

2.	The SEAL SM server authorizes the requestor and validates the request. If the request is authorized, the SEAL SM server determines one or more registered SM data source(s) according to the discovery filters.

The SEAL SM server evaluates if the SM data source location is in accordance with the area of interest included in the discovery request to determine the SM data source(s). The SEAL SM server can obtain UE location information and velocity information from the 3GPP core network by invoking the 3GPP Core Network Location Services exposed by the NEF as described in 3GPP TS 23.273 [6] and 3GPP TS 23.502 [5], or by invoking the SEAL Location Management APIs as described in 3GPP TS 23.434 [4]. If the request includes a deferred reporting indication, the SEAL SM server allocates and stores a deferred session identifier, and associates the determined SM data sources with the deferred session identifier.

3.	The SEAL SM server sends a SM data source notification to trigger the determined SEAL SM client(s) to send spatial mapping information. The notification includes details about the requested spatial mapping information and endpoint information for sending the spatial mapping information to the requestor, and may include a deferred session identifier. The SEAL SM client provides the information included in the notification to the VAL client that will send the spatial mapping information. If the notification includes a deferred session identifier, the VAL client does not provide spatial mapping information, stores deferred session identifier and information included in the notification, and waits to receive a triggering request including the deferred session identifier as described in clause 9.5.x.

4.	The SEAL SM server sends a SM data source discovery response to the requestor. The response includes information about SM data source(s) when at least one SM data source has been determined. If no SM information provided was determined, the SEAL SM server sends a failure indication with a reason for failure. If the SEAL server allocated a deferred session identifier,  the response includes the deferred session identifier that the requestor needs to trigger SM data reporting for the discovered sources as described in clause 9.5.x.

NOTE:	In this release, SM specific data source procedures are used. If and when generic data source related procedures are available, usage of generic procedures will be considered.

* * * Next Change * * * *

##### 9.5.5.3.1	SM data source discovery request

Table 9.5.5.3.1-1 describes information elements in the SM data source discovery request from the VAL server to the SEAL SM server.

Table 9.5.5.3.1-1: SM data source discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server) |
| Security credentials | M | The security credentials of the requestor |
| SM data reporting endpoint | O | The endpoint information where the triggered spatial mapping data source sends the SM data. |
| Deferred reporting indication | O | An indication to defer SM data reporting until a reporting trigger is received. |
| Discovery filters | O | Set of characteristics to determine SM data source(s) in the area of interest |
| > Area of interest | O | Three-dimensional area of interest or localization information to discover SM data source(s) in the area |
| > SM information details | O | The SM information details to match |
| >> SM data identifier | O | The identifier of SM information |
| >> SM data type (NOTE) | O | The type of SM information (e.g., LiDAR camera, RGB-D camera, high-resolution cameras, etc.) that is provided by the application layer. |
| >> SM data format | O | The format of a SM information (e.g., raw, processed, etc.) |
| >> Position information | O | Indicates the Three dimensional area of interest or location information of the SM data source |
| >> Availability State information | O | Indicates the time period availability information of the SM data source. |
| >> Data source update interval information | O | Indicates the rate at which data source updates or generates the data. It describes how often the new data is captured or collected. e.g., every x milliseconds generates data. |
| >> Mobility information | O | Indicates the mobility information of the SM data source.e.g., fixed, low or high mobility |
| NOTE: The definition of SM data types is application specific (e.g., VAL client) for the current release. |  |  |

* * * Next Change * * * *

##### 9.5.5.3.2	SM data source discovery response

Table 9.5.5.3.2-1 describes information elements in the SM data source discovery response from the SEAL SM server to the SM data source consumer (e.g., VAL server or SEAL server).

Table 9.5.5.3.2-1: SM data source discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the discovery request was successful |
| > SM data source profiles | O | The list of SM data source profiles as described in Table 7.3.3.2-1. |
| > Deferred session ID | O (NOTE) | The identifier of a deferred SM data reporting session. |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of the failure |
| NOTE: The IE shall be present if deferred SM data reporting is requested |  |  |

* * * Next Change * * * *

##### 9.5.5.3.3	SM data source notification

Table 9.5.5.3.3-1 shows the notification sent by a SEAL SM server to a SEAL SM client for triggering a spatial mapping information session from a SM data source.

Table 9.5.5.3.3-1: SM data source notification

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., SEAL SM server, VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| SM data source ID | M | The identifier of the SM data source. |
| SM information reporting session | M | The characteristics of SM information reporting session. |
| > Deferred session ID | O | The identifier of a deferred SM data reporting session. |
| > SM data identifier | M | The identifier of spatial SM data to report. |
| > SM data type (NOTE) | O | The type of SM information data to report. (e.g., LiDAR camera, RGB-D camera, high-resolution cameras, etc.) that is provided by the application layer. |
| > SM data format | O | The format of a SM information data to report. (e.g., raw, processed, etc.). |
| > SM duration | O | The duration for which the SM data is reported. |
| > SM frequency | O | The frequency at which the SM data is reported. |
| > SM endpoint | M | The endpoint where to report the SM data. |
| NOTE: The definition of SM data types is application specific (e.g., VAL client) for the current release. |  |  |

* * * Next Change * * * *

### 9.5.x	SM data source triggering

#### 9.5.x.1	General

The SM data source triggering procedure allows an SM data source consumer (e.g., VAL server or SEAL server) to trigger SM data reporting for an SM data reporting session.

#### 9.5.x.2	Procedure

Figure 9.5.x.2-1 depicts the procedure for an authorized SM data source consumer to trigger SM data reporting.

Pre-conditions:

1.	The SM data source consumer has received information (e.g. URI, IP address) related to the SEAL SM server;

2.	The SM data source consumer has received security credentials authorizing it to communicate with the SEAL SM server.

3.	The SM data source consumer has performed SM data source discovery as described in clause 9.5.5.2 and has obtained a deferred session identifier.

Figure 9.5.x.2-1: SM data source triggering procedure

1.	The requestor (e.g, VAL server or SEAL server) sends a SM data source triggering request to the SEAL SM server. The request includes a requestor ID, security credentials, and a deferred session identifier.

2.	The SEAL SM server authorizes the requestor and validates the request. If the request is authorized, the SEAL SM server determines the SM data source(s) associated with the deferred session identifier included in the request.

3.	The SEAL SM server sends a SM data source triggering notification to the SEAL SM client(s) of the determined SM data source(s) to trigger the reporting of SM data (e.g., spatial mapping information). The notification includes the deferred session identifier included in the request. Upon receiving the notification, the SEAL SM client validates the deferred session identifier. If the deferred session identifier is valid, the SEAL SM client sends the SM data according to the configuration associated with the deferred session identifier as described in clause 9.5.5.2.

4.	The SEAL SM server sends a SM data source triggering response to the requestor. The response includes an indication of success if the deferred session identifier is valid. Otherwise, the SEAL SM server sends a failure indication and may include a cause for failure.

#### 9.5.x.3	Information flows

##### 9.5.x.3.1	SM data source triggering request

Table 9.5.x.3.1-1 describes information elements in the SM data source triggering request from the SM data source consumer to the SEAL SM server.

Table 9.5.x.3.1-1: SM data source triggering request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server) |
| Security credentials | M | The security credentials of the requestor |
| Deferred session ID | M | The identifier of a deferred session to trigger. |

##### 9.5.x.3.2	SM data source triggering response

Table 9.5.x.3.2-1 describes information elements in the SM data source triggering response from the SEAL SM server to the SM data source consumer.

Table 9.5.x.3.2-1: SM data source triggering response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the triggering request was successful |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of the failure |

##### 9.5.x.3.3	SM data source triggering notification

Table 9.5.x.3.3-1 shows the notification sent by a SEAL SM server to a SEAL SM client for triggering a deferred session.

Table 9.5.x.3.3-1: SM data source triggering notification

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., SEAL SM server, VAL server). |
| Security credentials | M | The security credentials of the requestor. |
| Deferred session ID | M | The identifier of a deferred session to trigger. |

* * * End of Changes * * * *