3GPP TSG-SA WG6 Meeting #69	S6-254072

Wuhan, China 13th – 17th October 2025

Source:	ETRI

Title:	Pseudo-CR on Solution for Sensing service availability

Spec:	3GPP TR 23-700-15 0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Jong-Hwa Yi <jhyiee@etri.re.kr>

1. Introduction

This pCR proposes a solution for KI#1 on FS_Sensing_APP(Study on utilization of sensing results for vertical applications).

2. Reason for Change

3GPP TS 22.137 has defined a set of functional requirements for sensing services where the application enabler layer needs to support sensing management functionalities for vertical applications.

- Subject to operator’s policy, the 5G network may provide secure means for the operator to expose information towards trusted third-party on whether a given sensing service is available and the estimated quality of the given service for a certain geographic area and time.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23-700-15 0.1.0.

* * * First Change * * * *

6	Solutions

6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #X | X |  |  |  |
| Sol #Y |  |  |  |  |
| Sol #... |  |  |  |  |

* * * Next Change * * * *

6.x	Solution #x: Sensing Service Availability

6.x.1	Solution description

This solution addresses the key issue #1: Application enablement architecture enhancements to support utilization and exposure of sensing results. The clause 6.x.2 describes the procedure of sensing service availability discovery request/response.

6.x.2	Procedures

The sensing service availability discovery procedure enables the VAL server  to discover the availability of sensing services provided by the 5G system that can satisfy the VAL server’s sensing service requirements (e.g., sensing service type, KPI/QoS) within a given geographical area and time.

Pre-conditions:

- SEAL server provides Sensing Management services as application enablement services to support sensing- specific vertical applications.

- The VAL Server is registered and authorized to use the SEAL services for sensing.

- Sensing capability information is available in the 3GPP Core Network and can be retrieved by the SEAL Server.

Figure 6.x.2-1: Sensing service availability discovery procedure

- The VAL server sends a Sensing service availability discovery request to the SEAL server (i.e., Sensing Management server) to discover whether sensing services that satisfy the VAL server’s sensing requirements are available. This request may describe sensing service requirements of the VAL server including sensing service type (e.g., presence, object detection, localization, motion, identification, event detection, etc), KPI, service area of interest, and time.

- The SEAL Server (i.e., Sensing Management server) verifies whether the VAL server is authorized to request the sensing service. If the request is authorized, the SEAL server stores the sensing service requirements contained in the request message.

- Based on the VAL Server’s sensing service requirements, the SEAL Server (i.e., Sensing Management server) requests information on the sensing capabilities provided by the 3GPP Core Network. To do this, the SEAL Server translates or maps the VAL Server’s sensing service requirements to the parameters used by the 3GPP Core Network (e.g., type of sensing operation, sensing coverage, sensing duration, sensing QoS).

- Using the obtained sensing capability information in step 3, the SEAL Server (i.e., Sensing Management server) evaluates whether the available sensing services can satisfy the VAL Server’s sensing service requirements. Ths evaluation results may be expressed as one of the following: fully supported, partially supported, not supported, or undetermined.

- The SEAL server sends the Sensing service availability discovery response to the VAL server including the evaluation result and related information on the availability status and the estimated KPI/QoS.

6.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Changes * * * *