3GPP TSG-SA WG6 Meeting #69	S6-254118

Wuhan, China 13th – 17th October 2025	(revision of S6-25xxxx)

Source:	Telefónica, Apple, Fogus, UMA

Title:	TR 23.947 Overview of Frameworks CAPIF

Spec:	3GPP TR 23.947

Agenda item:	9.14

Document for:	Approval

Contact:	w_featherstone@apple.com

1. Introduction

This contribution provides the Overview of Frameworks CAPIF text for the Guidelines for 3GPP Application Enablement usage (AppEnable_EXT) technical report.

2. Reason for Change

This contribution proposes text for the Overview of Frameworks CAPIF of 3GPP TR 23.947.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.947, version 0.1.0.

* * * First Change * * * *

## 4.2	Common API Framework (CAPIF)

The Common API Framework (CAPIF) is a 3GPP framework that provides common functions for secure API exposure, discovery, onboarding, and authorization across multiple domains. Its primary role is to harmonize how APIs of network services (and other service platforms) are published and consumed, offering a single point of entry for API invokers (consumers) and a consistent method for API providers to expose their capabilities. By introducing a unified northbound API framework, CAPIF prevents fragmentation – application developers and third-party service providers can use one standardized approach to interact with operator APIs, rather than dealing with bespoke interfaces for each network or service. CAPIF was first specified in 3GPP Release 15 and has since been adopted for exposing APIs from core network functions (e.g. NEF for network capabilities, MBMS API, etc.) as well as application-layer services, in a secure and controlled manner.

The CAPIF architecture is based on a service-oriented model with three main entity groups as illustrated in Figure 4.2.1, specifically a CAPIF Core Function (CCF), the API Invoker and one or more API Provider functions:

- CAPIF Core Function (CCF): The central broker and repository of the framework. The CCF handles the onboarding of API providers and API invokers (establishing trust and identity for each), maintains the API catalogue for discovery, and provides common services like authentication, authorization token issuance, logging and charging for API usage. The CCF essentially acts as the authoritative service registry and policy enforcement point for all API interactions in CAPIF.

- API Exposing Function (AEF): A function on the provider side that hosts one or more service APIs and exposes them to external consumers. The AEF is the entry point through which an API invoker accesses the provider’s service API; it enforces the service’s access policies and handles the runtime API calls (e.g. rate limiting, protocol translation if needed). In practice, an AEF often “wraps” an existing 3GPP network function or application server to present its APIs in a standardized way.

- API Publishing Function (APF): A provider-side function responsible for publishing the API definitions and endpoints of the AEF to the CAPIF core’s catalogue. When an API is made available, the APF registers the API’s meta-information (e.g. name, version, resource paths, supported operations) with the CCF, so that prospective invokers can discover it. This publishing step abstracts the service’s location and details behind the common CAPIF discovery interface.

- API Management Function (AMF): A provider-side management entity that allows the API provider to administer and monitor their APIs. The AMF interfaces with the CCF for reporting and retrieving logs of API invocations (for auditing), subscribing to events (e.g. notifications of certain API usage or threshold breaches), and updating any API access policies or configurations. Through the AMF, a provider can enforce service-specific policies and collect metrics, while relying on CAPIF for uniform log and event formats.

- API Invoker: The consuming entity (application or client) that uses CAPIF to find and invoke service APIs. An API Invoker could be an external third-party application or another network function/application within the operator’s environment. The invoker first onboards with the CAPIF core to establish its identity and credentials, then queries the API catalogue to discover available services, and finally invokes the desired API on the corresponding AEF, obtaining authorization from the CCF as required. In a developer context, the API Invoker role is played by the developer’s application or service that seeks to utilize operator or third-party APIs in a secure manner.

Figure 4.2.1: Functional model for the CAPIF as defined in 3GPP TS 23.222 [2]

CAPIF operates through a set of well-defined procedures and interfaces (reference points CAPIF-1/2/… etc., as per 3GPP TS 23.222 [2]). At a high level, the lifecycle of API exposure and consumption in CAPIF is as follows:

- Onboarding: Both API providers and API invokers perform onboarding with the CAPIF Core Function. Onboarding involves security bootstrapping and registration of roles. For example, an API provider (such as an Edge Configuration Server in an edge computing scenario) will register with the CCF to become a recognized provider, typically using credentials or certificates issued by the operator’s CAPIF authority. Similarly, an application acting as an API invoker (e.g. a vertical application server or a UE client) also onboards with the CCF to obtain a secure invoker identity. During onboarding, the CAPIF core may provision digital certificates or JSON Web Tokens (JWT) and record the entity’s public information (like its name, domain, and roles) in its database. This establishes a trust relationship: the CCF recognizes the entity as an authorized API provider or consumer in the system.

- API Publication: Once onboarded, an API provider exposes its service API through CAPIF. The provider’s API Publishing Function sends the API description to the CAPIF core (via the CAPIF publishing interface, e.g. CAPIF-4) to register the API in the catalogue. This publication typically includes the API’s endpoint URI (which may point to the AEF), the resource structure and methods, required security scopes or policies, and any other metadata (version, category, etc.). The CCF stores this information in an API directory that is accessible to invokers for discovery. For instance, in an EDGEAPP scenario, after an Edge Configuration Server (ECS) onboards, it would publish its available service APIs (like edge node registration API, configuration APIs) to CAPIF so that edge clients or other components can later find them. The CAPIF core may verify that the publishing entity is authorized to expose those APIs (e.g. checking that the provider is indeed entitled to publish an “Edge Configuration” service). Once published, the API is discoverable by all invokers within the scope of that CAPIF instance.

- Discovery: An API invoker queries the CAPIF Core Function to find a service API that it needs. Discovery can be done using various criteria (service name, category, provider, etc.). The CCF will return the details of the API provider’s endpoint (the AEF address and interface details) and information on how to invoke it – including the needed security credentials or tokens. CAPIF supports a federated discovery if multiple CCFs are interconnected, but within a single domain the CCF acts as the central lookup for all APIs. For example, an Edge Enabler Client (EEC) that needs to call a service on an Edge Application Server (EAS) can query CAPIF (via the CCF) to get the API endpoint and requirements for the EAS’s service API. This discovery process abstracts away hard coding of service endpoints; an application can dynamically find services by type, which is especially important in flexible deployments (like edge environments where the nearest service instance may change). The discovery response from CCF typically includes the base URI of the service, available operations, and the required authorization method (e.g. OAuth2 scopes).

- API Invocation with Authorization: Before invoking the target API, the API invoker obtains authorization from CAPIF. In CAPIF’s security framework, the CCF can function as an OAuth2 Authorization Server for API access tokens. The invoker (acting as an OAuth client) requests an access token from the CCF, proving its identity (using credentials established at onboarding) and specifying the API it wants to call. The CCF issues a JWT or OAuth2 token if the invoker is allowed to use that API, with appropriate scope/permissions encoded. Armed with this token, the invoker then sends the API request to the provider’s AEF (over the CAPIF-internal interface to the AEF, e.g. CAPIF-2) and includes the token in the HTTP authorization header. The AEF validates the token (often by verifying its signature and checking with CCF if needed) and, if valid, allows the request to be processed by the underlying service. This ensures that only properly onboarded and authorized applications can invoke protected APIs. CAPIF also supports additional security models – for example, a mode where a “Resource Owner” (end-user or service owner) involvement is required for authorization (termed Resource Owner Aware API Access, RNAA). In such cases, CAPIF’s Authorization Function and Resource Owner function coordinate to grant the invoker access to specific resources. These details aside, the normal flow for most developer use cases is that CAPIF handles token-based authentication under the hood, so the developer’s application just needs to request a token and attach it to API calls.

- Logging and Monitoring: As APIs are invoked, CAPIF provides a logging service. Each API call and response can be logged via the CAPIF core (or the provider’s AMF reporting to CCF), recording details such as who invoked which API, when, and with what result. These logs enable auditing and troubleshooting. CAPIF can also emit events or notifications – for instance, if an API provider wants to be notified when a new invoker subscribes to its API or when certain thresholds are exceeded, the framework can support that through an events interface. For charging purposes, the CCF can gather usage records which an operator might integrate with billing systems if the API usage is monetized. All these management aspects are abstracted from the app developer; they are handled by CAPIF and the network provider, ensuring that robust operational controls exist for exposed APIs.

For application developers, CAPIF greatly simplifies integration with 3GPP network services and other service platforms. Instead of dealing with separate authentication and discovery mechanisms for each API (e.g. one for location services, another for messaging, etc.), a developer can rely on CAPIF to provide a one-stop hub for all API interactions. For example, a developer building a fleet management application could use CAPIF to discover the operator’s “vehicle location API” and invoke it securely, without needing to know the internal details of the network’s location function – CAPIF mediates trust and access. Likewise, if the developer’s application provides its own API (say it’s an analytics service that the operator might call), the developer can publish this API via CAPIF so that it’s uniformly accessible to other authorized parties.

Crucially, CAPIF is designed to be used in conjunction with other frameworks: it is agnostic to the content of the APIs and focuses on the exposure mechanism. Both EDGEAPP and SEAL leverage CAPIF to expose and consume their respective service APIs. In an Edge Application Enablement deployment, the edge functions (like EES, ECS, EAS) utilize CAPIF for secure service discovery and invocation – for instance, an EEC retrieving an edge service API from the catalogue, or an EAS publishing its application API for clients. In the SEAL context, a SEAL server implementing group communication or location services acts as a CAPIF API provider so that applications can find those services through the common framework. Thus, CAPIF serves as the cross-cutting enabler that ties multiple service frameworks together, offering a consistent developer experience and unified security for all API-based interactions.

* * * End of Changes * * * *