3GPP TSG-SA WG6 Meeting #69	S6-254120

Wuhan, China 13th – 17th October 2025	(revision of S6-25xxxx)

Source:	Telefónica, Apple, Fogus, UMA

Title:	TR 23.947 Use case Crowd Counting Video Analytics

Spec:	3GPP TR 23.947

Agenda item:	9.14

Document for:	Approval

Contact:	w_featherstone@apple.com

1. Introduction

This contribution provides a use case on Crowd Counting Video Analytics text for the Guidelines for 3GPP Application Enablement usage (AppEnable_EXT) technical report.

2. Reason for Change

This contribution proposes text for a new use case for clause 5 of 3GPP TR 23.947.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.947, version 0.1.0.

* * * First Change * * * *

## 5.1	Crowd Counting Video Analytics

### 5.1.1	Use case Description

“Acme” company is developing a crowd counting video analytics application that leverages edge computing to process video in real time. The application consists of an Application Client (AC) running on a User Equipment (UE), which captures live video and streams it for analytics, and a video analytics application running on an Edge Application Server (EAS), which performs the crowd counting function.

The UE also hosts an Edge Enabler Client (EEC). The AC communicates with the EEC via the EDGE-5 reference point. The EEC hides all network complexity and interacts with the Edge Enabler Server (EES), the Edge Configuration Server (ECS), and the CAPIF Core Function (CCF) to enable the AC to consume the video analytics service.

As depicted in Figure 5.1.1-1, the topology considered for this use case includes:

- one CCF,

- one ECS,

- two EES (each serving one edge zone),

- two EAS (one per edge zone),

- one UE with EEC and AC.

UE

AC

Central Zone

Edge Zone 2

Edge Zone 1

CCF

ECS

EES-1

EES-2

EAS-1

EAS-2

UE

EEC

AC

EEC

UE Moving from Edge Zone 1 to Edge Zone 2

Figure 5.1.1-1: Crowd Counting Video Analytics use case topology

The UE starts attached in Edge Zone 1, where the AC streams video to EAS 1. When the UE moves into Edge Zone 2, the system triggers an Application Context Relocation (ACR) to transfer the session to EAS 2, preserving service continuity with minimal disruption. The AC itself is only aware that the EEC provides it with an updated service endpoint; all control-plane interactions are performed by the EEC.

### 5.1.2	Use case Realisation over SA6 Frameworks

### 5.1.3	Use case Flows

## 5.2	Use case X

### 5.2.1	Use case Description

### 5.2.2	Use case Realisation over SA6 Frameworks

### 5.2.3	Use case Flows

* * * End of Changes * * * *