3GPP TSG-SA WG6 Meeting #69	S6-254142

Wuhan, P.R.China, 13th – 17th October 2025	(revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.180 | CR | 0017 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add service flow in a new annex |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Generic_IOPS |  |  |  |  |  | Date: |  |  | 2025-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently 3GPP TS 23.180 only defines the procedures under IOPS mode. However the whole work flow about switching between the normal MCX system with the normal network and the IOPS MC system with IOPS 3GPP system is missing. Such work flow could help the reader to have a overall view of how a user obtains the MCX service when entering into IOPS mode and recovering from IOPS mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a overall work flow about switching between the normal MCX system with the normal network and the IOPS MC system with IOPS 3GPP system is missing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The relationship between the normal MCX system and the IOPS MC system, and how a user interact with the normal MCX system and the IOPS MC system when entering and leaving the IOPS mode is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

Annex B (informative):
Switching between normal MC system and IOPS MC system

This clause provides an overall description about switching between normal MC system with macro network and the IOPS MC system with the IOPS 3GPP system.

The figure B.x-1 provides an example of high-level workflow of switching between normal MC system and IOPS MC system.

Figure B.x-1: switching between normal MC system and IOPS MC system

1.	The MC service UE has attached to the macro core network and requested the network connection for MC service.

2.	MC service client accesses the MC service via the macro core network. The MC service client performs identity authentication, service authorization, affiliation, group call, private call etc.

3.	The IOPS-capable base station detects loss of backhaul and enters IOPS mode. Further the IOPS-capable base station indicates the MC service UE about the IOPS event.

4.	Then the MC service UE attaches to the local core network and requests network connection for MC service.

5.	The MC service client accesses the MC service including identity authentication, service authorization, affiliation, group/private call etc. via the local core network.

6.	The IOPS-capable base station detects loss of backhaul has been restored and indicates to the MC service UE.

7.	The MC service UE detaches from the local IOPS network, attaches to the Macro core network and requests the network connection for MC service.

8.	The MC service client accesses the MC service including Identity authentication, service authorization, affiliation, group/private call etc via the macro core network.

NOTE:	There is no service continuity during switching between the MC system and the IOPS MC system.