3GPP TSG-SA WG6 Meeting #69	S6-254177

Wuhan, China, 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0073 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add reference to Pose definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-10-6 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The term “Pose” is defined in TS 26.119 but the reference is not included in the current specifications leading to incomplete requirements, eventhough its referenced in the related TR 23.700-21. Hence, the current specification requires to include the reference where the term “Pose” is defined for clarity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The definition of Pose from TS 26.119 reference is included in the below clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear stage-2 requirements leading to incorrect stage-3 implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.3.5.2.1, 8.3.5.3.1, 9.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.156: "Mobile Metaverse Services; Stage 1".

[3]	3GPP TR 22.856: "Feasibility Study on Localized Mobile Metaverse Services".

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[6]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[7]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[8]	3GPP TR 26.119: "Media Capabilities for Augmented Reality".

* * * Next Change * * * *

##### 8.3.5.2.1	Subscribe

Figure 8.3.5.2.1-1 illustrates the spatial anchor subscription procedure.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

Figure 8.3.5.2.1-1: Spatial anchor subscription

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor subscribe request to the SAn server. The request includes a requestor identifier, security credentials, a notification endpoint, and event information for which the requestor is subscribing. The request may include an expiration time.

a.	The requestor may include the "discovery" event type to indicate the SAn server to notify the requestor when the SAn server detects events related to the management of spatial anchors (e.g., creation, update, deletion, spatial anchor state changes (e.g. active, inactive, expired)) according to discovery event filters and spatial anchor filters provided in the request. If the requestor is a SAn client, the request includes a VAL client identifier.

b.	The VAL server may include the "UE with spatial anchor interest" event type to indicate the SAn server to notify the VAL server when the SAn server detects UE(s) with interest in specific spatial anchors according to the spatial anchor of interest event filters and spatial anchor filters provided in the request.

c.	The requestor may include the "spatial anchor in range" event type to indicate the SAn server to notify the requestor when the SAn server detects that spatial anchors become in range or out of range of the subscriber according to range event filters provided in the request. Range event filters can include a range for detecting spatial anchors, an indication to enable persistent search, the pose (as defined in clause 6.2.2 of 3GPP TS 26.119 [8]) of the subscriber. If the requestor is a SAn client, the spatial anchor subscribe request includes a VAL client identifier.

2.	Upon receiving the request from the requestor, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server creates the subscription and stores the subscription information.

a.	If the subscribed event is for "discovery", the SAn server starts monitoring spatial anchors management events according to the discovery event filters and spatial anchor filters.

b.	If the subscribed event is for "UE with spatial anchor interest", the SAn server starts monitoring spatial anchors of interest according to the UE with spatial anchor interest event filters and spatial anchor filters. If the spatial anchors of interest have been discovered by a UE, the SAn server can obtain the UE location information from the 5GS or the SEAL Location Management service. Additionally, the SAn server can consider data analytics, such as UE mobility prediction. The SAn server may monitor the distance between UEs and the spatial anchor of interest to determine whether the UEs enters, exits or is within range of the spatial anchors of interest.

c.	If the subscribed event is for "spatial anchor in range" and the requestor is a SAn client, the SAn server starts monitoring the corresponding UE location and direction corresponding to the pose information according to the range event filters and spatial anchor filters. The UE location information can be obtained by using capabilities of the 5GS or the SEAL Location Management service. Additionally, the SAn server can consider data analytics, such as UE mobility prediction. The SAn server monitors the distance between UEs and the spatial anchor of interest to determine whether the spatial anchors become in-range or out-of-range of the UE. If persistent search is enabled, the SAn server monitors location of the user (or UE) and pose of the user, along with a direction of movement, and discovers a list of spatial anchors which becomes in-range or out of range of the user device based on the location of the user device, the pose of the user and in the user's field of view. For spatial anchors associated with trackable objects, the SAn server can determine the in-range or out-of-range of the UE with the current position of the objects which are being tracked by SEAL SM server. Additionally, SAn server can consider trackable object mobility prediction which is provided by SEAL SM server based on the data analytics for position, direction and velocity of the object.

NOTE:	How to do trackable object mobility prediction and estimate the duration of in-range object tracking is implementation specific and out of scope.

Editor's Note: Spatial map management procedures to request object tracking need to be provided.

3.	The SAn server sends a spatial anchor subscribe response to the requestor. If the SAn server has created the subscription, the response includes an indication of success, the subscription identifier and may include an expiration time; to maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the spatial anchor subscription expires. If the SAn server has not created the subscription, the response includes an indication of failure and may include a reason for failure.

* * * Next Change * * * *

##### 8.3.5.3.1	Spatial anchor subscribe request

Table 8.3.5.3.1-1 describes information elements for the spatial anchor subscribe request from the SAn client or VAL server to the SAn server.

Table 8.3.5.3.1-1: Spatial anchor subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL client ID | O | The identifier of the VAL client when the requestor is a SAn client. |
| Notification Target Address | M | Notification target address (e.g. URL) where the notifications should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription. |
| Event type | M | The type of event for which the requestor is subscribing. Supported event types include: - "discovery" - "UE with spatial anchor interest" - "spatial anchor in range" |
| Persistent Search (NOTE) | O | Indicates to enable persistent search. |
| Event detection filters | O | Event type specific detection filters. |
| > Discovery event filters | O | Event filters applicable to the "discovery" event type. The event filter is considered by the SAn server for triggering the notification and can include management events (e.g., creation, update, deletion). |
| > UE with spatial anchor interest event filters | O | Event filters applicable to “UE with spatial anchor interest” event type. The event filter is considered by the SAn server for triggering the notification and can include an indication for detecting UEs that have discovered spatial anchors, that are within range or become out of range of spatial anchors, or that enter or leave the range of a spatial anchor, and a value for the range. |
| > Range event filters | O | Event filters applicable to “spatial anchor in range” event type. The event filter is considered by the SAn server for triggering the notification and can include an indication for spatial anchors becoming in range of the UE, out of range of the UE, or both and a value for the range. |
| >> current location | O | Current location of the user and direction. |
| >> pose | O | Current pose of the user |
| >> application specific range | M | Specifies range within which the spatial anchors are monitored to be in range or out of range of the subscriber's field of view. |
| Spatial anchor filters | O | The characteristics of spatial anchors that the SAn server considers for triggering a notification. |
| > Spatial anchor ID list | O | Identifiers of the spatial anchors of interest. |
| > Service ID list | O | Identifiers of the associated application services of interest. |
| > Area of interest | O | The area of interest (e.g., topological and/or geographical). |
| NOTE: This IE can be included with "spatial anchor in range" event. |  |  |

* * * Next Change * * * *

### 9.4.2	Procedure

Figure 9.4.2-1 depicts the procedure for the authorized spatial map consumer (VAL server or SEAL SM client) to invoke spatial localization services. The service is provided by SEAL SM server.

Pre-condition:

1)	Spatial map is created and the authorized consumer is aware of the spatial map identity.

Figure 9.4.2-1: Spatial localization services

1)	The VAL server (or SEAL SM client) sends request to perform localization service in the spatial map(s) to the SEAL SM server. The request includes a requestor identifier, security credentials, and identity of the map(s) on which the action is to be performed. The request may include list of users (or UEs, objects) to localize. The request may include a list of UE-object association information for the SEAL SM server to identify associations between UEs and objects in a spatial map.

2)	The SEAL SM server authorizes the requestor (e.g. VAL server or SEAL SM client). If the requestor is not authorized then the SEAL SM server sends failure response. If the request is authorized: the SEAL SM server, identifies all users/UEs localization along with their three dimensional location and pose (as defined in clause 6.2.2 of 3GPP TS 26.119 [8]) in the spatial map. If area of interest IE is present then the SEAL SM server identifies the target users/UEs within the area of interest only, otherwise, the SEAL SM server identifies the target users for whole map (according to the area associated with spatial map identifier). If UE-object association information has been provided, the SEAL SM server can use this information to determine if a spatial map object is associated with a UE. If a UE-object association is determined, the SEAL SM server can use localization information of the associated object to determine user/UE localization in the spatial map, or use localization information of the associated UE to determine object localization in the spatial map.

3)	The SEAL SM server sends response to the VAL server (or SEAL SM client). The response includes the object localization and identity.

NOTE:	In this release, the localization of UE is considered. Localization of all objects will be considered in future release.

* * * End of Change * * * *