3GPP TSG-SA WG6 Meeting #69	S6-254317

Wuhan, China, 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0176 | rev | - | Current version: | 19.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clause 10.2.2.2 EN resolution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Editor's note: How SEALDD server knows the recipient UE information is FFS. Above EN is removed by adding a NOTE with detailed clarification that, the SEALDD server receives the UE ID in the Sdd_RegularTransmissionConnection_Establish request from the SEALDD client as defined in the clause 9.2.2.2 which is part of step 2. This UE ID is used by the SEALDD server while subscribing for S&F satellite operation information from the 5GC in step 5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | EN is removed and clarification added with a NOTE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Un resolved EN leads to incomplete requirement leading to incomplete feature implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.2.2.2	Client initiated data transmission for delay-tolerant satellite services

Figure 10.2.2.2-1 illustrates the procedure for establishing S&F data transmission connection, and the SEALDD facilitates the delay-tolerant satellite services to transmit data initiated by the VAL client to VAL server.

Pre-condition:

-	The VAL client is aware of the availability of delay-tolerant satellite services to transmit data.

Figure 10.2.2.2-1: Client initiated data transmission for delay-tolerant satellite services

1.	The VAL client decides to use SEALDD service for delay-tolerant traffic transfer.

2.	Same as steps 2-4 of clause 9.2.2.7.

3.	The SEALDD client provides the acknowledgment for the request made in step 1.

4.	The SEALDD client sends the UL data to the SEALDD server via the 3GPP network.

5.	The SEALDD server subscribes for monitoring event for the UE registered in S&F mode, from the 5GC as specified in clause 5.6.3.10 of 3GPP TS 23.682 [24] and is notified about the UE S&F satellite operation information e.g., whether UE is registered in S&F Mode, the estimated S&F DL delivery time, feeder link availability period and change in S&F status for the UE. The SEALDD server stores the received UE S&F status events information to further control the downlink application data transmission.

NOTE 1:	The SEALDD server receives the UE ID in the Sdd_RegularTransmissionConnection_Establish request from the SEALDD client as defined in the clause  9.2.2.2, which is part of step 2. This UE ID is used by the SEALDD server while subscribing for S&F satellite operation information from the 5GC in step 5.

6.	The SEALDD server controls the downlink application data delivery to the SEALDD client based on the UE S&F status events information, when the recipient UE is in S&F mode and receives the estimated S&F DL delivery time is received from 3GPP CN. If the recipient UE is not available or not registered to S&F mode, then the SEALDD server handles the DL data as per the service policy.

7.	The SEALDD server sends the Sdd_S&FTransmission_notification to the SEALDD client and VAL client including the estimated S&F DL delivery time of the recipient UE.

NOTE 2:	VAL client can use the estimated S&F DL delivery time of the recipient UE to control the UL application data delivery.

8.	The SEALDD server sends the DL data to the VAL server.

NOTE 3:	Details of step 1, step 3 and step 7 are out of scope of the current specification.

* * * End of Change * * * *