3GPP TSG-SA WG6 Meeting #69	S6-254344

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

Source:	TNO KPN N.V

Title:	Resolution of Editor’s Note and Procedure Alignment for Multi-Client Split Operation Pipeline

Spec:	3GPP TR 23.700-83 1.0.0

Agenda item:	9.4

Document for:	Approval

Contact:	Anthony Pages (anthony.pages@tno.nl)

1. Introduction

This contribution resolves the remaining Editor’s Notes in clause 7.8.1.1 related to the feasibility of sharing a split AI/ML operation pipeline among multiple AIMLE clients. It also aligns the described procedures for pipeline creation and subscription with the mechanisms defined in 3GPP TS 23.482 by removing references to the previously proposed merged procedure (“split operation pipeline create subscription”).

2. Reason for Change

Editor’s Notes indicated that the feasibility of sharing a split operation pipeline among AIMLE clients was FFS. This contribution addresses and resolves this note by providing an evaluation of feasibility in clause 7.8.4.

Additionally, to ensure consistency with TS 23.482, the description of the solution in clause 7.8.1.2 has been updated to reflect the use of two distinct procedures instead of the merged create-and-subscribe procedure that was not pursued in the last SA6 meeting.

3. Conclusions

This contribution resolves the Editor’s Note concerning multi-client pipeline sharing and ensures alignment with existing split operation procedures defined in TS 23.482. No new procedures are introduced; instead, the existing standardized procedures are referenced and clarified for the multi-client pipeline case.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 V1.0.0.

* * * First Change * * * *

## 7.8	Solution #8: Multi-Client Split AI/ML Operations Pipeline

### 7.8.1	Solution Description

#### 7.8.1.1	General

This solution addresses key issues related to split AI/ML operations involving multiple AIMLE clients. The proposed solution enhances split AI/ML operations by facilitating multiple AIMLE client collaboration within the same pipeline. It allows the creation of multi-client AI/ML split operation pipeline when such a pipeline is not yet created and allows joining the pipeline when such a pipeline has already been created. The solution also keeps AIMLE clients in sync with the latest status of the pipeline through notifications as more AIMLE clients join and the pipeline is adjusted according to the resources of the newly joining AIMLE clients. The solution ensures efficient resource utilization and continuous service, even with mobility and varying device constraints.

Use Cases:

-	Enhanced Smart City Surveillance with Mobility: Surveillance cameras, both stationary and mobile, capture video footage and process raw data on the cameras, sending subsequent layers to an edge server for immediate processing. AIML split operations enable real-time improved surveillance and threat detection for public safety through a shared pipeline that allows collaborative data processing. The pipeline needs to be re-adjusted based on computing resource powers for the surveillance cameras. Pipeline re-adjustment is also needed as the mobile surveillance cameras change service areas. This results in efficient resource utilization, energy saving, and larger surveillance.

-	Collaborative Autonomous Driving: A group of autonomous vehicles collaborate by sharing and processing data together. A more concreate example is vehicles utilizing shared Local Dynamic Maps (LDM) which is collaboratively updated and used by each vehicle locally. The raw data of each vehicle is processed locally and it is sent for subsequent layers in the edge server for immediate processing. The shared AIML split operation enable real-time collaboration to optimize driving conditions (e.g., efficient routes) and reduces computational burden in the vehicles. Pipeline re-adjustment is needed based on the computing resource power of the participants. Pipeline-readjustment is also needed as the vehicles change service area during mobility. This results in a larger collaboration coverage.

#### 7.8.1.2	Multi-client pipeline operation

Figure 7.8.1.2-1: Multi-client AI/ML split operation

The solution involves a multi-client split operations pipeline facilitated by AIMLE clients and servers. Initially, an AIMLE client initiates a discovery request to the AIMLE server to participate in a split operation pipeline, as illustrated in Block A of the figure. This is similar to clause 8.14.2.2 in 3GPP TS 23.482 split operation pipeline discovery. A key addition for this solution is an expansion of the split operation discovery filters in clause 8.14.3.2-2 with additional criteria for multi-client split operations within the pipeline. Following the discovery request, the split operation profile in the discovery response, as in clause 8.14.3.3 Table 8.14.3.3-2, is also expanded to include information about a pipeline with multi-client split operation pipeline support, the usage of the multi-client split operation pipeline (in usage information), and whether such a pipeline has already been created or not.

If in the discovery stage, as in Block B, the discovered instance of the split operation pipeline is a pipeline that has not yet been created, the AIMLE client will then request the creation of a new pipeline, explicitly mentioning the option for multi-client operations in the split operation profile, as shown in Block B, and subsequently perform a subscription to the split operation event. This is similar to clause 8.14.2.3 in 3GPP TS 23.482 and clause 8.14.2.5 in 3GPP TS 23.482, with additional expansion of clause 8.14.3.4-1 in 3GPP TS 23.482 to include multi-client support in the pipeline, expansion of the usage information to include further details on the multi-client pipeline, and also the computing resources of the AIMLE client to be able to efficiently adjust the pipeline later on as more AIMLE clients join the pipeline.

Subsequently, once a multi-client AI/ML split operation pipeline has been created, other AIMLE clients who seek to also engage in the multi-client AI/ML split operation pipeline begin by initiating a discovery procedure as in Block C, which is similar to Block A. If a pipeline is already created according to Block B, it identifies the existing created pipeline that supports multi-client AI/ML split operation, fulfilling the respective split operation discovery filters. The AIMLE client then proceeds with the procedures described in the Block D, the split operation pipeline join and the split operation pipeline subscription as defined in 3GPP TS 23.482. To enable adjustment of the pipeline considering the newly joining AIMLE clients, each client indicates its respective computing resources during the join. This process can be scaled up to accommodate an n-number of AIMLE clients, allowing for extensive multi-client collaboration in split AI/ML operations.

As AIMLE clients with different computing resources join the split operation pipeline, the pipeline may need to be adjusted, and the participating AIMLE clients need to be notified of changes to the pipeline so that they can make client-side adjustments. For this purpose, as shown in Block E, a split AI/ML operation notification is sent to all AIMLE clients as per clause 8.14.2.5.3 of 3GPP TS 23.482.

* * * Next Change * * * *

### 7.8.4	Solution Evaluation

The feasibility of sharing a split AI/ML operation pipeline among multiple AIMLE clients has been assessed in the context of the proposed solution. It is considered feasible, provided that the AIMLE server maintains a global view of the pipeline and coordinates participation of multiple clients. Each client reports its computing resources when joining the pipeline, allowing the server to adjust task distribution dynamically. Pipeline sharing relies on the existing procedures defined in 3GPP TS 23.482. During discovery, clients identify existing pipelines that support multi-client operations. Clients then perform the join procedure and subscribe to pipeline events. The server orchestrates the pipeline adjustments as new clients join or leave, and notifications are sent to all participating clients to allow appropriate client-side adaptation.

This approach ensures that the pipeline can accommodate a variable number of AIMLE clients while maintaining efficient resource utilization and consistent operation. It is assumed that participating clients follow the protocol, and that network and computing resources are within acceptable limits for split operation execution.

* * * End of Change * * * *