# 3GPP TSG-SA WG6 Meeting #69	S6-254508

# Wuhan, China, 13th – 17th Oct 2025                                                              (revision of S6-254172)

Source:	ZTE Corporation

Title:	Solution for KI# 4 on support for AEF operational status

Spec:	3GPP TR 23.700-43 v0.1.0

Agenda item:	9.11

Document for:	Approval

Contact:	Wei Luo <luo.wei11@zte.com.cn>

1. Introduction

This paper proposes a solution to KI# 4 on support for AEF operational status.

2. Reason for Change

This paper proposes a solution to KI# 4 on support for AEF operational status. In particular, this solution is proposed to introduce a new CAPIF event to handle the scenarios in which the connection between the CAPIF core function and the API provider domain functions is disrupted due to network issues.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 v0.1.0.

* * * First Change * * * *

# 6.x	Solution #x: Enhanced CAPIF events for AEF status

6.x.1	General

This solution is related to key issue#4,  specifically on introducing new CAPIF events to handle the scenarios in which the connection between the CAPIF core function and the API provider domain functions is disrupted due to network issues.

6.x.2	Architecture Impacts

It is based on existing CAPIF architecture specified in 3GPP TS 23.222 [2].

6.x.3	Solution description

In the existing CAPIF framework, introducing a heartbeat mechanism similar to the EAS Availability Reporting Period would enable CCF(CAPIF core function) to detect whether the connection between the CAPIF core function and an AEF is disrupted or not.

Editor's note:	Whether there may be other mechanism than heartbeat and how the heartbeat mechanism can be enabled to detect AEF availability is FFS.

When such connectivity problems arise, the AEF associated APIs  may become unavailable. The consequence is that an ongoing service API invocation may terminate abruptly or time out, leading to transactions incomplete. Without timely notifications, API invoker may repeatedly attempt reconnections which may lead to unnecessary resource waste and heighten network congestion.

The following scenario is considered for this solution: when the CAPIF core function and a API provider domain functions are not co-located,  CCF detects the AEF is disconnected due to network issue and there are multiple service APIs are accessible through this AEF by API invoker(s). Though AEF availability could be reported through service API availability and service API update events, in order to reduce the number of Service API updated event notification, the CCF may consider following options:

Option 1:  CCF sends

a new CAPIF event (i.e.,AEF status in CAPIF)  without sending Service API updated event notification if the CCF consider Service API  information is changed due to AEF status changing.

Option 2: CCF sends AEF status in existing CAPIF event.

Editor’s Note：Whether the AEF identity information should be provided to the API invoker or should be hidden is FFS.

Editor’s Note：Which option is selected is FFS.

AEF status can be defined as the follows:

- Unreachable: Unable to communicate with the CCF (e.g., due to network issues), though the AEF itself may remain functional.

- Reachable: The connection between the AEF and CCF is restored to normal after network issues.

Regarding this new CAPIF event, as an example, once the CCF detects the connection between the CAPIF core function and an AEF is disrupted,  it generates event of AEF status and sends the event notification including AEF Identity information and the AEF status (i.e.,unreachable) to the subscribing entity (e.g.,the API invoker), then the API invoker may stop invoking the AEF associated APIs. After that, once the connection between the AEF and CCF is restored to normal, the CCF generates event of AEF status and sends the event notification including AEF Identity information and the AEF status (i.e.,reachable) to the subscribing entity (e.g.,the API invoker), then the API invoker may resume invoking the AEF associated APIs.

6.x.3.1	Information flows

The following updates (in bold) to clause 8.8.6 of 3GPP TS 23.222 [2] are proposed for supporting new events for AEF status change if option 1 is adopted.

8.8.6 List of CAPIF events Table 8.8.6-1 provides a non-exhaustive list of CAPIF events. Table 8.8.6-1: List of CAPIF events Events; Events Description. Availability of service APIs; Availability events of service APIs (e.g. active, inactive). Service API updated; Events related to change in service API information. Monitoring service API invocations; Events corresponding to service API invocations. API invoker status; Events related to API invoker status in CAPIF (onboarded, offboarded, onboarding criteria not met status along with the criteria information that is not met.). API topology hiding status; Events related to API topology hiding status in CAPIF (created, revoked). System related events; Alarm events providing fault information. Performance related events; Events related to system load conditions. CAPIF-1/1e interaction events; Events related to CAPIF-1/1e interactions for a service API (e.g. onboarded API invokers). AEF status; Events related to AEF status (e.g. unreachable, reachable) in CAPIF.

6.x.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *