3GPP TSG-SA WG6 Meeting #70	S6-255051

Dallas, USA 17th – 21st November 2025	(revision of S6-254xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0260 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location information support beyond group call procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To extend the optional Location information handling to additional MCVideo request procedures beyond the current group call, enabling consistent behaviour across all MC Services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an optional Location information element to MCVideo request procedure beyond group call. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent behaviour of MCVideo compared with other MC Services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.2.18, 7.1.2.2.22, 7.2.2.2.1, 7.2.2.2.2, 7.2.2.2.6, 7.2.2.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

##### 7.1.2.2.18	MCVideo emergency group call request

Table 7.1.2.2.18-1 describes the information flow emergency group call request from the MCVideo client to the MCVideo server, from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.18-1: MCVideo emergency group call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo group ID | M | The MCVideo group ID on which the call is to be conducted |
| Emergency indicator | M | Indicates that the group call request is an MCVideo emergency call |
| Alert indicator | M | Indicates whether an emergency alert is to be sent |
| Requested priority | O | Priority level requested for the call |
| Implicit transmit media request (see NOTE) | O | Indicates that the originating client requests the permission to transmit media. |
| Location information | O | Location of the calling party |
| NOTE: This element shall be included only when the originating client requests the the permission to transmit media. |  |  |

* * * Second Change * * *

##### 7.1.2.2.22	MCVideo imminent peril group call request

Table 7.1.2.2.22-1 describes the information flow MCVideo imminent peril group call request from the MCVideo client to the MCVideo server, from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.22-1: MCVideo imminent peril group call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo group ID | M | The MCVideo group ID on which the call is to be conducted |
| Requested priority | O | Priority level requested for the call |
| Imminent peril indicator | M | Indicates that the group call request is an imminent peril call |
| Location information | O | Location of the calling party |

* * * Third Change * * *

##### 7.2.2.2.1	MCVideo private call request (MCVideo client – MCVideo server)

Table 7.2.2.2.1-1 describes the information flow MCVideo private call request from the MCVideo client to the MCVideo server.

Table 7.2.2.2.1-1: MCVideo private call request (MCVideo client – MCVideo server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ID (see NOTE) | O | The MCVideo ID of the called party |
| Functional alias (see NOTE) | O | The functional alias of the called party |
| Transmit media request indication | M | This element indicates whether transmission control will be used for the private call. |
| SDP offer | M | Media parameters of MCVideo client. |
| Requested commencement mode | O | An indication that is included if the user is requesting a particular commencement mode |
| Implicit transmit media request | O | An indication that the user is also requesting the permission to transmit video |
| Location information | O | Location of the calling party |
| Push indication | O | Indicates that the private call request is for a one-to-one push call |
| Pull indication | O | Indicates that the private call request is for a one-to-one pull call |
| Requested priority | O | Application priority level requested for this call |
| NOTE: Either an MCVideo ID or a functional alias shall be present, but not both. |  |  |

##### 7.2.2.2.2	MCVideo private call request

Table 7.2.2.2.2-1 describes the information flow MCVideo private call request from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.2.2.2.2-1: MCVideo private call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ID | M | The MCVideo ID of the called party |
| Transmit media request indication | M | This element indicates whether transmission control will be used for the private call. |
| SDP offer | M | Media parameters of MCVideo client. |
| Requested commencement mode | O | An indication that is included if the user is requesting a particular commencement mode |
| Implicit transmit media request (see NOTE) | O | An indication that the user is also requesting the permission to transmit video |
| Location information | O | Location of the calling party |
| Push indication | O | Indicates that the private call request is for a one-to-one push call |
| Pull indication | O | Indicates that the private call request is for a one-to-one pull call |
| NOTE: This element shall not be included when the request is sent from the MCVideo server to the MCVideo client. |  |  |

* * * Fourth Change * * *

##### 7.2.2.2.6	MCVideo emergency private call request (MCVideo client to MCVideo server)

Table 7.2.2.2.6-1 describes the information flow MCVideo emergency private call request from the MCVideo client to the MCVideo server.

Table 7.2.2.2.6-1: MCVideo emergency private call request (MCVideo client to MCVideo server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ID | M | The MCVideo ID of the called party |
| Emergency indicator | M | Indicates that the private call request is an MCVideo emergency call |
| Alert indicator | M | Indicates whether an emergency alert is to be sent |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit transmit media request | O | An indication that the user is also requesting to transmit media |
| Location information | O | Location of the calling party |
| SDP offer | M | Media parameters of MCVideo client. |
| Requested priority | O | Priority level requested for the call |

##### 7.2.2.2.7	MCVideo emergency private call request

Table 7.2.2.2.7-1 describes the information flow MCVideo emergency private call request from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.2.2.2.7-1: MCVideo emergency private call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ID | M | The MCVideo ID of the called party |
| Emergency indicator | M | Indicates that the private call request is an MCVideo emergency call |
| Alert indicator | M | Indicates whether an emergency alert is to be sent |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit transmit media request | O | An indication that the user is also requesting to transmit media. |
| Location information | O | Location of the calling party |
| SDP offer | M | Media parameters of MCVideo client. |

* * * End of changes * * *