3GPP TSG-SA WG6 Meeting #70	S6-255077

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0398 | rev | - | Current version: | 19.8.0 |  |
| For HELP on usig this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Server-Server message (MCData) procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC_Ph2-MC |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The procedures in clause 7.17.3.2 Ad hoc group data communication involving multiple MC systems describing already the information flows from server to server but not clearly. Describing the actual step more in detail in the figures and its belonging descriptions clarify the actual situation. Further Notes in clause 7.17.3.2 become obsolete and can be deleted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying MCData server to server steps and figures in AHGC procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong figures may cause misunderstandings to the downstream groups and the readers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.17.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The changes in the corresponding information flows are discussed in CR0397 (S6-255076). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

#### 7.17.3.2	Ad hoc group data communication involving multiple MC systems

##### 7.17.3.2.1	Procedure for ad hoc group data communication setup – Participants list provided by the Initiator

Figure 7.17.3.2.1-1 illustrates the procedure for ad hoc group data communication setup procedure initiated by an authorized user wherein either a list of participants or an ad hoc group ID from an ad hoc group emergency alert is provided by the authorised user and the determined MCData users are from multiple MCData systems.

Pre-conditions:

1.	The security aspects of sharing the user information between primary and partner MC systems shall be governed as per the service provider agreement between them. In this case, it is considered that the partner MC system shares their users' information to the primary MC system.

2.	The authorized MCData user/dispatcher belongs to the primary MC system.

3.	The MCData server of the primary MC system is where the authorized MCData user/dispatcher creates the ad hoc group.

4.	Some users of the ad hoc group belong to partner MC systems.

5.	The preconfigured group identity and preconfigured group configuration (e.g. security related information) to be used for an ad hoc group have been preconfigured in the MCData client and other participants of ad hoc group have also received the relevant security related information to allow them to communicate in an ad hoc group communication.

6.	If an ad hoc group ID from an ad hoc group emergency alert is used to invite the ad hoc group members, MCData client is aware of the ad hoc group ID.

Figure 7.17.3.2.1-1: Ad hoc group data communication setup

1.	The MCData client of the authorized user initiates an ad hoc group data communication with multiple users from primary and partner MC systems. An ad hoc group data session request message with the information of the participants MCData IDs or an ad hoc group ID if the ad hoc group data session follows an ad hoc group emergency alert is routed to the MCData server of the primary MC system.

2.	If the ad hoc group data session is supported, the MCData server of the primary MC system verifies whether the user at MCData client is authorized to initiate an ad hoc group communication. If not authorized, the MCData server of the primary MC system rejects the ad hoc group data session request as specified in the step 3.

If the information received in the request in step 1 does not contain an ad hoc group ID from an ad hoc group emergency alert, the MCData server of the primary MC system forms the ad hoc group by using information received in step 1 and further determines the preconfigured group to be used for the configuration (e.g. security related information). The MCData server assigns an MCData ad hoc group ID for the newly formed ad hoc group.

If no MCData ad hoc group ID was included in the ad hoc group data session request of step 1, or if the provided MCData ad hoc group ID is not accepted by the MCData server, the MCData server assigns a MCData group ID for the newly formed ad hoc group. The MCData server considers the ad hoc group communication participants as implicitly affiliated to the ad hoc group.

NOTE 1:	The newly formed ad hoc group information including the MCData group ID and the list of users is held in dynamic data in the MCData server.

3.	The MCData server shall send the ad hoc group data session request return message to MCData client containing the below:

i.	The MCData ad hoc group ID either generated by the MCData server or provided by the MCData client if the ad hoc group ID is from an ad hoc group emergency alert (only included when the ad hoc group data session request is authorized);

ii.	The group ID of the pre-configured group to be used for the ad hoc group data communication (only included when the ad hoc group data session request is authorized); and

iii.	Result of whether the ad hoc group data session request is authorized or not.

If the ad hoc group data session request is not authorized, MCData client 1 shall not proceed with the rest of the steps.

4.	The MCData server of the primary MC system sends the ad hoc group data session request to the group members of the ad hoc group belonging to the primary MC system. While sending the ad hoc group data session requests, the MCData server shall remove the information elements that are not required to be conveyed to the target MCData clients (e.g. MCData ID list of the users who are required to acknowledge).

5a-5b.	The MCData clients receive in the ad hoc group data session request with the information of the MCData group ID for the ad hoc group and further notify their corresponding MCData user. The group members of the ad hoc group of the primary MC system may accept or reject the ad hoc group data session requests and respond with the ad hoc group data session responses.

6.	The primary MCData server further initiates an ad hoc group data session request message to the MCData users of the partner MC system. The ad hoc group data session request message is routed to the MCData users via the MCData server of the partner MC system.

7a-7b.	The MCData clients receive in the ad hoc group data session request the information of the MCData group ID for the ad hoc group and further notify their corresponding MCData user. The MCData users upon receipt of the invitation may accept or reject the ad hoc group data session requests, and respond with the ad hoc group data session responses. The ad hoc group data session response message is routed to the MCData server of the primary MC system via the MCData server of the partner MC system.

8.	The MCData server of the primary MC system provides an ad hoc group data session response message to the MCData client of the authorized MCData user upon receiving response to the corresponding ad hoc group data session request in step 1. The ad hoc group data session response will consist of the success or failure result and/or detailed reason information in case of failure.

NOTE 2:	The ad hoc group data session response messages may be used by The MCData client of the authorized user to determine if the ad hoc group communication will proceed.

9.	Upon successful ad hoc group data session setup, a group data communication is established amongst the group members from primary and partner MC systems. The media plane resources for data communication are established.

* * * Second Change * * *

##### 7.17.3.2.2	Procedure for ad hoc group data communication release by MCData server – Participants list provided by the Initiator

This procedure focuses on the case where an MCData server initiates the termination of an ongoing MCData ad hoc group data communication for all the participants of that group data communication, since at least one of the termination conditions are met e.g., due to hang time expiry, last participant leaving, second last participant leaving, initiator leaving, or minimum number of affiliated MCData group members are not present.

Procedures in figure 7.17.3.2.2-1 are the signalling control plane procedures for the MCData server initiating termination of an ongoing MCData ad hoc group data communication.

Pre-condition:

-	The MCData client 1 and MCData client 2 belong to primary MC system. The MCData client 3 belongs to partner MC system.

-	The MCData users on MCData client 1, client 2 and client 3 are already part of the ongoing ad hoc group data communication (e.g. as a result of ad hoc group data communication setup as specified in clause 7.17.3.2.1).

Figure 7.17.3.2.2-1: Ad hoc group data communication release

1.	The MCData server determines to release the ongoing ad hoc group data communication e.g., due to a hang timer expiry, last participant leaving, second last participant leaving, initiator leaving, or a minimum number of affiliated MCData group members not present.

2.	The MCData server identifies the participants of the ongoing ad hoc group data communication and generates ad hoc group data session release request to release ongoing data communication.

3.	The MCData server sends an ad hoc group data session release request to each participant of the ongoing group data communication. If the participants belong to the partner MC system, then the ad hoc group data session release request is routed to the MCData clients of the partner MC system via the partner MCData server.

4.	The MCData users are notified about the release of the ad hoc group data communication.

5.	The MCData client(s), send an acknowledgment to the MCData server by sending an ad hoc group data session release response. The ad hoc group data session release response message is routed to the MCData server of the primary MC system via the MCData server of the partner MC system.

6.	The MCData client 1, client 2 and client 3 have successfully released media plane resources associated with the ad hoc group data communication that is terminated. The primary MCData server removes the ad hoc group information from the dynamic data and thus the ad hoc group ceases to exist.

* * * Third Change * * *

##### 7.17.3.2.3	Ad hoc group data communication setup – Participants list determined by the MCData server

Figure 7.17.3.2.3-1 below illustrates the ad hoc group data communication setup procedure initiated by an authorized user wherein the list of participants is determined by the MCData server based on the criteria received from the MCData client and determined MCData users are from multiple MCData systems.

Pre-conditions:

1.	The security aspects of sharing the user information between primary and partner MC systems shall be governed as per the service provider agreement between them. In this case, it is considered that the partner MC system share their users' information to the primary MC system.

2.	The authorized MCData user/dispatcher belongs to the primary MC system.

3.	The MCData server 1 of the primary MC system is where the authorized MCData user/dispatcher creates the ad hoc group.

4.	Some users of the ad hoc group may belong to MCData server 2 of the partner MC systems.

5.	The pre-configured group identity and pre-configured group configuration to be used for an ad hoc group have been preconfigured in MCData client and other participants of ad hoc group have also received the relevant security related information to allow them to communicate in an ad hoc group communication.

Figure 7.17.3.2.3-1: Ad hoc group data communication setup involving multiple MCData systems

1-3.	Same as described in subclause 7.17.3.1.1.

4.	The MCData server 1 determines the list of participants from the primary MC system and determines the partner MC system to be involved in the ad hoc group data communication based on the information present in the information element Criteria for determining the participants. This information element carries the criteria, indicator identifying pre-defined criteria, or a combination of both. Depending on the criteria provided and based on local policy, the MCData server may modify the content of the criteria received in step 1 to determine the list of participants.

NOTE 1:	The content of the Criteria information element, the details of the pre-defined criteria, and the way how the MCData server determines the list of participants are left to implementation.

5.	The MCData server 1 needs to involve the partner MC system based on the agreement and based on the criteria for determining the participants list, it sends the ad hoc group data session get userlist request to the MCData server 2. This request carries the data session resulting criteria to be used by the partner MC system as determined in step 4.

6.	MCData server 2 evaluates the criteria and determines the participants satisfying the criteria (i.e. MCData client 3 and MCData client 4) and sends the response containing the list of MCData users satisfying the criteria. The MCData server 2 may apply local policies if any while determining the participants satisfying the criteria.

7.	The MCData server 1 compiles the list of participants to be invited for the ad hoc group data communication including the participants from both primary and partner MC system.

8a-8b.	The MCData server 1 sends the ad hoc group data session request towards the MCData client 3 and MCData client 4 via MCData server 2. While sending the ad hoc group data session request, the MCData server shall remove the information elements that are not required to be conveyed to the target MCData clients. This request carries the pre-configured group ID whose configuration is to be applied for this ad hoc group data communication if end-to-end encryption is requested. The MCData server 1 considers the ad hoc group communication participants as implicitly affiliated to the ad hoc group.

9.	The MCData server 1 sends the ad hoc group data session request towards the MCData client 2. While sending the ad hoc group data session request, the MCData server shall remove the information elements that are not required to be conveyed to the target MCData clients. This request carries the pre-configured group ID whose configuration is to be applied for this ad hoc group data communication if end-to-end encryption is requested.

10a-10c.	The MCData clients receive incoming ad hoc group data communication and further notify their corresponding MCData users.

11.	The MCData client 2 accepts the ad hoc group data session request, and sends ad hoc group data session response to the MCData server 1.

12.	The MCData client 3 accepts the ad hoc group data session request, and sends ad hoc group data session response to the MCData server 1 via MCData server 2.

13.	The MCData client 4 accepts the ad hoc group data session request, and sends ad hoc group data session response to the MCData server 1 via MCData server 2.

14.	The MCData server 1 sends the ad hoc group data session response to MCData client 1 through the signalling path to inform about result of the participants responses. The ad hoc group data session response contains the data session resulting criteria used by the MCData server to determine the list of participants to be invited.

15.	The MCData server 1 may notify the initiating MCData user of all MCData users who acknowledged the ad hoc group data session request and joined the ad hoc group data communication. The MCData server 1 more than once during the data communication may send this notification to the initiating MCData user whenever an MCData user joins or leaves the MCData ad hoc group data communication. If the authorized users (not shown in figure) are configured to receive the participants information of ad hoc group data communication, the MCData server provides ad hoc group data session notify about all MCData users who acknowledged the ad hoc group data session request and joined the ad hoc group data communication, and when MCData users joins or leaves the MCData ad hoc group data communication. All ad hoc group data session notify messages contain the data session resulting criteria used by the MCData server to determine the list of participants to be invited.

16.	The MCData client 1, MCData client 2, MCData client 3 and MCData client 4 establish media plane resources for data communication.

The MCData server at the primary MC system and MCData server at the partner MC system continuously check whether other MCData users in the primary MC system and MCData users in the partner MC system, respectively, meet the criteria in order to add them to the ongoing ad hoc group communication, or whether any of the participating MCData users in the primary MC system and MCData users in the partner MC system, respectively, no longer meet the criteria in order to remove them from the ongoing ad hoc group communication, according to clause 7.17.3.2.4.

* * * Forth Change * * *

##### 7.17.3.2.4	Modifying of ad hoc group data communication participants by the MCData server

Figure 7.17.3.2.4-1 below illustrates the MCData server modifying the ad hoc group data communication participants procedure involving multiple MCData systems.

Pre-conditions:

1.	The MCData user at MCData client 1 is authorized to initiate ad hoc group data communication.

2.	The MCData server 1 of the primary and MCData server 2 of the partner MCData systems determined the participants for the ad hoc group data communication based on the criteria specified by the MCData client 1 while initiating the ad hoc group data communication.

3.	The ad hoc group data communication is established and on-going with the participants MCData client 1, MCData client 2, and MCData client 4. The participants list is determined by both primary and partner MC systems.

4.	The MCData server 1 of the primary and MCData server 2 of the partner MCData systems continuously evaluates the criteria to monitor the list of users who meets or not meets the criteria for participating in the on-going ad hoc group data communication.

5.	The MCData server 1 of the primary MCData system detects that the MCData client 3 meets the criteria and MCData client 2 stops to meet the criteria specified by the MCData client 1.

6.	The MCData server 2 of the partner MCData system detects that the MCData client 5 meets the criteria and MCData client 4 stops to meet the criteria specified by the MCData client 1.

Figure 7.17.3.2.4-1: Modification of ad hoc group data communication participants by the MCData server

1.	The MCData server 1 detects that MCData client 3 now meets criteria and successfully added to group data communication.

2.	The MCData server 1 detects that MCData client 2 does not meet criteria any more and successfully removed from a group data communication.

3.	The MCData server 2 detects that the MCData client 5 meets the criteria specified by the MCData client 1.

4.	The MCData server 2 sends the notification to MCData server 1 of the primary MCData system to add the MCData user at MCData client 5 to on-going ad hoc group data communication.

5.	The MCData server 1 sends the ad hoc group data session request to the MCData client 5 via MCData server 2.

6.	The MCData client 5 notifies the user about the incoming ad hoc group data communication.

7.	The MCData client 5 accepts the ad hoc group data session request and sends the ad hoc group data session response to the MCData server 1 via MCData server 2.

8.	The on-going ad hoc group data communication is updated by adding MCData client 5 which meets the criteria specified by the MCData client 1.

9.	The MCData server 2 detects that the MCData client 4 is no longer satisfying the criteria to be the participant of the ad hoc group data communication.

10.	The MCData server 2 sends the notification to MCData server 1 of the primary MCData system to remove the MCData user at MCData client 4 from on-going ad hoc group data communication.

11.	The MCData server 1 sends the ad hoc group data session leave request to the MCData client 4 and removes it from the on-going ad hoc group data communication via MCData server 2.

12.	The MCData client 4 notifies the user of the ad hoc group data session leave request.

13.	The MCData client 4 sends the ad hoc group data session leave response to the MCData server 1 via MCData server 2.

14.	The on-going ad hoc group data communication is updated by removing MCData client 4, which no longer meets the criteria specified by the MCData client 1.

* * * Fifth Change * * *

##### 7.17.3.2.5	Release ad hoc group data communication and stop determining the ad hoc group data communication participants by partner MCData system – Participants list determined by the MCData server

Figure 7.17.3.2.5-1 below illustrates the release of ad hoc group data communication and stopping of MCData server at the partner MC system from determining the ad hoc group data communication participants’ procedure once involving multiple MCData systems. This procedure, in particular describes about how the partner MC system is notified about ad hoc group data communication release to cease the determining of the participants by the partner MC system.

Pre-conditions:

1.	The MCData user at MCData client 1 is authorized to initiate ad hoc group data communication.

2.	The ad hoc group data communication is established and on-going with the participants MCData client 1, MCData client 2, MCData client 3, and MCData client 4.

3.	The MCData server 1 of the primary and MCData server 2 of the partner MCData systems determined the participants for the ad hoc group data communication based on the criteria specified by the MCData client 1 while initiating the ad hoc group data communication.

4.	The MCData server 1 of the primary and MCData server 2 of the partner MCData systems continuously evaluates the criteria to monitor the list of users who meets or not meets the criteria for participating in the on-going ad hoc group data communication.

Figure 7.17.3.2.5-1: MCData server releases an ad hoc group data communication and stops the determination of ad hoc group data communication participants involving multiple MCData systems

1.	The MCData server 1 detects that the ad hoc group data communication which is ongoing is to be released e.g., due to hang time expiry, last participant leaving, second last participant leaving, initiator leaving, or minimum number of affiliated MCData group members are not present.

2.	The MCData server 1 sends the ad hoc group data session release notification to MCData server 2 of the partner MCData system about the ad hoc group data communication release to stop determining the participants list by MCData server 2 of the partner MCData system.

3a-3d.	The MCData server 1 sends ad hoc group data session release request to all the participants of the ad hoc group data communication. MCData client 3/MCData client 4 which belong to the partner MCData system, receive the ad hoc group data session release request via the partner MCData server 2 from MCData server 1 of the primary MCData system.

4a-4d.	The MCData clients notifies the user about the release of the ad hoc group data communication.

5a-5d.	All the participants of the ad hoc group data communication receive the ad hoc group data session release request and send the ad hoc group data session release response to the MCData server 1 of the primary MCData server. MCData client 3/MCData client 4 which belong to the partner MCData system, send the ad hoc group data session release response via the partner MCData server 2 to the MCData server 1 of the primary MCData system.

* * * Sixth Change * * *

##### 7.17.3.2.6	Modification of ad hoc group data communication participants by an authorized user

Figure 7.17.3.2.6-1 below illustrates the MCData user modifying the ad hoc group data communication participants procedure involving multiple MCData systems.

Pre-conditions:

1.	An MCData ad hoc group data communication is already in progress the participants list provided by the originating MCData user while initiating the MCData ad hoc group data communication.

2.	The participants of the MCData ad hoc group data communication belongs to the multiple MCData system.

3.	The MCData user at MCData client 1 determines that the MCData client 3 needs to be invited to on-going ad hoc group data communication and MCData client 2 needs to be removed from the on-going ad hoc group data communication from the primary MCData system.

4.	The MCData user at MCData client 1 determines that the MCData client 5 needs to be invited to on-going ad hoc group data communication and MCData client 4 needs to be removed from the on-going ad hoc group data communication from the partner MCData system.

Figure 7.17.3.2.6-1: Modification of ad hoc group data communication participants by an authorized user

1.	The MCData client 1 sends the modify ad hoc group data session participants request to the MCData server 1 to remove both MCData client 2 and MCData client 4 from the on-going ad hoc group data communication and to add both MCData client 3 and MCData client 5 into on-going ad hoc group data communication.

2.	The MCData server 1 verifies whether the MCData client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group data communication.

3.	The MCData server 1 sends modify ad hoc group data session participants response to the MCData client 1.

4.	The MCData server 1 determines that MCData client 1 has requested that MCData client 3 from the primary MC system be invited to an on-going ad hoc group data communication and successfully added to the data communication according to the procedure defined in clause 7.17.3.1.4.

5.	The MCData server 1 determines that MCData client 1 has requested that MCData client 2 from the primary MC system be removed from the on-going ad hoc group data communication and successfully removed from data communication according to the procedure defined in clause 7.17.3.1.4.

6.	 The MCData server 1 determines that MCData client 1 has requested that MCData client 5 from the partner MC system be invited to an on-going ad hoc group data communication. The MCData server 1 sends the ad hoc group data session request to the MCData client 5 via MCData server 2.

7.	The MCData client 5 notifies the user about the incoming ad hoc group data communication.

8.	The MCData client 5 accepts the ad hoc group data session request and sends the ad hoc group data session response to the MCData server 1 via MCData server 2.

9.	The on-going ad hoc group data communication is updated by adding MCData client 5 which is based on the modify participant list provided by the MCData client 1.

10.	The MCData server 1 determines that MCData client 1 has requested that MCData client 4 from the partner MC system be removed from the on-going ad hoc group data communication. The MCData server 1 sends the ad hoc group data session leave request to the MCData client 4 via MCData server 2.

11.	The MCData client 4 notifies the user of the ad hoc group data session leave request.

12.	The MCData client 4 sends the ad hoc group data session leave response to the MCData server 1 via MCData server 2.

13.	The on-going ad hoc group data communication is updated by removing MCData client 4, which is based on the modify participant list provided by the MCData client 1.

14.	The MCData server 1 may notify the initiating MCData user of all the users who are added to the on-going ad hoc group data communication. This notification may be sent to the initiating MCData user by the MCData server more than once during the data communication when MCData users join or leave the ad hoc group data communication.

15.	The MCData server 1 may notify the authorized participants about the change in the participants list of on-going ad hoc group data communication.

* * * Seventh Change * * *

##### 7.17.3.2.7	Modification of ad hoc group data session criteria by an authorized user

Figure 7.17.3.2.7-1 illustrates the modification of ad hoc group data session criteria procedure by the initiator of the ad hoc group data session between multiple MC systems.

Preconditions:

-	The MCData user at the MCData client 1 is authorized to modify the criteria.

-	The MCData server A and MCData server B are aware of the criteria related to the ongoing ad hoc group data session.

-	MC system A and MC system B are interconnected.

Figure 7.17.3.2.7-1: Modifying the criteria for determining the participants during an ongoing ad hoc group data session between multiple MC systems

1.	An ad hoc group data session has been established based on criteria sent by an authorized MCData client upon initiating the ad hoc group data session.

2.	The MCData user at the MCData client 1 is authorized and wishes to modify the criteria for determining the list of ad hoc group data session participants.

3a.	MCData client 1 sends a modify ad hoc group data session criteria request to MCData server A.

3b.	MCData server A determines that the modify ad hoc group data session criteria request received in step 3a has impact on users in MCData server B. Depending on the criteria provided and based on local policy, the MCData server may modify the content of the criteria received in step 3a to determine the list of participants before sending it to MCData server B.

3c.	MCData server A sends a modify ad hoc group data session criteria request to MCData server B containing the data session resulting criteria to be used by MCData server B.

NOTE 1:	If MCData server A identifies that the criteria require to add participants from another MC system then MCData server A sends an ad hoc group data session request to the MCData server in that MC system.

4a/b.	The MCData server A receives the ad hoc group data session criteria modify response and forwards the response to MCData client 1 in MC system A

5.	MCData server B detects that MCData client 3 meets the criteria for the ongoing ad hoc group data session initiated at MC system A.

5a.	MCData server B sends an ad hoc group data session add user notification message towards MCData server A.

5b.	MCData server A sends an ad hoc group data session request towards MCData client 3 via MCData server B.

5c.	The MCData user 3 is notified of entering an ongoing ad hoc group data session.

5d.	MCData client 3 sends an ad hoc group data session response towards MCData server A via MCData server B.

5e.	MCData server B sends an ad hoc group data session remove user notification message towards MCData server A.

5f.	MCData server A sends an ad hoc group data session leave request towards MCData client 4 via MCData server B.

5g.	The MCData user 4 is notified of leaving an ongoing ad hoc group data session.

5h.	MCData client 4 sends an ad hoc group data session leave response towards MCData server A via MCData server B.

6.	The MCData server A may notify the authorised MCData user of all the users who are added to the on-going ad hoc group data session. This notification may be sent to the authorised MCData user by the MCData server A more than once during the data session when MCData users join or leave the ad hoc group data session. All ad hoc group data session notify messages contain the call resulting criteria used by the MCData server to determine the list of participants to be invited.

7.	The MCData server A adds MCData client 3 to the ad hoc group data session and removes MCData client 4 from the ad hoc group data session.

The MCData servers continuously check whether other MCData clients meet or if participating MCData clients no longer meet the criteria for the ad hoc group data session.

NOTE 2:	If the ad hoc group data session is associated with an ad hoc group emergency alert and the change of criteria caused the modification of the ad hoc group data session participant list then the ongoing ad hoc group emergency alert is modified accordingly.

# * * * End of changes* * *