3GPP TSG-SA WG6 Meeting #70	S6-255274

Dallas, USA 17th – 21st November 2025	(revision of S6-254xxx)

Source:	Ericsson

Title:	Enhancements to XR data transmission using ETI

Spec:	3GPP TR 23.700-51 V1.1.0

Agenda item:	9.5

Document for:	Approval

Contact:	Ashish S Sharma, ashish.sanjay.sharma@ericsson.com

1. Introduction

This paper provides solution for Key issue #1: Transmission enhancement in application layer

2. Reason for Change

The XR application traffic is dynmaic in nature. It could generates large payloads in short period. And due to varying network conditions, it is critical for network to handle large traffic to maintain the QoS and user experience. It becomes challenging in cases of network congestion. 5GS has defined capabilities like Expedited transfer indication support to expedite the transfer of such large payloads. The proposed pCR provides solution to KI#1 to use the underlying 5GS capabilies for dynamic XR traffic characteristics to enhance the XR data delivery.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51 V1.1.0

* * * First Change * * * *

<Proposed change in revision marks>

##### 9.12.2.1.2 SEALDD enabled multi-modal data transmission establishment Figure 9.12.2.1.2-1 illustrate the procedure for establishing multi-modal data transmission connection, and the SEALDD facilitates the multi-modal application to transmit its data between the VAL client and VAL server with RTP packetilization and PDU set inclusion. Pre-condition: - The VAL server has discovered and selected the SEALDD server by CAPIF functions. Figure 9.12.2.1.2-1: SEALDD enabled multi-modal ata transmission connection establishment procedure 1. The VAL server decides to use SEALDD service for multi-modal traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_multi-modalTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, list of requested flows, including SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements. As one of key parameters for multi-modal application, Crossflow measurement can be measured as procedure in clause 9.7.2.2 step 1. 2. Same as step 2 of clause 9.2.2.2. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5 for each requested flow provided in step 1. The AF request may also include the Multi-modal Service ID, which could be determined by the SEALDD server according to VAL service ID and/or VAL server ID. 3-5. Same as step 3-5 of clause 9.2.2.2. 6. The SEALDD client establishes multi-modal transmission connection with the SEALDD server. The request includes the SEALDD client ID, VAL user/UE ID, VAL server ID, VAL service ID, SEALDD-UU flow IDs, Expedited transfer capability support, and traffic descriptors for the multiple flows from the SEALDD client side. The SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-UU flow) to the SEALDD client, and may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the multi-modal transmission connection response. The response also includes a multi-modal SEALDD flow ID which the SEALDD server allocates. And the SEALDD client policy may also be included in its response. The SEALDD client shall then perform authorization checks and reply with a policy configuration response as specified in clause 9.10.2.4. 7-8. Same as step 8-9 of clause 9.2.2.2. NOTE : If multiple requested flows are provided in step 1, the SEALDD client repeats steps 6-8 for each requested flow provided in step 1. The multi-modal application traffic is exchanged between VAL client and VAL server via SEALDD layer as described in clause 9.2.2.2. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP). If the SEALDD client indicates support of Expedited transfer capability, the SEALDD layer performs packetization of the RTP packet and adds a RTP header extension to the SEALDD-UU traffic(e.g. RTP payload )for Expedited data transfer indication as per clause 4.7 TS 26.522 when e.g., the SEALDD Server detects that a big data chunk is to be delivered to the DL or when the SEALDD server receives from the SEALDD client the RTP header extension for Expedited tranasfer to deliver large data chunk in UL. The SEALDD server requests 5GS for ETI as per clause 5.2.6.9.2 TS 23.502 and in clause 9.12.2.1.2. If the PCF rejects the request for expedited transfer as described in 3GPP TS 23.503 [7], the SEALDD server performs expedites the transfer by modifying the QoS flow. #### 9.12.3.3 SEALDD multi-modal transmission connection establishment request Table 9.12.3.3-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the multi-modal transmission connection establishment. Table 9.12.3.3-1: SEALDD multi-modal transmission connection establishment request Information element; Status; Description. SEALDD client ID; M; Identity of the SEALDD client.. Identity; O; The VAL user ID of the VAL user or VAL UE ID. SEALDD-UU flows info; M; Information of the list of SEALDD-UU flows.. > SEALDD-UU flow ID; M; Identity of the SEALDD-UU flow.. > SEALDD traffic descriptor; M; SEALDD traffic descriptor for the flow of the SEALDD client side used to establish SEALDD multi-modal XR connection.. >> Addressing info; M; Indicates the addressing information (e.g. address, port, URL).. >> Transport layer protocol; M; Indicates the transport layer protocol.. VAL server ID; O; Identity of the VAL server.. VAL service ID; O; Identity of the VAL service.. Expedited transfer capability support; O; Indicates the support for Expedited transfer capability in the VAL UE.

* * * Next Change * * * *

#### 5.3.2.1 SEALDD enabled data transmission quality guarantee based on traffic characteristics Figure 5.3.2.1-1: SEALDD enabled data transmission quality guarantee based on traffic characteristics 1. VAL server sends request for SEALDD enabled data transmission quality guarantee based on traffic characteristics. The request may include the VAL Server ID, VAL Service ID, multi-modal flow IDs, media configuration information (i.e., Video: Resolution, encoded method (e.g., Compression Ratio, Color Encoding Model (bits per pixel, bpp)), Frames Per Second (FPS), group of pictures (GOP); Audio: sampling rate, bit depth, number of channels; Haptic: sampling rate, etc.). 2a. SEALDD server calculates the traffic characteristics based on received media configuration information. 2b. If the received media information is not enough to calculate the traffic characteristics, the SEALDD server request analytics or prediction of user plane performance (i.e. average/maximum traffic rate) from NWDAF (or via NEF) as defined in TS 23.288[9], clause 6.14.4. Based on the calculated traffic characteristics the SEALDD server determines the need for expedited data transfer to maintain the data transmission quality guarantee. NOTE: The Bitrate calculation based on traffic rate is implementation based, depending on specific transmission protocol. 3a. Based on the traffic characteristics from step2, SEALDD server could do the transmission enhancement by request the rate control based on the bitrate. The traffic characteristics could be used to calculate the suggested traffic transmission bandwidth to the SEALDD client, as defined in 3GPP TS 23.433 [6], clause 9.8.2.1, step7. Based on the traffic characteristics, the SEALDD server performs packetization of the RTP packet by adding a RTP header extension to the SEALDD-UU traffic(e.g. RTP payload )for Expedited data transfer indication as per clause 4.7 TS 26.522. 3b. SEALDD server request AF Session with required QoS as defined in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3] can be reused to provision the protocol description and the calculated Traffic Periodicity, bitrate. The SEALDD server also requests 5GS for the RTP header extension Expedited data transfer handling as per clause 5.2.6.9.2 TS 23.502 and in clause 9.12.2.1.2.

* * * End of Changes * * * *