3GPP TSG-SA WG6 Meeting #70						S6-255318

Dallas, United States 17th – 21st November 2025

3GPP SA3#99-LI	s3i250674

# 04-07 November 2025, Overland Park, Kansas (US)

# Title:	Reply LS on IMS Data Channel

Response to:	S2-2507657 from SA2 and S3-253806 from SA3 on LI requirements on IMS Data Channel

# Release:	Rel-19

# Work Item:	LI19

Source:	SA3-LI

To:	SA2, CT1, CT4, SA3

Cc:	SA6

Contact Person:

Name:	Nagaraja Rao, Maurizio Iovieno

E-mail Address:	nagaraja.rao@nokia.com, maurizio.iovieno@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA3-LI thanks SA2 and SA3 for the Reply LS on IMS Data Channel in tdocs S2-2507657 and S3-253806 respectively. SA3-LI would like to request SA2 additional clarifications on the IMS Data Channel related aspects.

SA3-LI anticipates that based on the current understanding, the architecture and functions currently specified may need adjustments in some IMS DC scenarios in order to fulfil the LI requirements of countries requiring to provide decrypted content of communication for LI targets.

SA3-LI would kindly request SA2 to confirm whether the SA3-LI understanding of each of the points 1 to 7 mentioned below is correct.

SA3-LI would also kindly request CT1 and CT4 to clarify some specific aspects related to redirection cases, as mentioned in bullet point 8.

NOTE: These points assume that the involved network providers have the LI obligation to report decrypted media for communications related to a LI target.

- SA3-LI understands that when the remote user and the local user are in the same network, it cannot be assumed that the MF and the DCSF are always present at the remote user side of the IMS session. This would limit the possibility to provide IMS DC LI (signalling and decrypted media) in case the remote user is a LI target.

- SA3-LI understands that when the local user and the remote user are in different networks and the remote network supports the IMS Data Channel capabilities, it cannot be assumed that the MF and DCSF are always present at the remote network side of the IMS DC session. This would limit the possibility to provide IMS DC LI (signalling and decrypted media) in case the remote user is a LI target.

- SA3-LI understands that the MF can act as HTTP Proxy only when the media is HTTP based. This would limit the possibility of using the MF as HTTP Proxy to fulfil the LI requirements when media is not HTTP based.

- SA3-LI understands that according to TS 23.228, the IMS DC Application Proxy configuration can only be used in case of network initiated P2P IMS DC connection. This implies that in case of UE initiated P2P, this configuration is not applicable. This would limit the possibility to use the MF as DC application Proxy to fulfil the LI requirements unless IMS DC is P2P initiated by the network.

- SA3-LI understands that when the DC-AS is provided by a third-party provider, in order to allow reporting of decrypted media, the MF shall not act as an UDP proxy; however, when the MF acts as a DC Application Proxy the limitations addressed in bullet 4 apply and, if the MF acts as an HTTP proxy, the limitations listed in bullet 3 apply.

- SA3-LI understands that when the remote network does not support IMS DC capabilities one of the following shall happen:

- Remote network may reject an IMS Data Channel setup request received from the other network.

- The MF present in the local network may provide media transcoding towards the remote network so that the IMS Data Channel related media looks like non-IMS Data Channel related media.

If SA3-LI's assumption in 6b above is correct, SA3-LI requests confirmation on how the local network is able to determine whether or not the remote network has IMS Data Channel capabilities.

- SA3-LI understands that when the remote network does support IMS DC capabilities but the remote UE does not, one of the following shall happen:

- Remote network may reject an IMS Data Channel setup request received from the other network.

b.	The MF present in the local network may provide media transcoding towards the remote network so that the IMS Data Channel related media looks like non-IMS Data Channel related media.

- The MF present in the remote network may provide media transcoding toward the remote UE so that the IMS Data Channel related media looks like non-IMS Data Channel related media.

If SA3-LI's assumptions in 7b and 7c above are correct, SA3-LI requests confirmation on how the local network is able to determine either (i) whether or not the remote UE has IMS Data Channel capabilities or (ii) whether or not the remote network will do transcoding .

- SA3-LI understands that when an IMS session is redirected, the two users present on the IMS session after the redirection can setup an IMS Data Channel connection provided the respective networks have IMS Data Channel capabilities. The assumptions stated in the previous bullets would be applicable to the IMS Data Channel related procedures between the two users.

2. Actions:

To SA2 group.

ACTION: 	SA3-LI kindly request the SA2 group to provide clarifications to the issues raised in this LS in bullets 1 to 7 and consider possible enhancements to the SA2 specs, also based on possible SA3 feedbacks.

To SA3 group.

ACTION: 	SA3-LI kindly request the SA3 group to take the above points into account and provide any feedback/hint to solve the existing issues

To CT1 and CT4 groups.

ACTION: 	SA3-LI kindly request the CT1 and CT4 groups (as appropriate) to provide clarifications to the questions raised in this LS in bullet 8.

3. Date of Next SA3-LI Meetings:

SA3LI Meeting #100	27 – 30 January 2026	Sophia Antipolis, FR