3GPP TSG-SA6 Meeting #70	S6-255334

# Dallas, USA, 17th - 21st Nov 2025												   (revision of S6-255056)

Source:	Airbus, AT&T

Title:	Updated solution #3: Recording MC Data MSRP sessions

Spec:	3GPP TR 23.700-39 V0.4.0

Agenda item:	9.2

Document for:	Approval

Contact:	Jukka Vialen (jukka.vialen@airbus.com)

Jerry Shih (JS9053@att.com)

1. Introduction

This contribution proposes an update to solution to Solution#3.

2. Reason for Change

Solution #3 in 3GPP TR 23.700-39 V0.4.0 is not complete.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-39 V0.4.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[3]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[4]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[5]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[6]	3GPP TS 23.283: " Mission Critical Communication Interworking with Land Mobile Radio Systems; Stage 2".

[7]	3GPP TS 23.289: "Mission Critical services over 5G System; Stage 2".

[8]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[9]	3GPP TS 33.180: "Security of the Mission Critical (MC) service".

[10]	3GPP TS 23.180: " Mission critical services support in the Isolated Operation for Public Safety (IOPS) mode of operation; Functional architecture and information flows".

[xx]	IETF RFC 4975 (September 2007): "The Message Session Relay Protocol (MSRP)".

[yy]	3GPP TS 24.582: “Mission Critical Data (MCData) media plane control; Protocol specification”.

* * * Second Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

B2BUA		Back-to-back user agent

MSRP	Message Session Relay Protocol

* * * Third Change * * * *

## 6.3	Solution 3 (for KI#6): Recording MC Data MSRP sessions

### 6.3.1	Description

#### 6.3.1.0	General

MC Data SDS and FD can use either SIP MESSAGEs (in signalling control plane) or MSRP on a SIP session (media plane). This solution focuses on recording of the SDS and FD traffic when they utilize MSRP on a SIP session.

#### 6.3.1.1	Functional model and reference points

Figure 6.3.1.1-1 shows a simplified functional model for recording MSRP traffic from the MCData server.

Figure 6.3.1.1-1 A simplified functional model for recording MSRP traffic from the MCData server.

The usage of MSRP protocol for MCData SDS and FD is described in 3GPP TS 24.582 [yy].

In this solution, an MSRP B2BUA, amended with recording functionality, is capable of intercepting and duplicating MSRP messages sent between MCData client 1 and 2. MCData server delivers the intercepted traffic to the recording server using MSRP [xx] protocol.

NOTE:	Clause 6.3.1.1 is for information only and shall not be added to a normative TS. The required changes to normative TS(s) are described in the following clauses.

#### 6.3.1.2	Configurations

No changes. All configurations related to MSRP are out of the scope of MC stage 2 TSs.

#### 6.3.1.3	Procedures

Editor's note: Impacts on 3GPP TS 23.280 [2] clause 10.18.3 (Recording procedures) are FFS.

### 6.3.2	Impacts on existing functional entities and reference points

1)	For the existing reference points, Figure 7.3.1.2-3 in 3GPP TS 23.280 [2] (Relationships between reference points of MC service application plane and signalling control planes) needs a small update i.e. adding MSRP to the REC-4 reference point, see Figure 6.3.2-1.

Figure 6.3.2-1 is an updated Figure 7.3.1.2-3 in 3GPP TS 23.280 [2]. The updates are in blue font.

Figure 6.3.2-1: Updated Figure 7.3.1.2-3 in 3GPP TS 23.280 [2]. Relationships between reference points of MC service application plane and signalling control planes

#### 7.5.2.34	Reference point REC-4 (between recording server and MC service servers)

The REC-4 reference point, which exists between recording server and MC service servers, is used for transmitting metadata and media of the communication sessions of target users and target groups from the MC service servers to the recording server.

NOTE:	In this version of this specification the communication sessions that may be recorded are MCPTT and MCVideo group and private calls.

The REC-4 reference point shall use the SIP-2 reference point. for transport and routing of signalling and communication sessions related metadata. If an MC service server and a recording server are served by different SIP cores, then the REC-4 reference point shall also use the SIP-3 reference point. for transport and routing of signalling and communication sessions related metadata.

For the recording of RTP media and related metadata of session-based user communications (e.g. MCPTT and MCVideo private and group calls), MC service server and recording server shall use Session Recording Protocol as specified in IETF RFC-7866 [40]. for establishing and controlling recording sessions between MC service servers and recording server for MCPTT and MCVideo private and group communications. IETF RFC-7866 [40] specifies the use of SIP, Session Description Protocol (SDP), and RTP to establish a recording session.

For the recording of MC Data SDS and FD using MSRP/SIP sessions, the MC Data server and recording server shall use MSRP protocol as specified in IETF RFC 4975 [xx]. The recording logic in MCData server may be located in an MSRP B2BUA.

The detailed recording functionality in the MC service servers and in the Recording server are out of the scope of the present document.

2)	Clause 7.5.2.34 in 3GPP TS 23.280 [2] needs to be updated:

Editor's note: Other impacts on 3GPP TS 23.280 [2] and impacts on 3GPP TS 23.282 [5], if any, are FFS.

* * * End of changes * * * *