3GPP TSG-SA WG6 Meeting #70	S6-255501

Dallas, USA, 17th – 21st November 2025	revision of S6-254512, 5207

Source:	Ericsson

Title:	New Solution for KI#1 on New Service on Support Energy Data Collection and Processing

Spec:	3GPP TR 23.700-44 v0.2.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #1 (TR 23.700-44 clause 5.1), the study aspects on enhance application enablement layer architecture and services to support energy saving include:

- Whether enhancements to existing application enablement layer services or a new application enabler service is required to support energy savings operations in the application enablement layer.

Energy data or processing on energy data may request by consumer to support different applications/operation considering energy requirements. Thus, service on energy data collection and processing is needed for supporting energy saving services. For example, SEAL server/client requests the SEAL server (energy saving) for energy data collection and processing to generate aggregated energy data (e.g. calculate the average/maximum/minimum energy consumption with required granularity) for target entity, to support energy saving of specific operations by using the aggregated energy data. VAL server requests SEAL server (energy saving) for energy data collection from different data sources (from EIF, SEAL client, etc.) and process the collected data with desired data format, to support energy saving of specific application by using the energy data with uniformed format.

This pCR proposes new service to support energy data collection and processing.

2. Reason for Change

This pCR proposes a new solution for Key Issue #1 by introducing a new service to support energy data collection and processing.

3. Conclusions

This paper proposes solution on new service to support energy data collection and processing.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.2.0.

* * * First Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |
| Sol #2 | X |  | X |  |  |  |
| Sol #3 |  |  | X |  |  |  |
| Sol #4 |  |  |  |  |  |  |
| Sol #5 | X |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  |
| Sol #8 |  |  |  |  | X |  |
| Sol #9 |  |  |  |  |  | X |
| Sol #10 |  |  |  |  |  |  |
| Sol #X1 | X |  |  |  |  |  |

* * * Next Change * * * *

## 6.X1	Solution #X1: New Service on Support Energy Data Collection and Processing

### 6.X1.1	Solution Description

#### 6.X1.1.0	General

The following clauses specify the procedure and information flow of SEAL service on energy data collection for support energy saving.

#### 6.X1.1.1	Procedure

Figure 6.X1.1.1-1: Energy data collection and processing

- A consumer (e.g., VAL server, SEAL server/client) sends energy data collection and processing subscription request to SEAL server (Energy Saving). The request includes information as described in Table 6.X1.1.2.2-1.

Editor’s Note:	Whether the SEAL server (Energy Saving) is a standalone energy saving enablement server or integrate energy saving functionality with the existing SEAL enablement server will be decided during solution evaluation and conclusion stage of the study phase.

Editor’s Note:	Whether energy data collection via new energy saving enablement service/functionality or via existing enablement service (e.g. A-DCCF) is FFS.

- The SEAL server (Energy Saving) authenticates and authorizes the request. If authorized, the SEAL server (Energy Saving) determines the data source (e.g. EIF, SEAL client (Energy Saving), and/or other network entities (e.g. OAM, 3rd party energy entity)) for collecting the energy data. It is assumed that SEAL server (Energy Saving) has a registry with the data sources from which it can collect energy data and what kind of data they can provide.

- The SEAL server (Energy Saving) sends energy data collection and processing subscription response to the consumer. The response includes information as described in Table 6.X1.1.2.3-1.

4.	The SEAL server (Energy Saving) collects energy data from the data source.

4a.	The SEAL server (Energy Saving) may interact with 5GC NF for energy data collection, by using the EIF service introduced in 3GPP TS 23.501 [3].

4b.	The SEAL server (Energy Saving) may collect energy data from SEAL client(s) (Energy Saving).

Editor’s Note:	Whether energy information can be collected from other network entities (e.g. OAM, 3rd party energy entity) is FFS.

NOTE:	The procedures for energy data collection need to take user consent into account.

5.	The SEAL server (Energy Saving) aggregates the collected energy data. Aggregation includes combining or performing a statistical operation on the received data according to the data processing requirements in step 1. If necessary, steps 4-5 are repeated to complete all the required data collection.

6.	The SEAL server (Energy Saving) notifies the consumer the energy data collection and processing result. The notification includes information as described in Table 6.X1.1.2.4-1.

#### 6.X1.1.2	Information Flows

##### 6.X1.1.2.1	General

The following information flows are specified for energy data collection and processing.

##### 6.X1.1.2.2	Energy data collection and processing subscription request

Table 6.X1.1.2.2-1 shows the request sent by a SEAL service consumer to a SEAL server (Energy Saving) for energy data collection and processing procedure.

Table 6.X1.1.2.2-1: Energy data collection and processing subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| VAL service ID | M | The identity of the VAL service for which the request applies. |
| Target entity ID(s) | M | The identity(ies) of the target data collection entity(ies) (e.g. VAL UE ID(s), SEAL server ID(s)) |
| Data collection requirements | M | Identifies the requirement for energy data collection. |
| >Energy consumption | O | Identifies the requirement on energy consumption to be collected (e.g. energy consumption per application, energy consumption per task operation). |
| >Time of Interest | M | Indicate the interested time for the data collection. |
| Area of Interest | O | The geographical or service area for which the request applies. |
| Reporting requirements | O | It describes the requirements for reporting. This requirement may include e.g. the type of reporting (periodic or one-time report), the reporting periodicity in case of periodic. |
| Data processing requirements | M | Indicates the requirements on data processing, as defined in Table 6.X1.1.2.2-2. |
| Additional info | O | Identifies addition requirement of the data to be collected (e.g. time stamp of the data, quality of the data). |

Table 6.X1.1.2.2-2: Data processing requirements

| Information element | Status | Description |
| --- | --- | --- |
| Processing type | M | Identifies the processing type, e.g. aggregation, shape data format. |
| Aggregation requirements | O | Identifies the requirements for aggregation based on the energy data. |
| >Average | O (NOTE) | Indicates that calculate the average value of the energy data. |
| >Minimum | O (NOTE) | Indicates that calculate the minimum value of the energy data. |
| >Maximum | O (NOTE) | Indicates that calculate the maximum value of the energy data. |
| Granularity | M | Identifies the granularity for the processing, e.g. per UE, or per application level, or per application service level. |
| Format requirement | O | Indicates the requirement on format for the energy data. |
| NOTE: At least one of the IEs shall be provided if the Processing type is calculation. |  |  |

##### 6.X1.1.2.3	Energy data collection and processing subscription response

Table 6.X1.1.2.3-1 shows the response sent by the SEAL server (Energy Saving) to the consumer for energy data collection and processing procedure.

Table 6.X1.1.2.3-1: Energy data collection and processing subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the request (positive or negative acknowledgement). |
| Subscription ID | O | Identifier of the subscription. It shall be provided if the Result is positive acknowledgement. |

##### 6.X1.1.2.4	Energy data collection and processing notify

Table 6.X1.1.2.4-1 shows the notification sent by the SEAL server (Energy Saving) to the consumer for energy data collection and processing procedure.

Table 6.X1.1.2.4-1: Energy data collection and processing notify

| Information element | Status | Description |
| --- | --- | --- |
| Success status | O (NOTE) | Indicates that data collection was successful. |
| >Data processed result | M | Provides outputs for data collection and processing result (e.g. aggregated result, energy data with required format), it may also include the additional information of the processed data based on request. |
| Failure response | O (NOTE) | Indicates that data collection was failed. |
| >Cause | M | Reason for the failure. |
| NOTE: One of the information elements shall be provided in the output. |  |  |

### 6.X1.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.X1.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***