3GPP TSG-SA WG6 Meeting #70	S6-255502

Dallas, USA, 17th – 21st November 2025	revision of S6-254727, 5208

Source:	Ericsson, InterDigital

Title:	New Solution for KI#2 on Energy Monitoring and Exposing in Application Enablement Layer

Spec:	3GPP TR 23.700-44 v0.2.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

This pCR proposes a solution for the key issue #2.

2. Reason for Change

A solution to address the key issue #2 on monitoring and exposure of energy consumption information within the application enablement layer is proposed in this paper. The Energy Monitoring service enables the consumer, for example, to track the energy usage status (e.g. energy consumption at specific entity exceeds threshold, energy anomaly usage at specific entity) and receive corresponding notifications (e.g. reporting for energy threshold event, reporting for energy anomaly event). Different from data collection and processing service, which is for data aggregation and format shaping, the energy monitoring service focuses on tracking the energy information/status and sending consumers notifications based on event-triggered or periodic frequency. The notification is about, e.g., the reporting of the required event or periodically monitored energy usage status.

3. Conclusions

A solution for energy monitoring is proposed to address the key issue #2.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.2.0.

* * * First Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |
| Sol #2 | X |  | X |  |  |  |
| Sol #3 |  |  | X |  |  |  |
| Sol #4 |  |  |  |  |  |  |
| Sol #5 | X |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  |
| Sol #8 |  |  |  |  | X |  |
| Sol #9 |  |  |  |  |  | X |
| Sol #10 |  |  |  |  |  |  |
| Sol #x |  | X |  |  |  |  |

* * * Next Change * * * *

## 6.x	Solution #x: Monitoring and exposing energy consumption within the application enablement layer

### 6.x.1	Solution Description

#### 6.x.1.0	General

According to key issue #2, there is a need for a service that can monitor and expose the energy information/status (e.g. energy source type change, anomaly energy consumption, etc.). This solution provides energy saving monitoring service, that can be used by the consumer (e.g., VAL server, SEAL client (Energy Saving), SEAL server) to receive notifications regarding energy consumption information/status, which in turn can be useful for energy savings.

#### 6.x.1.1	Energy monitoring procedure

Figure 6.x.1.1-1 illustrates the procedure for a consumer (e.g., VAL server, SEAL client (Energy Saving), SEAL server) to monitor energy information/status.

Figure 6.x.1.1-1: Energy monitoring subscription and notification

1.	The consumer (e.g., VAL server, SEAL client (Energy Saving), other SEAL server) sends an Energy Monitoring Subscription request to the SEAL Server (Energy Saving) with an energy monitoring profile that contain energy monitoring requirements, and reporting requirements. The reporting requirements may include the frequency of reporting is periodic reporting, or triggering event (e.g. Reporting energy threshold event, Reporting energy anomaly event) for event triggered reporting.

The subscription request includes information as defined in Table 6.x.1.2.1-1.

Editor’s Note:	Whether the SEAL server (Energy Saving) is a standalone energy saving enablement server or integrates energy saving functionality with an existing SEAL enablement server will be decided during solution evaluation and conclusion stage of the study phase.

2.	The SEAL server (Energy Saving) checks if the consumer is authorized for the energy monitoring subscription.

3.	The SEAL server (Energy Saving) obtains the energy monitoring requirements from the energy monitoring profile and reporting requirements provided by the consumer in the subscription request. Based on the obtained energy monitoring and reporting requirements, the SEAL server (Energy Saving) determines the data source(s) (e.g., EIF, SEAL server/client) to monitor, and applies for subscription for energy information with the selected data source(s).

4.	The SEAL server (Energy Saving) sends an Energy Monitoring Subscription response to the consumer to indicate the subscription status. The subscription response includes information as defined in Table 6.x.1.2.2-1.

5	The SEAL server (Energy Saving) receives notifications about energy information from the subscribed data source(s), for example, energy consumption per UE from EIF via NEF, energy consumption at the VAL UE (via SEAL client (Energy Saving)).

6.	The SEAL server (Energy Saving) determines whether to send a notification to the consumer based on the reporting requirements in step 1. The reporting can be periodic, or event triggered (e.g. the energy consumption at the VAL UE exceeds the threshold provided in step 1, which triggers SEAL server (Energy Saving) to send notification to the consumer).

7.	The SEAL server (Energy Saving) sends an Energy Monitoring notification to the consumer. The notification includes information as defined in Table 6.x.1.2.3-1.

The consumer may adjust the network requirements (e.g., QoS requirements) to the VAL client for energy saving, based on the energy monitoring data provided by the SEAL server (Energy Saving).

#### 6.x.1.2	Energy monitoring Information elements

##### 6.x.1.2.1	Energy monitoring subscription request

Table 6.x.1.2.1-1 shows request sent by the consumer (e.g. VAL server, SEAL client (Energy Saving), other SEAL server) to the SEAL server (Energy Saving) for the energy monitoring subscription.

Table 6.x.1.2.1-1: Energy monitoring subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Consumer ID | M | The identifier of the consumer (e.g. VAL server, SEAL client (Energy Saving), SEAL server). |
| Target VAL UE ID(s) | M | List of VAL UE(s) for which the energy monitoring applies. |
| Target VAL service ID | O | The identifier of the VAL service for which the energy monitoring applies. |
| Target Slice ID | O | The slice ID (S-NSSAI) for which the energy monitoring applies. |
| Notification endpoint | M | Indicates the address (e.g., IP address, FQDN, URI) where the notifications should be sent. |
| Energy Monitoring profile | M | Represents the energy monitoring profile. Indicates the energy monitoring requirements of the consumer. |
| Reporting requirements | O | The requirements for energy consumption data reporting. Requirements may include, e.g., the type and frequency of reporting (periodic or event triggered), the reporting frequency in case of periodic, and reporting thresholds in case of event triggered. |
| > Reporting periodicity | O | It describes the Frequency of reporting is periodic. |
| > Reporting Event Triggered | O | It describes the event triggered reporting. |
| >> Reporting energy threshold event | O (NOTE) | Indicates the reporting threshold with the corresponding threshold reaching direction (e.g., corresponding energy source type exceeds the limit, slices accumulated energy consumption excess, slice average energy consumption excess, average energy consumption (e.g., per hour) excess, maximum renewable energy excess, slice renewable energy ratio excess). |
| >> Reporting energy anomaly event | O (NOTE) | Indicates the anomaly events for energy consumption or usage (e.g., spike in energy consumptions/usage, high-consumption for a longer interval, black-out). |
| Time validity | O | The time validity of the subscription request. |
| NOTE: Only one of the IEs shall be present. |  |  |

Editor’s Note:	Whether Data Source, Energy Consumption Category or other IEs to be contained in the Energy Monitoring profile is FFS.

##### 6.x.1.2.2	Energy monitoring subscription response

Table 6.x.1.2.2-1 shows response sent by the SEAL server (Energy Saving) to the consumer (e.g. VAL server, SEAL client (Energy Saving), other SEAL server) for the energy monitoring subscription procedure.

Table 6.x.1.2.2-1: Energy monitoring subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result. |
| Subscription ID | O | Represents the identifier of the subscription. This IE shall be present if the subscription status is success. |

##### 6.x.1.2.3	Energy monitoring notification

Table 6.x.1.2.3-1 shows the notification sent by the SEAL server (Energy Saving) to the consumer (e.g. VAL server, SEAL client (Energy Saving), other SEAL server) for the energy monitoring procedure.

Table 6.x.1.2.3-1: Energy monitoring notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Represents the identifier of the subscription. This IE shall be present if the subscription status is success. |
| Slice ID | O | The network slice whose energy monitoring data is provided. |
| Energy monitoring result | M | Energy monitoring result is generated based on aggregation of energy consumption related data collected from the data source, e.g., reporting for energy threshold event, reporting for energy anomaly event. |
| Triggering event | O (NOTE) | The event that triggered the monitoring notification. |
| NOTE: This information element applies to event triggered reporting. |  |  |

### 6.x.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.x.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

* * * End of Changes * * * *