3GPP TSG-SA WG6 Meeting #70	S6-255506

Dallas, USA, 17th – 21st November 2025	revision of S6-255268

Source:	Ericsson

Title:	New Solution for KI#5 Enhance Network Slice Adaptation Considering Energy Information for VAL Application

Spec:	3GPP TR 23.700-44 v0.2.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

This pCR proposes a solution for the key issue #5.

2. Reason for Change

A solution to address the key issue #5 NSCE enhancement for energy efficiency is proposed in this pCR.

3. Conclusions

A solution is proposed to enhance NSCE service on VAL server-triggered slice adaptation for VAL application (in clause 9.11 of TS 23.435) to address key issue #5.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.2.0.

* * * First Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |
| Sol #2 | X |  | X |  |  |  |
| Sol #3 |  |  | X |  |  |  |
| Sol #4 |  |  |  |  |  |  |
| Sol #5 | X |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  |
| Sol #8 |  |  |  |  | X |  |
| Sol #9 |  |  |  |  |  | X |
| Sol #10 |  |  |  |  |  |  |
| Sol #x |  |  |  |  | X |  |

* * * Next of Change * * * *

## 6.x	Solution #x: Enhance Network Slice Adaptation Considering Energy Information for VAL Application

### 6.x.1	Solution Description

#### 6.x.1.1 General

This solution proposes to enhance the NSCE service on VAL server-triggered and network-based network slice adaptation for VAL application (in clause 9.11 of 3GPP TS 23.435 [13]). The enhancement enables NSCE to guide UEs to use different slices to improve energy efficiency and promote the use of renewable energy.

The high-level principles are as follows:

1.	VAL server/VAL requests NSCE server to perform network-based network slice adaptation for VAL application with information considering network slice energy related information.

2.	NSCE service retrieves slices’ status data including energy related information of a slice from 5GC/OAM.

3.	Based on energy related information, the NSCE server performs network slice modification considering energy-related information to improve energy efficiency or promote the use of renewable energy.

#### 6.x.1.2 Procedure

The procedure on VAL server-triggered and network-based network slice adaptation for VAL application in clause 9.11.2 of 3GPP TS 23.435 [13] is enhanced as follows (new text in bold italics):

### 9.11.2 Procedures #### 9.11.2.1 Procedure for VAL server-triggered and network-based network slice adaptation for VAL application - request and response model Figure 9.11.2.1-1 illustrates the VAL server-triggered and network-based procedure where the NSCE server supports the network slice adaptation with the underlying 3GPP system for the VAL UEs of the VAL application. Figure 9.11.2.1-1: Network slice adaptation for VAL application 1. The VAL server sends a network slice adaptation request to the NSCE server for the VAL application (and the VAL UEs within the VAL application). This request may be in the form of exact requested network slice (and optionally DNN) for all the VAL UEs of the VAL application; or indication that the VAL application needs to be remapped to a different network slice (and optionally DNN). The request optionally includes the adaptation threshold of network slice adaptation as defined in Table 9.11.3.1-1. For example: - slice average energy consumption: EnC per reported period - slice energy efficiency KPI: DL DV/EnC, UL DV/EnC - slice renewable energy ratio or renewable energy is used or not - renewable energy schedule Editor’s Note: Whether renewable energy related information in bullet 3 and 4 is available from OAM needs to be coordinated with SA5. 2. [Optional] NSCE server collects the network slice status information, including network slice performance measurements in clause 5.1.1.1, clause 5.1.1.2, clause 5.1.1.3, in 3GPP TS 28.552 [19] and key performance indicators in clause 6.3, energy performance in clause 6.7 (e.g. slice energy consumption, slice energy efficiency), in 3GPP TS 28.554 [20], and network slice related E2E latency analytics report in clause 8.4.2.4.3, in 3GPP TS 28.104 [21] from network slice management functions by utilizing MnS of create MOI operation defined in clause 11.1 and MnS of streaming data reporting service or file data reporting service defined in clause 11.5 and 11.6, 3GPP TS 28.532 [7], and renewable energy related information. 3. The NSCE server processes the request and triggers the network slice configuration per VAL UE within the VAL Application. If network slice status from step 2 is considered, the NSCE server analyses the network slice status information before triggering the network slice configuration. If the threshold is crossed for the current network slice of adaptation and the objective network slice satisfy the requests, NSCE server triggers the network slice configuration per VAL UE within the VAL Application. For example, the slice energy consumption for the current network slice of adaptation crossed the enegy threshold or the renewable energy used for the current newtork slice of adaptation is lower than a energy threshold, the NSCE server triggers network slice modification considering energy-related information to improve energy efficiency or promote the use of renewable energy. 4. The NSCE server acting as AF provides the updated S-NSSAI and DNN per VAL UE. In particular, NSCE server sends this information to the PCF via NEF as part of the AF-driven guidance for URSP determination to 5G system (as specified in 3GPP TS 23.502 [12] clause 4.15.6.10, 3GPP TS 23.503 [17] clause 6.6.2.2, 3GPP TS 23.548 [18] clause 6.2.4). This guidance may update the route selection parameters to indicate different sets of PDU Session information (DNN, S-NSSAI) that can be associated with applications matching the application traffic. The guidance may update RSD priority for the slice according to energy threshold info, i.e. the NSCE sets the higher priority of the RSD for the slice which has higher slice energy efficiency KPI or lower priority for the slice which has higher average energy consumption. NOTE: NSCE server provides the updated S-NSSAI/DNN as a suggestion/guidance to PCF; however it is up to PCF to decide whether to perform the slice/DNN re-mapping 5. Upon successful adaptation of the route selection parameters, the NSCE server provides a network slice adaptation response to the VAL server, providing information on the fulfilment of the network slice adaptation request per VAL application.

#### 6.x.1.3 Information flows

The information flows on network slice adaptation request in clause 9.11.3 of 3GPP TS 23.435 [13] is enhanced as follows (new text in bold italics):

#### 9.11.3.1 Network slice adaptation request Table 9.11.3.1-1 describes the information flow network slice adaptation request from the VAL server to the NSCE server. Table 9.11.3.1-1: Network slice adaptation request Information element; Status; Description. VAL service ID; M; The VAL service ID of the VAL application for which the network slice adaptation may corresponds to.. List of VAL UE IDs; M; List of the VAL UE IDs within the VAL service for which the slice adaptation request corresponds. Requested Network slice related identifier(s); O; Identifier of network slice for which the VAL server requests to use for adaptation. Requested monitored Network slice related identifier(s); O (see NOTE 1); Identifier of the provisioned network slice(s) which are provisioned for the listed UE(s) and requested to be monitored by NSCE server. Requested DNN; O; Indication of the new DNN which is requested.. Requested Adaptation threshold; O (see NOTE 2); The threshold of network slice adaptation. >Requested adaptation threshold of the delay of network slice; O; The network slice delay defined clause 5.1.1.1, 5.1.1.2, 5.1.1.3, in 3GPP TS 28.552 [19] and key performance indicators in clause 6.3 and clause 6.7 (e.g. slice energy consumption in clause 6.7.6 and slice energy efficiency in clause 6.7.3), in TS 28.554 [20], and network slice related analytics report in clause 8.4.2.4.3, in 3GPP TS 28.104 [21], and renewable energy related information.. NOTE 1: If this IE is not present then the NSCE server monitors all the slices provisioned for the listed UE(s) mentioned in this request. If this IE is present, the NSCE server monitors the network slice(s) only indicated by the identifier(s). NOTE 2: The NSCE is requested to adapt UE to the requested network slice only when the status of the provisioned network slice for the listed UE(s) crosses the requested threshold..

### 6.x.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.x.3	Solution evaluation

Editor’s Note:	The solution evaluation of the solution is FFS.

* * * End of Changes * * * *