3GPP SA WG6 Meeting #70	S6-255522

# Nov. 17 – 21st, 2025; Dallas, US. 					(Revision of S6-255143)

Source: 	China Mobile

Title: 	KI#4 Solution: Application enablement layer enhancement for efficient content delivery over satellite access

Document for: 	Approval

Agenda Item: 	9.12

Work Item / Spec.:	FS_5GSAT_Ph4_APP / TR 23.700-02 v0.1.0

Abstract of the contribution: This document proposes a solution to address the KI#4 (TR 23.700-02), i.e., application enablement layer enhancement for efficient content delivery over satellite access.

## 1	Introduction

This pCR proposes a solution to address the KI#4, i.e., application enablement layer enhancement for efficient content delivery over satellite access. The solution targets at enhancing the SEALDD layer with the allocation and provisioning of data storage in on-ground SEALDD client(s) to optimize the content delivery and provide better coverage. The enhanced SEALDD data transmission will leverage the satellite access characteristics for efficient content delivery.

## 2	Reason for Change

The 3GPP TS 22.261[2] provides the service requirements for the support of efficient content delivery. It brings up the potential allocation & provisioning of in-network content caching to improve user experience, reduce backhaul resource usage and utilize radio resource efficiently. The operation of in-network caching includes the location management of the content cache as well as the efficient delivery of content to and from the appropriate content caching applications. The location requirement includes the support of content caching applications in both the network (e.g., AN/access network and CN/core network) and the UE (e.g., terminal equipment, smartphone, etc.).

When the services are extended to accommodate the satellite access, it implies fundamentally three aspects: (1) having large to global coverage via the content caching service; (2) if possible, the content cache should be geographically distributed in AN, CN, and/or in UE;  (3) efficient delivery from application servers (i.e., VAL server) with the consideration of satellite access characteristics.

The 3GPP TS 23.433[3], i.e., the SEALDD enabler, offers not only the advanced data delivery functionalities, but also the storage capabilities. The clauses 4.6 and 9.5 in the Spec. indicate that the SEALDD server supports the data storage and storage management for VAL client/server, SEALDD client and other SEALDD servers. The VAL client and VAL server may coordinate to determine the usage of the SEALDD storage service. Currently, while the content caching service may be provided by the SEALDD server, the normal deployment locations of SEALDD servers, i.e., closer to VAL servers, will make the distributed requirement of content cache less achievable. This impacts the desired ubiquitous coverage for satellite access.

In TS 23.434[4] upon describing the NRM handling the satellite S&F services, the clause 21.3.2.2 says a VAL UE may report (via the NRM client on the UE) the maximum data storage quota which indicates the maximum data storage quota per application layer on the UE for all of services. This suggests a content caching application might be deployable on a UE for better distributedness. This part of the solution proposes to enhance the SEALDD service enabler by providing the data storage capability in SEALDD client(s). Also in the solution, a SEALDD client (in VAL UE) is deployed on-ground over satellite access.

The 3GPP TS 23.434[4] clause 21, SEAL services over satellite access, implies the satellite footprint or application satellite coverage availability information (ASCAI) can be leveraged for efficient content delivery. For example, applications leveraging the discontinuous coverage pattern between UEs and AS/AFs can help applications design themselves for handling discontinuity accordingly. For instance, if the intermittent connectivity pattern is predicted in advance and then exposed to the application enablement layer, then the application layer could use this information to better schedule content delivery to distributed caching applications (e.g. UEs supporting caching applications may be given precedence during a connectivity available interval, UEs with large caching demand may be better off utilizing longer connectivity interval, etc.). This part of the solution supplements the 1st part of the enhanced SEALDD transmission process by leveraging the satellite access characteristics for efficient content delivery.

## 3	Proposal

It is proposed to agree and capture the solution for the inclusion in the TR 23.700-02 v0.1.0.

# * * * * First change * * * *

## 5	Solutions

## 5.1	Mapping of solutions to key issues

Table 5.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| …… |  |  |  |  |  |  |
| Sol #X |  |  |  | X |  |  |

# * * * * next change * * * *

## 5.x	Solution #x: Using SEALDD to enhance application enablement layer for efficient content delivery over satellite access

### 5.x.1	Architecture

### This solution is based on the SEALDD architecture as in 3GPP TS 23.433 [3].

### 5.x.2	Solution description

According to the 3GPP TS 23.433[3], the SEALDD server offers the storage capabilities, which supports the data storage and storage management for VAL client/server, SEALDD client and other SEALDD servers. The VAL client and VAL server may coordinate to determine the usage of the SEALDD storage service. For efficient content delivery with satellite access, while the in-network content caching service may be provided by the SEALDD server, the normal deployment locations of SEALDD servers, i.e., closer to VAL servers, will make the distributed requirement of content cache less achievable. This impacts the desired ubiquitous coverage for satellite access.

In TS 23.434[4] upon describing the NRM handling the satellite S&F services, the clause 21.3.2.2 says a VAL UE may report (via the NRM client on the UE) the maximum data storage quota which indicates the maximum data storage quota per application layer on the UE for all of services. This suggests a content caching application might be deployable on the VAL UE for better distribution. This part of the solution proposes to enhance the architecture of the SEALDD enabler by providing the data storage capability in SEALDD client(s). The SEALDD client (in VAL UE) is deployed on-ground for satellite access.

The 3GPP TS 23.434[4] clause 21, SEAL services over satellite access, implies the satellite footprint or application satellite coverage availability information (ASCAI) can be leveraged for efficient content delivery. For example, applications leveraging the discontinuous coverage pattern between UEs and AS/AFs can help applications design themselves for handling discontinuity accordingly. For instance, if the intermittent connectivity pattern is predicted in advance and then exposed to the application enablement layer, then the application layer could use this information to better schedule content delivery to distributed content cache in VAL UE (e.g. VAL UEs supporting content caching applications may be given precedence during a connectivity available period, UEs with large caching capacity may be better off utilizing longer connectivity interval, etc.). This part of the solution supplements the previous SEALDD layer architecture enhancement proposal by leveraging the satellite access characteristics for efficient content delivery.

### 5.x.2.1	Procedures

### 5.x.2.1.1	SEALDD enabled transmission for efficient content delivery

Figure 5.x.2.1.1-1 illustrates the procedure for the SEALDD server to determine the content caching capability of the SEALDD client, establishing the data transmission connection with the SEALDD client for efficient content delivery (or so-named ECD), as well as the SEALDD layer utilizing the satellite services to transmit contents between the VAL client and VAL server for better global service coverage.

Pre-condition:

-	The VAL server discovers and selects the SEALDD server by CAPIF functions.

-	VAL servers, VAL clients and SEALDD servers are deployed on the ground.

Figure 5.x.2.1.1-1: SEALDD enabled data transmission for efficient content delivery via satellite services

### 1.	The VAL server decides to use SEALDD service to enable efficient content delivery (ECD) for data transmission and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_ECDforSatAccessTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S Data transmission connection information of the VAL server side, ECD for satellite access requirement (e.g., indication, max cache capacity, etc.).

### 2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, SEALDD server allocates the SEALDD-S data transmission connection information (e.g., address/port) of the SEALDD server side to receive the application data packets from the VAL server to be delivered to the VAL UE. The SEALDD server responds with a Sdd_ECDforSatAccessTransmission response.

### NOTE 1:	The SEALDD-S data transmission connection information of the SEALDD server side is optional, if the SEALDD server uses the downlink pull mode to fetch the application data from the address provided by the VAL server in step 1, and uses the uplink push mode to send the application data to the address provided by VAL server.

### 3.	If the SEALDD server receives from the VAL server the ECD for satellite access indication, the SEALDD server may invoke the SEAL NRM service to enquire the content caching capability and the data storage capacity from the VAL UE. A procedure like the TS 23.434 [4] clause 21.3.2.2 could be utilized. The NRM client on a VAL UE can report the content caching capability and maximum data storage quota per application layer on the VAL UE for all of services to the NRM server, which then sends to the SEALDD server.

### NOTE 2:	While the procedure in TS 23.434[4] clause 21.3.2.2 is about VAL UE reporting information from NRM client to NRM server, the procedure for NRM server to enquire and collect the VAL UE information from the NRM client makes no significant difference.

### 4.	If the received ECD indication shows the support of ECD in VAL UE, the SEALDD server and the SEALDD client enable the ECD for satellite access data transmission. Based on the ECD for satellite access requirement received from the VAL server or local configuration, the SEALDD server requires the SEALDD client to set up the content caching service and allocate content cache with provided content storage capacity.

### NOTE 3:	Whether a VAL UE can support the ECD and accordingly store received contents is subject to the regulatory requirements and operator’s policy.

### 5.	The VAL server sends downlink application data to the SEALDD server which delivers it further to the VAL UE with the efficient content delivery or ECD.

### NOTE 4:	Based on implementation, the SEALDD server can calculate the available capacity of the content cache in the VAL UE and adjust the volume of the DL transmission data to the SEALDD client accordingly.

### NOTE 5:	The SEALDD server may receive the DL application data from the VAL server before the ECD is enabled in SEALDD client, during which the SEALDD server may store the application data locally.

### 6.	The SEALDD server controls the efficient downlink application data delivery to the SEALDD client based on the application satellite coverage availability information (or ASCAI).

### NOTE 6:	The SEALDD server can leverage the procedures like in the 3GPP TS 23.434[4] clause 21.2.2 to get the application satellite coverage availability information (or ASCAI).

### 7.	The SEALDD server sends the DL data to the SEALDD client in VAL UE via the 3GPP network.

### NOTE 7:	The SEALDD enabled regular data transmission procedure in the 3GPP TS 23.433[3] clause 9.2.2.2 can be leveraged for the data transmission.

### 8.	The SEALDD client receives the DL data and save it into the content cache. Later, the VAL UE distributes the cached contents to other VAL UEs/VAL clients based on the settings of the content cache applications.

### NOTE 8:	When contents are received and cached in the VAL UE, the VAL UE can distribute the contents via the refence point SEALDD-PC5. The 3GPP TS 23.433[3] clause 7.2 illustrates the architecture for SEALDD enabler service to support VAL UE-to-VAL UE communication. However, how a VAL UE/VAL client distributes the cached contents is implementation specific that is out of the scope of the document.

### 9.	Optionally, the SEALDD server may send Sdd_ECDforSatAcessTransmission notification to the VAL server to influence the DL application data transmission from the VAL server, e.g., if the content cache at the VAL UE is near its capacity, if the VAL UE connection status changes thanks to the variation of satellite connectivity, or if the DL data transmission from the VAL server needs to be adjusted.

### 5.x.2.2	Information flows

### 5.x.2.2.1	SEALDD enabled ECDforSatAccess transmission request

Table 5.x.2.2.1-1 describes the information flow from the VAL server to the SEALDD server to enable the efficient content delivery (ECD) for satellite access transmission service.

Table 5.x.2.2.1-1: SEALDD enabled ECDforSatAccess transmission request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | M | Identifier of specific UE or VAL user |
| SEALDD-S endpoint information | M | Address/port and/or URL of the VAL server to receive the application packets from the SEALDD server. |
| ECD requirement | O | ECD requirement for satellite access (e.g., support indication, max storage size). |
| DL data delivery status subscription indication | O | Indicates the VAL server expected to receive the DL delivery status notification |

### 5.x.2.2.2	SEALDD enabled ECDforSatAccess transmission response

Table 5.x.2.2.2-1 describes the information flow from the SEALDD server to the VAL server to respond to the efficient content delivery (ECD) for satellite access transmission service.

Table 5.x.2.2.2-1: SEALDD enabled ECDforSatAccess transmission response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-S endpoint information | O | Address/port and/or URL of the SEALDD server to receive the packets from the VAL server for application traffic transfer |
| Cause | O (see NOTE) | Indicates the reason for the failure, e.g. SEALDD policy mismatch, ECD not support. |
| NOTE: The IE is only present if the Result is failure. |  |  |

### 5.x.2.2.3	SEALDD enabled ECDforSatAccess transmission notification

Table 5.x.2.2.3-1 describes the information flow from the SEALDD server to the VAL server to notify the events related to the efficient content delivery (ECD) for satellite access transmission service.

Table 5.x.2.2.3-1: SEALDD enabled ECDforSatAccess transmission notification

| Information element | Status | Description |
| --- | --- | --- |
| Event ID | O (see NOTE) | Identifies event SEALDD client connection status for DL ECD e.g., reachable, unreachable, cache near capacity. |
| Identity | M | Identifier of VAL UE or VAL user. |
| VAL service ID | O | Identity of the VAL service. |
| DL data delivery instructions | O (see NOTE) | Indicates the instructions to the VAL server regarding the DL efficient data delivery |
| > Adjust DL data volume | O | Indicates adjusting DL data volume with suggested data volume |
| > Pause DL ECD | O | Indicates pausing DL ECD due to e.g., cache near capacity |
| > Resume DL ECD | O | Indicates enforcing DL ECD due to e.g., more capacity available |
| NOTE: Either Event ID IE or the DL data delivery instruction ID is present. |  |  |

### 5.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# * * * * End of changes * * * *