3GPP TSG-SA WG6 Meeting #70	S6-255528

Dallas, USA 17th – 21th November 2025	(revision of S6-255272)

Source:	Ericsson

Title:	Enhancements to XR data transmission for data burst support

Spec:	3GPP TR 23.700-51 V1.1.0

Agenda item:	9.5

Document for:	Approval

Contact:	Ashish S Sharma, ashish.sanjay.sharma@ericsson.com

1. Introduction

This paper provides solution for Key issue #1: Transmission enhancement in application layer

2. Reason for Change

The 5GS specified capbilities to support bounded latency for time sensitive networks, where the traffic characteristics such as burst arrival time or periodicity are signalled via control plane procedures, are no suitable for XR applications. The XR traffic is dynamic in nature, like XR application generating large number of packets in short period (burstiness in XR traffic) whose periodicity may also dynamically change. 5GS has specified capabilities to provide dynamically changing traffic characteristics via user plane, making use of RTP header extension to indicate the data burst size and the time to the next burst to the 5GCN. The proposed pCR provides solution to KI#1 to use the 5GS capabilities to enhance the XR transmission.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51 V1.1.0.

* * * First Change * * * *

## 5.1	Mapping of solutions to key issues

Table 5.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 |
| --- | --- | --- | --- | --- | --- |
| Sol #1 | x |  |  |  |  |
| Sol #2 |  | x |  |  |  |
| Sol #3 |  |  | x |  |  |
| Sol #4 |  |  |  | x |  |
| Sol #5 | x |  |  |  |  |
| Sol #6 | x |  |  |  |  |
| Sol #7 |  |  |  |  | x |
| Sol #8 |  |  |  | x |  |
| Sol#Y | x |  |  |  |  |

* * * Next Change * * * *

## 5.Y	Solution #Y: Enhancements to XR data transmission for data burst support

This solution provides support in the SEALDD server to indicate to the 5GC whether the downlink traffic will include the information about burst size and time to next burst. It is assumed that the SEALDD server is aware of whether the 5GCN supports the dynamically changing of traffic characteristics via user plane as defined in 3GPP TS 23.501 [X], clause 5.37.10.

### 5.Y.1	Architecture Impacts

None

### 5.Y.2	Solution description

3GPP TS 23.433 [6] defines the possibility to perform packetization and enrich RTP, SRTP traffic by adding RTP HE, so that the SEALDD server, the SEALDD client, optionally in collaboration with the 5GC, apply enahnced control on the delivered traffic. This solution provides support in the SEALDD server to determine the data burst size and the time to the next burst and carry them in an RTP Header Extension as defined in 3GPP TS 26.522 [Y].

#### 5.Y.2.1	Impact to SEALDD procedures

The SEALDD enhancements to XR data transmission for data burst support are based on the of SEALDD services specified in 3GPP TS 23.433 [6], highlighted as follows.

##### 9.12.2.1.2 SEALDD enabled multi-modal data transmission establishment Figure 9.12.2.1.2-1 illustrate the procedure for establishing multi-modal data transmission connection, and the SEALDD facilitates the multi-modal application to transmit its data between the VAL client and VAL server with RTP packetilization and PDU set inclusion. Pre-condition: - The VAL server has discovered and selected the SEALDD server by CAPIF functions. Figure 9.12.2.1.2-1: SEALDD enabled multi-modal ata transmission connection establishment procedure 1. The VAL server decides to use SEALDD service for multi-modal traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_multi-modalTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, list of requested flows, including SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements. As one of key parameters for multi-modal application, Crossflow measurement can be measured as procedure in clause 9.7.2.2 step 1. 2. Same as step 2 of clause 9.2.2.2. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5 for each requested flow provided in step 1. The AF request may also include the Multi-modal Service ID, which could be determined by the SEALDD server according to VAL service ID and/or VAL server ID. 3-5. Same as step 3-5 of clause 9.2.2.2. 6. The SEALDD client establishes multi-modal transmission connection with the SEALDD server. The request includes the SEALDD client ID, VAL user/UE ID, VAL server ID, VAL service ID, SEALDD-UU flow IDs, and traffic descriptors for the multiple flows from the SEALDD client side. The SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-UU flow) to the SEALDD client, and may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the multi-modal transmission connection response. The response also includes a multi-modal SEALDD flow ID which the SEALDD server allocates. And the SEALDD client policy may also be included in its response. The SEALDD client shall then perform authorization checks and reply with a policy configuration response as specified in clause 9.10.2.4. 7-8. Same as step 8-9 of clause 9.2.2.2. NOTE : If multiple requested flows are provided in step 1, the SEALDD client repeats steps 6-8 for each requested flow provided in step 1. The multi-modal application traffic is exchanged between VAL client and VAL server via SEALDD layer as described in clause 9.2.2.2. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP). Based on the data burst size and time to next burst indication received within the protocol description in the request message in step 1, the SEALDD server performs packetization of the RTP packet to include databurst indication in RTP header extension as defined in 3GPP TS 26.522 [TS26522]), and also configures the 5GS to handle the data burst for the XR traffic as per clause 5.2.6.9.2 TS 23.502[TS23502]. #### 9.12.3.1 SEALDD enabled multi-modal transmission request Table 9.12.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting multi-modal transmission service. Table 9.12.3.1-1: SEALDD enabled multi-modal transmission request Information element; Status; Description. VAL server ID; M; Identity of the VAL server. VAL service ID; O; Identity of the VAL service. Identity; O; Identifier of specific UE or VAL user. List of requested flows; M; List of the requested multi-modal flows. >SEALDD-S Data transmission connection information for multi-modal XR; M; Address(s)/port(s) and/or URL(s) of the VAL server to receive the multi-flow packets from the SEALDD server (e.g. audio flow, tactile flow). >QoS information; O; QoS information provided by VAL server. >Protocol description; O; The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), RTP header extension for data burst size and time to next burst indication, packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP..

### 5.y.3	Solution evaluation

This solution addresses KI #1 to support:

-	How could application enabling layer be enhanced to support for XR related policy (e.g. multi-modal XR policy, multi-modal SEALDD policy) configuration, management, synchronization of multiple VAL clients.

This solution provides support in the SEALDD server to determine the data burst size and the time to the next burst and carry them in an RTP Header Extension as defined in 3GPP TS 26.522 [Y]. This solution has no architecture imapct.

NOTE:	The additional information or pre-conditions required to enable the SEALDD server can determine the data burst size and the time to next burst information to include in the downlink traffic will be discussed during normative phase.

* * * Next Change * * * *

### 7.3.1	Key issue #1 solutions evaluation

The open issues studied in the Key Issue #1 are as follows:

-	How could application enabling layer be enhanced to support for policy (Multi-modal flows alignment policy, Synchronization policy, UE-to-UE policy, expedited transfer indication, policy enforcement notification, etc.).

-	How could application enabling layer be enhanced to support for XR related policy (e.g. multi-modal XR policy, multi-modal SEALDD policy) configuration, management, synchronization of multiple VAL clients.

Solution #1, Solution #5, Solution #6 and Solution#y address the key issue 1.

Solution #1 resolves open issue 1) and 2) and enable VAL server subscribes to SEALDD server for policy enforcement notification and be notified when policy is enforced.

Solution #5 resolves open issue 2) and enables SEALDD server to provide suitble QoS requirements based ont the type of XR traffic.

Solution #6 resolves open issue 1) and enhancement to support for multi model flow delay differences measurement.

Solution #y resolves open issue 2) and provides support in the SEALDD server to determine the data burst size and the time to the next burst and carry them in an RTP Header Extension as defined in 3GPP TS 26.522 [Y].

* * * Next Change * * * *

## 8.2	Conclusions of key issue #1

Solution #1, solution#5 and solution #6 can be modified and adopted into normative work with the following principles:

-	Solution #1 can be used to subscribe and notify VAL server of policy enforcement.

-	Solution #5 can be used for XR traffic transmission.

-	Solution #6 can be used for multi model flow delay differences measurement enhancement.

NOTE 1: 	Definition of the multimodal delay difference will be discussed and completed during normative phase.

Solution#y can be modified and adopted into normative work considering that the adopted normative solution solves the NOTE below:

NOTE 2:	The additional information or pre-conditions required to enable the SEALDD server can determine the data burst size and the time to next burst information to include in the downlink traffic will be discussed during normative phase.

NOTE 3:	Enhancing the current procedure or creating a new procedure to support these XR related features, as data burst support, will be addressed in normative work.

* * * End of Changes * * * *