3GPP TSG-SA WG6 Meeting #70	S6-255544

Dallas, USA, 17th – 21st November 2025	(revision of S6-255173)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on update to solution#13

Spec:	3GPP TR 23.700-26 0.2.0

Agenda item:	9.8

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This pCR propose to update the solution#13.

2. Reason for Change

The information flow table is missing from solution#13.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.2.0.

* * * First Change * * * *

7.13.1	Solution description

7.13.1.1	General

This solution address key issue#2, and #4 to introduce a solution for AIoT data processing of the network exposured AIoT inventory/command result.

The inventory results in the notification from 3GPP network is further processed and transformed into the application layer interested service events using the provisioned service data.

7.13.1.2	Procedures

The figure 7.13.1.2-1 shows the high-level procedure of AIoT data processing management and event notification.

Pre-conditions:

1.	The AIoT task initiate service has been invoked by the AIoT application layer consumer, and the AIoT enabler has initiated the AIoT service request to the 3GPP core network as described in sol#6.

Figure  7.13.1.2-1: AIoT Data Processing management

1.	 The AIoT enabler server receives the notification(s) with inventory/command result from the 3GPP core network as described in 3GPP TS 23.369 [3].

2.	The AIoT enabler processes the network expoured data from step 1 and transforms them into service event based on the provisioned service data and the AIoT taks initiate request previously received. The AIoT service event contains the information of “what AIoT device ID(s)”,  “in which area,” and “what happened”. For “what happened”, there are some examples (not limited to) as below:

Type1: AIoT device appearance, the AIoT device is being reported at the service region for the first time;

Type2: AIoT device disappearance, the AIoT device is not being reported during the limited time;

Type3: AIoT device stay, the AIoT device is continuously reported at the same service region;

Type4: AIoT device moving, the AIoT device is reported from different regions.

The reported AIoT device ID is transformed back to the application layer object ID.

Further the AIoT enabler server processes the network expoured data per AIoT service scenario type indicated in the AIoT task initiate service request.

-	Inbound or Outbound type: for all the AIoT devices appearance events during the task execution period, compare the objects ID(s) reported from 3GPP network and the objects ID lists from the AIoT tasks. If the ID(s) can match, then, the application objects can be successfully inbound or outbound; otherwise, it is considered an illegal operation and an alert information is triggered.

-	Inventory type: for all the AIoT devices appearance and stay events, the corresponding objects can be seen as the normal inventory information. However, for the AIoT devices disappearance event, an alert or other method such as specific AIoT device ID inventory should be initiated.

-	Monitor type: for all the AIoT devices appearance, disappearance, moving events, the AIoT device state changed, the AIoT enable layer should record and report.

Editor’s note:	The potential overapping between inbound/outbound type and inventory type and how to resolve it is FFS.

3.	Report the interested AIoT service events to the application layer and the AIoT task is completed.

7.13.1.3	Information flows

Table 7.13.1.3-1 describes the information flow of the AIoT service event notification service (from AIoT enable server to AIoT application layer consumer.

Table 7.13.1.3-1: AIoT service event notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | O | Identity of the VAL service. |
| Interested AIoT service event | M | The interested AIoT service event being detected. The AIoT service event is defined in table 7.6.1.3.1-5. |
| Task execution ouput | M | The additional information with the interested AIoT service event. |
| > AIoT task execution result | M | The output from the AIoT task execution corresponding the interested AIoT service event. For example, for the Interested AIoT service event is inbound/outbound/alarm event, it includes e.g., application object ID(s), type(s), name(s), application region(s). For moving event, it includes the missing e.g., application object ID(s), type(s), name(s), application region list where the application object moves along. |
| > AIoT data | O | Indicates the network reported AIoT device information (AIoT device ID list, target service area, etc.) |