3GPP TSG-SA WG6 Meeting #70	S6-255581

Dallas, USA, 17th – 21st November 2025	(revision of S6-255297)

Source:	Ericsson

Title:	New Solution for KI#2 on Presence Detect in No Drone Zone

Spec:	3GPP TR 23.700-15 v0.2.0

Agenda item:	9.13

Document for:	Approval

Contact:	Ashish S Sharma, ashish.sanjay.sharma@ericsson.com

1. Introduction

As described in Key Issue #2 (TR 23.700-15 clause 4.2), the study on Enhance UAV service utilizing sensing results include:

- How to enhance the UAS application enabler to support No Drone Zone avoidance (detect and notify) utilizing the sensing results.

This pCR proposes to address new SEAL Sensing Server service on presence detect of UAV in No Drone Zone and notify.

2. Reason for Change

To address the study aspects in Key Issue #2, this pCR proposes a new SEAL Sensing Server service on presence detect of UAV in No Drone Zone and notify.

3. Conclusions

This paper proposes solution on new SEAL Sensing Server service on presence detect of UAV in No Drone Zone and notify.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.2.0.

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 | x |  |  |  |
| Sol #2 | x |  |  |  |
| Sol #3 | x |  |  |  |
| Sol #4 |  | x |  |  |
| Sol #5 |  | x |  |  |
| Sol #6 |  | x |  |  |
| Sol #7 |  | x |  |  |
| Sol #8 |  |  | x |  |
| Sol #9 |  |  | x |  |
| Sol #10 |  |  | x |  |
| Sol #11 |  |  |  | x |
| Sol X |  | x |  |  |

* * * Next Change * * * *

## 6.X	Solution #X: Presence Detect of UAV in No Drone Zone

### 6.X.1	Solution Description

#### 6.X.1.1	General

The following clauses specify SEAL Sensing Server service on presence detect of UAV in No Drone Zone and notify.

Pre-condition:

- The UAE server has the No Drone Zone (no UAV flight zone) list of geographical areas.

#### 6.X.1.2	Procedure

Figure 6.X.1.1-1: Presence detect in No Drone Zone and notify

- The UAE Server sends presence detect in No Drone Zone subscription request to the SEAL Sensing Server. The request contains the location information of the UAV and the list of geographical areas labelled as no UAV flight zone. The UAE Server may update the presence detect in No Drone Zone subscription request if the No Drone Zone is changed.

- The SEAL Sensing Server interacts with 5GC NF (NEF) for sensing result.

- The SEAL Sensing Server detects presence of object in the No Drone Zone, based on the sensing result received from 5GC NF (NEF) in step 2. The SEAL Sensing Server determines whether the position of the object matches with a position of the received UAV location information (e.g. trajectory).

- The SEAL Sensing Server sends presence in No Drone Zone notification to the UAE Server, with one or more detected object(s), and the location of the object(s).

- The UAE Server maps the object and UAV ID(s) based on the position of the object and the location of the UAV ID.

- The UAE Server determines actions for the UAV(s), e.g. adjust flight route of the UAV(s).

#### 6.X.1.3	Information Flows

Editor’s Note:	The information flows of the solution is FFS.

### 6.X.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.X.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***