3GPP TSG-SA WG6 Meeting #70	S6-255594

Dallas, United States 17th – 21st November 2025	(revision of S6-255227)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0335 | rev | 1 | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Improvements on CAPIF framework description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph4-APP |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are two types API exposing functions supported in 3GPP API frame system: 1) Border AEF for interacting with external API invoker normally deployed in API gateway. 2) API service logic execution AEF for processing service logical requested by API invoker(s) normally supported by any network/service layer function entity which support exposed services(e.g.,locaion service, QoS gurantee for application traffic transmission). However,there are serveral issues existing in current TS which may cause confusion as the following: - The defintion and function element description of AEF does not cover API service logic execution. - No clear description for distinguishing two types of AEF in the archieture which will cause the cofusion about how to implement and deploy different features of AEF in the real expsoure system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Adding the description of service execution AEF to the CAPIF reference architeture in 6.2 to make it distinguishing-able with entry point AEF. 2. Add the description in annex for the mapping between different 3GPP entities with the service execution AEF to guide the system implementention. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The incompleted description of API framework system will cause some confusion how to use to API framework in the exposure system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1,3.2,6.2.0, 6.2.1, 6.2.2, 6.2.3, 6.3.4,6.3.x(new),6.4.12, anne X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *********************************Next change************************************

6.2.0	Functional model description for the CAPIF

Figure 6.2.0-1 shows the reference point based functional model for the CAPIF.

Figure 6.2.0-1: Functional model for the CAPIF

The CAPIF is hosted within the PLMN operator network (or even an SNPN). The API invoker is typically provided by a 3rd party application provider who has service agreement with PLMN operator. The API invoker may reside within the same trust domain as the PLMN operator network.

In a reference point based model, the API invoker within the PLMN trust domain interacts with the CAPIF via CAPIF-1 and CAPIF-2. The API invoker from outside the PLMN trust domain interacts with the CAPIF via CAPIF-1e and CAPIF-2e. The API exposing function, the API publishing function and the API management function of the API provider domain (together known as API provider domain functions) within the PLMN trust domain interacts with the CAPIF core function via CAPIF-3, CAPIF-4 and CAPIF-5 respectively.

When API exposing functionmay be deployed in a seperated entity which behave as the service communication entry point for third party to connect to invoke the service APIs supported by different AEFs which perform the API logical exxcution within PLMN.  When the AEF acting as service communication entry point for third party received API request from third party, it interacts with the AEF acting as the API logial execution entity via CAPIF-7.

In 3GPP exposure system, 3GPP service execution AEF respresent any 3GPP function entity (e.g., an OAM function entity, or Core network function entity, or an application enabler layer service entity ), as Annex I.

# /****************************** Next changes **********************/

6.3.4	API exposing function

The API exposing function is the provider of the service APIs and is also the service communication entry point of the service API to the API invokers. The API exposing function consists of the following capabilities:

-	Authenticating the API invoker based on the identity and other information required for authentication of the API invoker provided by the CAPIF core function;

-	Requesting the revocation of the API invoker authorization;

-	Enforcing the authorization provided by the CAPIF core function;

-	Supporting authorization revocation;

-	Logging the service API invocations at the CAPIF core function; and

-	Hiding the topology of the PLMN trust domain from API invokers, depending on configured policy. Routing the API invoking messages to/from service execution AEF, acting as API invoker of CAPIF-7/7e interacts with the service execution AEF for API logical execution.

# /****************************** Next changes **********************/

6.4.12	Reference point CAPIF-7 (between the API exposing functions)

The CAPIF-7 reference point, which exists between the API exposing functions acting as third party communication entry point and API exposing functions performing API logical execution belonging to the same trust domain, is used for the forwarding or routing of the API invoker's service API invocation from one API exposing function acting as third party communication entry point to API exposing functions performing API logical execution deployed in the PLMN trust domain.

The CAPIF-7 reference point supports all the functions required for MNO internal API invocation:

- Authentication and authorization mechanism for an API invocation over SBA interface as defined in 3GPP TS 33.501

The CAPIF-7 reference point supports invocation of service APIs originated by the API invoker using CAPIF-2.

NOTE 1:	The aspects related to the specific service API invocation in reference point CAPIF-7 are out of scope of the present document.

NOTE 2:	The security aspects of CAPIF-7 are the responsibility of SA3.

# /****************************** Next changes **********************/

Annex X(informative):3GPP exposure system

Annex X.1 Layered representation of 3GPP network exposure system

X.1.1 General

The figure H1.1-1 is to show how CAPIF system can be used for 3GPP system providers to expose their services supported by different domain.

FigureH.1-1: Layered representation of SEAL service layer in 3GPP network exposure system

NOTE1:	In figure H1.1-1, it assumes that enabler layer clients belong to the same enabler service provider as enabler layer servers. If enabler layer clients are deployed by different party, it should invoke enabler layer APIs via API exposing function and apply to the CAPIF-1/1e.

X.1.2	Functional Elements

X1.2.1	3GPP Exposure layer

3GPP Exposure layer acts entry points for exposing services APIs supported by 3GPP system, to a services APIs customer (the customer can be Application layer/or a third party's re-exposure system). This layer mainly includes API exposing function and CAPIF core function.

The APIs exposed by this layer including CAPIF-1 and CAPIF-2 APIs defined in clause 10 and 11, and also services APIs for requesting the services from service execution functions in different layer and different domains.

X1.2.2	Service execution AEFs at 3GPP service layer

3GPP service layer includes the legacy MNO services like IMS, SMS, and 3GPP application enabler layer services.

3GPP application enabler layer provides value added service based on network layer capabilities (including capabilities supported by Core network and OAM). it provides the following services based on generality level:

- Edge application enabler layer services as described in H1.2.2.1.

- SEAL service layer as described in H1.2.2.2.

- Vertical specific services layer as described in H1.2.2.3.

NOTE2:	In some organizations, this layer is also called value added service layer or service abstraction layer.

X1.2.2.1 Edge application enabler layer

The services and APIs provided by EEC/EES/ECS as defined in 3GPP TS 23.558[20] at this layer, enable an application on mobile UE to discover and select an edge application server (EAS) deployed at Edge site (at or close to 3GPP network border), and application server rediscovery and reselection due to UE mobility or overloaded.

The services and APIs provided by ECSP management system as defined in3GPP TS 28.538 [19] at this layer, enable an Entity within ECSP or third party to trigger EAS instantiation on Edge nodes.

X1.2.2.2 SEAL service layer

The services at this layer defined in 3GPP TS 23.434[3], 3GPP TS23.435[4], 3GPP TS23.436[5], 3GPP TS23.437[6], 3GPP TS23.438[7], 3GPP TS23.482[12] provide the value-added services to enable application layer to utilize 3GPP network and UE capabilities in an easy-to-use way. SEAL services and APIs are generic to any application type.

SEAL layer includes SEAL services server(s) and SEAL client(s).

- SEAL services server(s): it is implemented in network by MNO or 3rd part and are authorized to (1) provide APIs to application and SEAL client, and (2) invoke network APIs to get 3GPP network services.

- SEAL client(s): it is implemented in UE and can be provided by same organization as SEAL service server or other origination e.g., UE vendor/OS and are authorized to (1) provide APIs to application, (2) invoke APIs from SEAL service server to get Network services, or (3) trigger UE modem to initiate NAS procedures to invoke network capability so as to provide service result to application.

X1.2.2.3 Vertical specific services layer

A service at this layer provides certain application layer service functions, which is specific for certain vertical application.

In 3GPP, the application specific service includes 3GPP TS 23.255 [22] for UAV, or 3GPP TS 23.286 [21] for V2X.

X1.2.2.4 User communication service

It provides another type of service layer services, which mainly refers to the legacy user-orient operator applications, like IMS voice, SMS. These applications can also be exposed as the services to a third-party application.

The APIs specified in 3GPP includes 3GPP TS 23.228 [24], or 3GPP TS 23.239 [23]

X1.2.3	Service execution AEFs at 3GPP network connection layer

The smallest granularity services that a 3GPP network layer function entity or OAM function entity, or a 3GPP UE is able to support e.g., 3GPP network connectivity services, QoS control, 3GPP position, slice creation/updating/deletion.

Network functions and capabilities defined at this layer are designed generally to serve multiple applications/scenarios.

NOTE:	The principles of network layer design normally consider how to use one network capability to support multiple scenarios. So normally the capabilities defined at this layer is in a small granularity. These capabilities can be used as element capabilities and been further aggregated to a larger granularity added value service.

In this layer, it also includes the NAS procedure supported by both UE and network layer so as to provide service result to application by UE.

X.1.3	Relationship between different layer

APIs from 3GPP network connection layer and APIs from SEAL service server of application enabler layer can be provided by MNO and offered certain services towards application layer via a unified API framework.

3GPP service layer consumes the API services from 3GPP network connection layer and also terminal side APIs services.

APIs from SEAL client can be provided by MNO or 3rd party e.g., UE vendor and offer certain SEAL layer services towards application. When these API applies, the SEAL client may further invoke APIs from SEAL service server to get Network services, or SEAL client may request network layer services (e.g., PDU session) via UE modem.