3GPP TSG-SA WG6 Meeting #70	S6-255666

Dallas, USA 17th – 21st November 2025	(revision of S6-255437)

Source:	Ericsson

Title:	Pseudo-CR on solution for KI#5, on CCF Support of Roaming Considerations for Service API Invocation

Spec:	3GPP TR 23.700-43

Agenda item:	9.11

Document for:	Approval

Contact:	Fuencisla Garcia (fuencisla.garcia@ericsson.com)

1. Introduction

This pCR provides a solution for KI#5 for TR 23.700-43.

2. Reason for Change

KI#5 describes one existing gap in current deployments, about how to avoid in a controlled way that a service API call is served when e.g. the UE is roaming and no exposure related roaming agreements with the visiting network exists in the home network.

This solution proposes enhancements to the CAPIF framework to enable the determination and correct handling of service API invocations when the UE is roaming by:

- Getting awareness about UE roaming information (whether the UE is roaming or not and where (serving PLMN ID)), by the update of the CAPIF Architecture with the N33 reference point, that enables the CCF to interact, when necessary, with the CSP to retrieve and subscribe UE roaming information via 3GPP CN exposed service APIs.

-	Enhancing CAPIF-3 reference point to support that the AEF can obtain and enforce roaming authorization information.

-	Enhancing the CCF to support the storage and evaluation of policies related to UE roaming conditions. Based on the AEF request to apply roaming policies, the CCF determines whether the roaming information needs to be retreived from the CSP, evaluates the roaming conditions that apply for the service API invocation, and returns the roaming policy evaluation result (roaming authorization information) to the AEF for enforcement.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 (v0.2.0)

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[3]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[4]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[5]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[X]	3GPP TS 23.502: " Procedures for the 5G System (5GS)".

[6]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

* * * Next Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

API provider administrator: An authorized user with special permissions for API provider domain operations.

Exposure roaming agreement: A formal contract between (typically two) CSPs for allowing API Invokers of the home network to use and receive services when the UE is roaming to a visited network.

Roaming policies for exposure: The roaming policies for exposure consist of the conditions that enable an API Invoker to use and receive services when the UE is roaming to a visited network. The roaming policies for exposure can be based e.g., on CSP defined conditions or can be based on API Invoker provided preferences.

* * * Next Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | … |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  | X |  | X |  |  |  |
| Sol #6 |  | X |  |  |  |  |  |
| Sol #X |  |  |  |  | X |  |  |

* * * Next Change * * * *

## 6.x	Solution #x: CCF support of Roaming Considerations for Service API invocation

### 6.x.1	General

This solution relates to KI #5.

KI#5 describes one existing gap in current deployments, experienced in live deployments, about how to avoid in a controlled way that a service API call is progressed towards the VPLMN when e.g. no exposure related roaming agreements with the visiting network exists in the home network. Another problem exists when, e.g., the application invoker preferences to access a service API depends on whether the UE is roaming, or the API invoker preferences indicate that the service API (e.g. a subscription to a event) will be only monitored and detected when the UE is in the HPLMN only.

To ensure the correct handling of a Service API invocation, knowledge of whether the UE is roaming or not is necessary for evaluating roaming policies for exposure, specially for information retrieval (i.e. simple request/response, with not further interactions required). To reduce barriers in the usage of service APIs by applications, the API invoker should not be aware and should not be forced to be aware of the serving PLMN of the UE. To avoid increased complexity in the API Provider domain exposed service APIs and fragmentation derived from different solutions for roaming handling (e.g. per API Provider domain), the service API should not be forced to become aware of the serving PLMN of the UE.

This solution proposes the following enhancements to the CAPIF Common Function of the CAPIF framework to enable the determination and correct handling of service API invocations when the UE is roaming:

- Enhancing the CCF to support the storage and evaluation of roaming policies for exposure based on available UE roaming information.

- Enhancing CAPIF-3 reference point enabling the AEF to obtain and enforce roaming authorization grants.

- Getting awareness about UE roaming information (whether the UE is roaming or not and where (serving PLMN ID)), by the update of the CAPIF Architecture with the N33 reference point, that enables the CCF to interact, when necessary, with the CSP to retrieve and subscribe UE roaming information via CSP exposed service APIs.

Editor's Note:	Whether roaming policies are required or not it is FFS.

Editor's Note:	It is FFS the analysis of a solution based on AEF evaluation of roaming policies.

### 6.x.2	Architecture Impacts

The CAPIF architecture is impacted by the definition of the N33 reference point, that enables the CCF to interact, when necessary, with the CSP to retrieve UE roaming information via CSP exposed service APIs.

Figure 6.x.2-1 shows the updated reference point based functional model for the CAPIF.

Figure 6.x.2-1: Functional model for the CAPIF

The functional model for CAPIF is described in 3GPP TS 23.222 [2] clause 6.2.0.

The reference point based model described in figure 6.x.2-1 represents the N33 reference point, between the CCF and the 3GPP Core Network, which supports the subscription or retrieval of UE roaming information by the CCF from the related exposed Service API. The security aspects of the N33 reference point are specified by SA3.

### 6.x.3	Solution description

#### 6.x.3.0	General

##### 6.x.3.0.1	Roaming Considerations during Service API invocation

Figure 6.x.3.0.1-1 illustrates the procedure to determine and handle the roaming information during service API invocation, showing the interaction between the different CAPIF functions when Roaming policies for exposure need to be applied.

Pre-conditions:

1.	The API Invoker is onboarded and has received an API Invoker identity.

2.	The CAPIF Core Function is configured with roaming policies for exposure (e.g., to restrict service API invocations to when the UE is roaming in a list of VPLMNs).

Figure 6.x.3.0.1-1: Procedure for handling Roaming information during API invocation

- The API invoker requests to obtain authorization information to invoke the service API exposed by the API exposing function (AEF) as specified in 3GPP TS 23.222 [2], clause 8.11.

- The API invoker sends a Service API invocation request to the AEF.

- The API invoker request is authenticated. The AEF may interface with the CCF to obtain API invoker information to obtain security related information as specified in 3GPP TS 23.222 [2], clause 8.16.

- The AEF determines that roaming policies need to be evaluated for the service API invocation and interacts with the CCF by sending an Apply roaming policy request. Based on the accessed service API, the Apply roaming policy request may indicate the retrieval of the roaming authorization information or may also indicate the subscription to changes in the roaming authorization grant.

- The CCF evaluates the roaming authorization policies for the Service API and API invoker. If it is needed to retrieve roaming information for the involved UE, the CCF interacts with the 3GPP Core Network using the N33 reference point (e.g., invoking the Nnef_Location service specified in 3GPP TS 23.502 [x], clause 5.2.6.21).

- The CCF returns to the AEF the result of the roaming policy authorization in the roaming authorization information.

- If roaming authorization is granted, the service API exposed by the AEF applies the regular service logic, e.g., request QoS to 5GC as requested by the service API.

- The API invoker receives the service API invocation response.

##### 6.x.3.0.2	Roaming Authorization changes

Figure 6.x.3.0.2-1 illustrates the procedure where either the UE roaming information changes or the roaming policies change while the service delivered by the service API is running, showing the interaction between the different CAPIF functions when roaming policies for exposure need to be applied.

Pre-conditions:

1.	The API Invoker is getting service from the authorized service API as described in Figure 6.x.3.0.1-1.

Figure 6.x.3.0.2-1: Procedure for handling roaming information changes during service delivery

1a.	The CCF, in collaboration with the 3GPP Core Network, detects that roaming information changes for a UE for whom a previous roaming authorization grant was provided.

1b.	The CCF detects a change in the roaming exposure policies.

2.	The CCF, re-evaluates roaming policies for exposure, and if the roaming authorization information changes, the CCF sends the Apply roaming notification service operation, indicating the roaming authorization information change.

3. The service API exposed by the AEF applies the regular service logic according to the changes in the roaming authorization information. E.g., if the roaming authorization is revoked, the service logic terminates the QoS request in 5GC).

##### 6.x.3.0.3	Apply Roaming Unsubscribe

Figure 6.x.3.0.3-1 illustrates the procedure where the service API invocation is terminated, showing the interaction between the different CAPIF functions when Roaming Control policies need to be applied.

Pre-conditions:

1.	The API Invoker is getting service from the authorized service API as described in Figure 6.x.3.0.1-1.

5

Figure 6.x.3.0.3-1: Procedure for Apply Roaming Unsubscribe

1.	If the AEF subscribed to roaming authorization information changes, the AEF terminates the subscription.

2.	The CCF acknowldeges the unsubscription request.

3.	If there are no other service APIs that require to monitor UE roaming information changes, the CCF terminates the subscription to roaming information changes with the 3GPP Core Network.

#### 6.x.3.1	Information flows

Editor's Note:	It is FFS the analysis of a solution based on access control policies.

##### 6.x.3.1.1	Apply Roaming Policy subscription request

Table 6.x.3.1.1-1 describes the information flows for Apply Roaming Policy subscription request from the AEF to the CAPIF core function.

Table 6.x.3.1.1-1: Apply Roaming Policy subscription request

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | The information that determines the identity of the API invoker |
| Service API identification | M | The identification information of the service API for which invocation is requested. |
| Target UE | M | Target UE whose roaming information is used during roaming policies evaluation (i.e. GPSI). |
| Notification type | M | Indicates whether the notification is roaming information change event based or one time. |
| Notification reception information | O (NOTE) | The information of the subscribing entity for receiving notification about roaming information changes. |
| NOTE: The inclusion of this information element depends on whether the notification type is event based. |  |  |

##### 6.x.3.1.2	Apply Roaming Policy subscription response

Table 6.x.3.1.2-1 describes the information flows for Apply Roaming Policy subscription response from the CAPIF Core Function to the AEF.

Table 6.x.3.1.2-1: Apply Roaming Policy subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of service API invocation. |
| Subscription identifier | O | Identifier of the subscription to roaming authorization changes. This information element shall be present when the result is success and the notification type included in the subscription request indicated to notify based on roaming information change event. |
| Roaming Authorization information | O | The roaming authorization information indicates whether the service API invocation is authorized or not for the involved UE. This information element shall be present when the result is success. |
| Failure Cause | O | Failure Cause. This information element may be present when the result is failure |

##### 6.x.3.1.3	Apply Roaming Policy Notify request

Table 6.x.3.1.3-1 describes the information flows for Apply Roaming Policy notify request from the CAPIF Core Function to the AEF.

Table 6.x.3.1.3-1: Apply Roaming Policy Notify request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The unique identifier of the apply roaming policy subscription |
| Roaming Authorization information | O | The roaming authorization information indicates whether the service API invocation is authorized or not for the involved UE. |

##### 6.x.3.1.4	Apply Roaming Policy unsubscription request

Table 6.x.3.1.4-1 describes the information flows for Apply Roaming Policy unsubscription request from the AEF to the CAPIF core function.

Table 6.x.3.1.4-1: Apply Roaming Policy unsubscription request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The unique identifier of the apply roaming policy subscription |

#### 6.x.3.2	Updates in Configuration Data for CAPIF

The configuration data for CAPIF is specified in 3GPP TS 23.222 [2] table E-1. The updates in Configuration Data for CAPIF to support UE roaming information to authorize API invoker request are highlighted in Italic and Bold format.

Table E-1: Configuration data for CAPIF

| Reference | Parameter description |
| --- | --- |
| Subclause 4.2.2 | List of published service API discovery restrictions |
|  | > Service API identification |
|  | > API invoker identity information |
|  | > Service API category |
| Subclause 4.3.2 | List of service API authorization restrictions per API invoker |
|  | > API Invoker identity information |
|  | > Service API identification |
|  | > Per resource, operation authorization information. (see NOTE 2) |
|  | > Network slice identification |
| Subclause 4.7.2 | List of service API log storage durations |
|  | > Service API identification |
|  | > Service API log storage duration (in hours) (see NOTE 1) |
| Subclause 4.7.4 | List of API invoker interactions log storage durations |
|  | > Service API identification |
|  | API invoker interactions log storage duration (in hours) (see NOTE 1) |
| Subclause 4.10 | List of access control policy (for an API provider domain) per API invoker and optionally per network slice |
|  | > Volume limit on service API invocations (total number of invocations allowed) |
|  | > Time limit on service API invocations (The time range of the day during which the service API invocations are allowed) |
|  | > Rate limit on service API invocations (allowed service API invocations per second) |
|  | > Service API identification |
|  | > API invoker identity information |
|  | > Network Slice Info |
| Subclause 8.34.3 | Group context information |
|  | > Group identifier |
|  | > List of UE IDs (group members) |
|  | > UE identifier of the Group Resource Owner |
|  | > Authorization information related to the list of supported applications |
|  | >> Application identifier |
|  | >> Purpose |
|  | >> Scope |
| (new)Subclause 8.x.y | List of Roaming policies for exposure per API Invoker and per Service API: |
|  | > List of allowed VPLMN IDs. |
|  | > Service API identification |
|  | > API Invoker identity information |
| NOTE 1: If no value is set for the duration, the duration is assumed to be unlimited. NOTE 2: The API invoker request for access authorization to specific resources or operations of a Service API is specified in 3GPP TS 33.122 [12]. |  |

### 6.x.4	Solution evaluation

This section provides solution evaluation based on the open issues specified in the related key issue(s).

* * * End of Change * * * *

<Proposed change in revision marks>