3GPP TSG-SA WG3 #4, London, 16-18 June 1999	S3-99151

3G TS 33.103 V0.0.3 (1999-06)

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

3G Security;

Integration Guidelines

3G TS 33.103 V 0.0.3

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Reference

DTS/TSGS-________

Keywords

Security, Authentication and Key Agreement, Security Information Stored, Location of Security Functions, Parameter Lengths

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorised by written permission.
The copyright and the foregoing restrictions extend to reproduction in all media.

© 3GPP 1999

All rights reserved.

Contents

# Foreword

This document has been drafted by 3GPP TSG-SA WG 3, i.e., the Workgroup devoted to “Security” issues, within the Technical Specification Group devoted to “System Aspects”.

# 1	Scope

This technical specification defines how elements of the security architecture are to be integrated into the following entities of the system architecture.

- Home Environment Authentication Center  (HE/AuC)

- Serving Network Visited Location Register ( SN/VLR)

- Radio Node Controller ( RNC)

- Mobile station User Identity Module (UIM)

- Mobile Equipment  (ME)

This specification is derived from 3G "Security architecture".

The structure of this technical specification is as follows:

Clause 5 lists the security information to be stored in the above entities of the 3G system in terms of Static information and Dynamic information.

Clause 6 defines the external specification of the security-related algorithms in terms of input and output parameters, and the parameter lengths.

The equivalent information for the alternative Temporary Key proposal is included in an appendix to this document

# 2	References

References may be made to:

a)	specific versions of publications (identified by date of publication, edition number, version number, etc.), in which case, subsequent revisions to the referenced document do not apply; or

b)	all versions up to and including the identified version (identified by “up to and including” before the version identity); or

c)	all versions subsequent to and including the identified version (identified by “onwards” following the version identity); or

d)	publications without mention of a specific version, in which case the latest version applies.

A non-specific reference to an ETS shall also be taken to refer to later versions published as an EN with the same number.

## 2.1	Normative references

The above heading is optional - if a need to sub-divide the references clause is felt to be appropriate.

[1]	 TR S3.03 Security Architecture

## 2.2	Informative references

The above heading is optional - if a need to sub-divide the references clause is felt to be appropriate.

# 3	Definitions, symbols and abbreviations

Delete from the above heading those words which are not applicable.

## 3.1	Definitions

Clause numbering depends on applicability. Defined terms should be ordered alphabetically.

For the purposes of the present document, the following definitions apply:

Confidentiality:	The property that information is not made available or disclosed to unauthorised individuals, entities or processes.

Data integrity: 	The property that data has not been altered in an unauthorised manner.

Data origin authentication:  The corroboration that the source of data received is as claimed.

Entity authentication: The provision of assurance of the claimed identity of an entity.

Key freshness: 	A key is fresh if it can be guaranteed to be new, as opposed to an old key being reused through actions of either an adversary or authorised party.

## 3.2	Symbols

Clause numbering depends on applicability. Symbols should be ordered alphabetically.

For the purposes of the present document, the following symbols apply:

||	Concatenation

	Exclusive or

f1	Message authentication function used to compute MAC

f2	Message authentication function used to compute RES and XRES

f3	Key generating function used to compute CK

f4	Key generating function used to compute IK

f5	Key generating function used to compute AK

f6	Encryption function used to encrypt the IMUI

f7	Decryption function used to decrypt the IMUI (=f6-1)

f8	Access link encryption function

f9	Access link message authentication function

K	Long-term secret key shared between the USIM and the AuC

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

3GPP	3rd Generation Partnership Project

AK	Anonymity key

AuC	Authentication Centre

AUTN	Authentication token

CK	Cipher key

EMUI	Encrypted Mobile User Identity

GK	User group key

IK	Integrity key

IMUI	International Mobile User Identity

IPR	Intellectual Property Right

MAC	Medium access control (sublayer of Layer 2 in RAN)

MAC	Message authentication code

MAC-A	MAC used for authentication and key agreement

MAC-I	MAC used for data integrity of signalling messages

PDU	Protocol data unit

RAND	Random challenge

RES	User response

RLC	Radio link control (sublayer of Layer 2 in RAN)

RNC	Radio network controller

SEQ_UIC	Sequence for user identity confidentiality

SDU	Signalling data unit

SQN				Sequence number

UE	User equipment

USIM	User Services Identity Module

XMAC-A	Expected MAC used for authentication and key agreement

XMAC-I	Expected MAC used for data integrity of signalling messages

XRES	Expected user response

Clause numbering depends on applicability. Abbreviations should be ordered alphabetically.

# 4	Overview of the security architecture

## 4.1	Overview of the complete 3G security architecture.

Figure 1 : Overview of the security architecture

Five security feature groups are defined. Each of these feature groups meets certain threats, accomplishes certain security objectives:

- Network access security (I): the set of security features that provide users with secure access to 3G services, and which in particular protect against attacks on the (radio) access link;

- Network domain security (II): the set of security features that enable nodes in the provider domain to securely exchange signalling data, and protect against attacks on the wireline network;

- User domain security (III): the set of security features that secure access to mobile stations

- Application domain security (IV): the set of security features that enable applications in the user and in the provider domain to securely exchange messages.

Visibility and configurability of security (V): the set of features that enables the user to inform himself whether a security features is in operation or not and whether the use and provision of services should depend on the security feature.

## 4.2	Overview of Authentication and Key Agreement Mechanism

Upon receipt of a request from the SN/VLR, the HE/AuC sends an ordered array of n authentication vectors (the equivalent of a GSM “triplet”) to the SN/VLR.

When the SN/VLR initiates an authentication and key agreement, it selects the next authentication vector from the array and sends the parameters RAND and AUTN to the user. The USIM checks whether AUTN can be accepted and, if so, produces a response RES which is sent back to the SN/VLR. The USIM also computes CK and IK. The SN/VLR compares the received RES with XRES. If they match the SN/VLR considers the authentication and key agreement exchange to be successfully completed.

Figure 2: Generation of an authentication vector

Figure 3: User authentication function in the USIM

# 5	Security Information Stored

## 5.1	Information common to Sequence Number and Temporary Key Mechanism

This Clause lists the security information to be stored in the relevant entities of the 3G system in terms of Static information and Dynamic information and relates to the preferred sequence number mechanism, the alternative temporary key mechanism is detailed in the Appendix.  However, sections 5.1 contains information, which is common to both mechanisms.

Notes on tables

Parameter Lengths: The length of the parameters is given to allow estimates of the message sizes on the air interface to be determined and the maximum width for algorithm design. The effective key length used will be determined by national policy.

Lifetimes: The entry in the tables for parameter lifetime is defined in the table below

| ref. | Definition |
| --- | --- |
| a | For the life of the equipment |
| b | For lifetime of a registration (from one registration to the next) |
| c | Changed stored from one authentication to the next |
| d | Lifetime dependent on Home Environment policy |
| e | Lifetime dependent on Serving Node policy |
| f | Lifetime dependent on User Policy |
| g | Lifetime Dependent on Manufacturer Policy |
| h | No need to be stored in non-volatile memory |

Identities: These are shown in the tables, but unless TSG SA WG3 have a specific security issue with the length uniqueness etc there are shown in Italics as these are defined in other TSG working groups.  For example

| IMUI | * 96 bits? |
| --- | --- |

### 5.1.1	Home Environment Authentication Center  HE/AuC

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Long Term Secret Key | K | 128 bits | a |
| User 1 permanent identity | IMUI | * |  |
| User n permanent identity | IMUI | * |  |
| Optional Enhanced user identity confidentiality |  |  |  |
| Home Environment Id | He_id | 32 bit | a |
| user group 1 |  |  |  |
| Group Key | GK | 128 bits | a |
| Group Identity | GI | 32 bits | d |
| user 1 |  |  |  |
| Sequence for UIC | SEQ_UIC | 32 bits | c |
| Encrypted IMUI | EMUI | 128 bit | h |
| user group n |  |  |  |
| Group Key | GK | 128 bits | a |
| Group Identity | GI | 32 bits | d |
| user n |  |  |  |
| Sequence for UIC | SEQ_UIC | 32 bits | c |
| Encrypted IMUI | EMUI | 128 bit | h |
| Network Domain Security |  |  |  |
| Network’s own Private Key ( signing) | PVTK s | < or = 2048 bits | d,e |
| Network’s own Private Key (decryption) | PVTK d | < or = 2048 bits | d,e |
| PKR1 Public Key for network #1 (verfiy) | PUBKv1 | < or = 2048 bits | d,e |
| PKRn Public Key for network #n (encryption) | PUBKen | < or = 2048 bits | d,e |
| PKR1 Public Key for network #1 (verify) | PUBKv1 | < or = 2048 bits | d,e |
| PKRn Public Key for network #n (encryption) | PUBKen | < or = 2048 bits | d,e |
| Symmetric Send Key #i for sending data from X to Y | KSXY(i) | 128 bits | d,e |
| Symmetric Send Key #j for sending data from Y to X | KSYX (j) | 128 bits | d,e |
| Session key Sequence Number (for sending data from X to Y) | I | 32 – 64 bits | d,e |
| Session key Sequence Number (for sending data from Y to X) | j | 32 – 64 bits | d,e |
| Unpredictable Random Value generated by X | RNDX | 128 bits | h |
| Unpredictable Random Value generated by Y | RNDy | 128 bits | h |

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Time Variant Parameter | TVP | 64 bits | h |
| Network Operator Identifier A, B | X, Y | 32 bits | a |
| Network-Wide User Traffic Confidentiality |  |  |  |
| Access link cipher key (traffic) | Ka | 128 bits | c |
| Access link cipher key (signaling) | Ka’ | 128 bits | c |
| Received access link cipher key | Kb | 128 bits | c |
| Received access link cipher key (signaling) | Kb’ | 128 bits | c |
| NWUTC Session key | KS | 128 bits | c |

### 5.1.2	Serving Node Visited Location Register SN/VLR

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Registered User1 Permanent Identity | IMUI |  |  |
| Registered User1 Old Temporary Identity | TMUIo | * |  |
| Registered User1 New Temporary Identity | TMUIn | * |  |
| Optional Enhanced user identity confidentiality for user 1 |  |  |  |
| Encrypted IMUI | EMUI | 128 bit | h |
| Home Environment Id | He_id | 32 bit | a |
| Group Identity | GI | 32 bits | d |
| Registered User n Permanent Identity | IMUI |  | a |
| Registered User n Old Temporary Identity | TMUIo | * | c |
| Registered User n New Temporary Identity | TMUIn | * | c |
| Optional Enhanced user identity confidentiality for user n |  |  |  |
| Encrypted IMUI | EMUI | 128 bit | h |
| Home Environment Id | He_id | 32 bit | a |
| Group Identity | GI | 32 bits | d |

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Network Domain Security |  |  |  |
| Symmetric Send Key #i for sending data from X to Y | KSXY(i) | 128 bits | d,e |
| Symmetric Send Key #j for sending data from Y to X | KSYX (j) | 128 bits | d,e |
| Network-Wide User Traffic Confidentiality |  |  |  |
| Access link cipher key (traffic) | Ka | 128 bits | c |
| Access link cipher key (signaling) | Ka’ | 128 bits | c |
| Received access link cipher key | Kb | 128 bits | c |
| Received access link cipher key (signaling) | Kb’ | 128 bits | c |

### 5.1.3	Radio Node Controller RNC

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Cipher Key – Circuit switched | CKCS | 128 bits | c |
| Cipher Key – Packet switched | CKPS | 128 bits | c |
| Integrity key | IK | 128 bits | c |
| Access link cipher key (traffic) | Ka | 128 bits | c |
| Access link cipher key (signaling) | Ka’ | 128 bits | c |
| Received access link cipher key | Kb | 128 bits | c |
| Received access link cipher key (signaling) | Kb’ | 128 bits | c |

### 5.1.4	USIM

| Name | Symbol | Parameter Length actual or min-max | Lifetime |
| --- | --- | --- | --- |
| Long Term Secret Key | K | 128 bits | a |
| User Permanent Individual Identity | IMUI | * | a |
| Registered User Old Temporary Identity | TMUIo | * | c |
| Registered User New Temporary Identity | TMUIn | * | c |
| Optional Enhanced user identity confidentiality |  |  |  |
| Home Environment Id | He_id | 32 bits | a |
| Group Key | GK | 128 bits | a |
| Group Identity | GI | 32 bits | d |
| Sequence for UIC | SEQ_UIC | 32 bits | c |
| Encrypted IMUI | EMUI | 128 bits | h |
| confidentiality and integrity |  |  |  |
| Cipher Key – Circuit switched | CKCS | 128 bits | c |
| Cipher Key – Packet switched | CKPS | 128 bits | c |
| Integrity Key | IK | 128 bits | c |
| Key set identifier | KSI | 32 bits | c |
| Cipher Key Lifetime | LIFECKCS | 32 bits | d |
| Integrity Key Lifetime | LIFEIKCS | 32 bits | d |
| Cipher Key Use Counter – Circuit switched | USECKCS | 32 bits | a |
| Integrity Key Use Counter – Circuit switched | USEIKCS | 32 bits | a |

5.1.5	Mobile Equipment ME

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Cipher Key Circuit Switched | CKCS | 128 bits | c |
| Cipher Key Packet Switched | CKPS | 128 bits | c |
| Integrity Key | IK | 128 bits | c |
| Access link cipher key (traffic) | Ka | 128 bits | c |
| Received access link cipher key | Kb | 128 bits | c |
| NWUTC Session key | KS | 128 bits | c |

## 5.2	Sequence Number Mechanism

### 5.2.1	Home Environment Authentication Center  HE/AuC

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Sequence Number | SQN | 32-64 bits | d |
| Anonymity Key | AK | 32-64 bits | d |
| Message Authentication Code | MAC-A | 64 bits | Is part of AUTN change with each new request |
| AV1 Random Number | RAND | 128 bits | Is part of AUTN change with each new request |
| Expected Response | XRES | 32-128 bits | Is part of AUTN change with each new request |
| Cipher Key | CK | 128 bits | Is part of AUTN change with each new request |
| Integrity Key | IK | 128 bits | Is part of AUTN change with each new request |
| Authentication Token | AUTN | 97- 129 bits | Is part of AUTN change with each new request |
| AVn Random Number | RAND | 128 bits | Is part of AUTN change with each new request |
| Expected Response | XRES | 32-128 bits | Is part of AUTN change with each new request |
| Cipher Key | CK | 128 bits | Is part of AUTN change with each new request |
| Integrity Key | IK | 128 bits | Is part of AUTN change with each new request |
| Authentication Token | AUTN |  | Is part of AUTN change with each new request |
| Key set identifier | KSI | 32 bits | d |
| Anonymity Key | AK | 32-64 bits | h |
| Mode | MODE | 1 bit | c |
|  | XMAC-S | 64 bits |  |
|  | XAUTS |  |  |
| Option 1 |  |  |  |
| User 1 Counter- Circuit switched | SQNHE/CSHEhe | 32-64 bits | c |
| User 1 Counter – Packet switched | SQNHE/PS | 32-64 bits | c |
| User n Counter- Circuit switched | SQNHE/CSHEhe | 32-64 bits | c |
| User n Counter – Packet switched | SQNHE/PS | 32-64 bits | c |
| Option 2 |  |  |  |
| Global Counter | SQNhe | 32-64 bits | c |

Note: The Authentication center will store up to 10 Authentication vectors

5.2.2	Serving Node Visited Location Register SN/VLR

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| AV1 Random Number | RAND | 128 bits | c |
| Expected Response | XRES | 32-128 bits | c |
| Cipher Key | CK | 128 bits | c |
| Integrity Key | IK | 128 bits | c |
| Authentication Token | AUTN | 1 | c |
| AVn Random Number | RAND | 128 bits | c |
| Expected Response | XRES | 32-128 bits | c |
| Cipher Key | CK | 128 bits | c |
| Integrity Key | IK | 128 bits | c |
| Authentication Token | AUTN | 97 –129 bits | c |
|  | MODE | 1 bit | c |

### 5.2.3	Radio Node Controller 	RNC

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| See Common items section 5.1 |  |  |  |

### 5.2.4	USIM

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Received Random Number | RAND | 128 bits | c |
| Received Authentication Token | AUTN | 97–129 bits | h |
| Computed Anonymity Key | AK | 32-64 bits | h |
| Computed Message authentication Code | XMAC-A | 64 bits | h |
| Retrieved Sequence Number | SQN | 32-64 bits | c |
| Computed Response | RES | 32-128 bits | h |
| Re-synchronisation MAC | MAC-S | 64 bits | h |
| Re-synchronisation token | AUTS | 297–129 bits | h |
| - and common items – section 5.1 |  |  |  |
| Option 1 |  |  |  |
| Counter- Circuit switched | SQNMS/CSHEhe | 32-64 bits | c |
| Counter – Packet switched | SQNMS/PS | 32-64 bits | c |
| SQN Threshold | delta | 24 bits |  |
| Option 2 |  |  |  |
| Sequence Number Window- Circuit switched | SQNMS/CS - WHEhe | 10-30 | c |
| Sequence Number Window- Packet switched | SQNMS/PS - W | 10-30 | c |
| Option 3 |  |  |  |
| Sequence Number List – Circuit switched | LISTSQNCS | 10-30 | c |
| Sequence Number List – Packet switched | LISTSQNPS | 10-30 | c |

### 5.2.5	Mobile Equipment ME

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| See common items – section 5.1 |  |  |  |

# Location of Security Functions

## 6.1 Functions common to Sequence Number and Temporary Key Mechanism

### 6.1.1	Home Environment Key Administration Center HE/KAC

| Block Encryption Algorithm for Network Operators | BEANO | Input: KSXY, Data Output: Encrypted data |
| --- | --- | --- |
| Secure Hash Function | HASH | Input: Data Output: hashed data |
| Public Key encryption Function | E_PK(data) | Input: PK, data Output: edata |
| Public Key decryption Function | D_SK(edata) | Input: SK, edata Output: data |
| Public Key Signing Function |  |  |
| Public Key Verifying Function |  |  |

### 6.1.2	Serving Node Visited Location Register SN/VLR

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| Public Key Signing Function |  |  |
| Public Key Verifying Function |  |  |

### 6.1.3	Radio Node Controller 	RNC

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| Encryption algorithm | F8 | Input: Count, Direction Bearer, Length, CK Output: Keystream |
| Integrity algorithm | F9 | Input: Message, Count, Fresh, IK Output: MAC-I |

### 6.1.4	USIM

| Integrity algorithm | UIA |  |
| --- | --- | --- |

### The location of the integrity function is for further study.

### 6.1.5	Mobile Equipment ME

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| Encryption algorithm | F8 | Input: Count, Direction Bearer, Length, CK Output: Keystream |
| Integrity algorithm | F9 | Input: Message, Count, Fresh, IK Output: MAC-I |
| Network-wide user traffic confidentiality Algorithm | UNA | Input: IV, KS Output: Keystream |

The location of the integrity function is for further study.

6.2		Sequence Number Mechanism

### 6.2.1	Home Environment Authentication Center HE/AuC

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| Message Authentication Function | F1 | Input: K,SQN,RAND, MODE Output: MAC-A |
| Resynchronisation Message Authentication Function | F1* | Input: K, SQNMS , RAND MODE output: MACS |
| Message Authentication Function | F2 | Input: K,RAND Output: XRES |
| Cipher Key Generating Function | F3 | Input: K,RAND Output: CK |
| Integrity Key Generating Function | F4 | Input: K, RAND Output: IK |
| Anonymity Key Generating Function | F5 | Input: K, RAND Output: AK |
| Identity Decryption Function | F7 | Input: GK, EMUI Output: SEQ_UIC IMUI |

### 6.2.2	USIM

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| Message Authentication Function | F1 | Input: K,SQN,RAND, MODE Output: XMAC-A |
| Message Authentication Function | F2 | Input: K,RAND Output: RES |
| Cipher Key Generating Function | F3 | Input: K,RAND Output: CK |
| Integrity Key Generating Function | F4 | Input: K,RAND Output: IK |
| Anonymity Key Generating Function | F5 | Input: K, RAND Output: AK |

# Appendix 1 Temporary Key Mechanism

When the mobile first requests service from the SN/VLR, a random seed RSu created by the user (USIM or terminal) is included in the request message.  The message including RSu is forwarded to the HE/AuC, which generates its own random challenge RSn.  An authentication vector is returned to the SN/VLR.  The vector contains {RSn, RES1, XRES2, KT}, where RES1 is the response to the user’s challenge, XRES2 is the response to the network’s challenge which is expected from the user, and KT is the temporary authentication key shared with the SN/VLR.  The network’s challenge RSn and the network authentication response RES1 are sent to the MS.  If the MS verifies RES1, thereby authenticating the identity of the network, it responds with RES2 and generates the new temporary key KT.  The SN/VLR then verifies that RES2 equals XRES2, thereby authenticating the identity of the USIM, and stores the new temporary key KT.  Furthermore, both the USIM and the SN/VLR immediately use KT with the random seeds RSu and RSn to generate the first session keys CK and IK. This process is shown in Figure 4 below.

Figure 5 shows how the SN/VLR can offer secure service to the USIM without reference to the home system HE/AuC by using the temporary key KT.

Figure 4: Temporary Key Generation Protocol

Figure 5. Locally authenticated session key agreement

## A1	Security Information stored

### A1.1	Home Environment Authentication Centre  HE/AuC

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Dynamic Information |  |  |  |
| Random Seed User | RSU | 128 bits | c |
| AV1Random Seed Network | RSN | 128 bits | c |
| Response to User Challenge RSU | RES1HEhe | 32-128 bits | c |
| Response to User Challenge RSN | XRES2 | 32-128 bits | c |
| Temporary Key | KTHEhe | 128 bits | b |
| AVn Random Seed Network | RSN | 128 bits | c |
| Response to User Challenge RSU | RES1HEhe | 32-128 bits | c |
| Expected Response to Nwk Challenge RSN | XRES2 | 32-128 bits | c |
| Temporary Key | KTHEhe | 128 bits | b |
| Fixed Initial Value | PAR1 | TBD | a |
| Fixed Initial Value | PAR2 | TBD | a |
| Fixed Initial Value | PAR3 | TBD | a |
| Fixed Initial Value | PAR4 | TBD | a |
| Fixed Initial Value | PAR5 | TBD | a |
| - and common items – section 5.1 |  |  |  |

### A1.2	Serving Node Visited Location Register SN/VLR

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Dynamic Information |  |  |  |
| Temporary Key | KT | 128 bits | b |
| Random Seed User | RSU | 128 bits | c |
| Random Seed Network | RSN | 128 bits | c |
| Response to Users Challenge | RES1HEhe | 32-128 bits | c |
| Response to Network Challenge | RES2 | 32-128 bits | b |
| Response to Network Challenge | XRES2 | 32-128 bits | b |
| Cipher Key | CK* | 128 bits | b |
| Integrity Key | IK* | 128 bits | b |
| Response to SN/VLR challenge (local) | RES | 32-128 bits | c |
| Expected response to challenge | XRES | 32-128 bits | C |

* May be computed at HE/AuC

### A1.3	Radio Node Controller RNC

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| See common items – section 5.1 |  |  |  |

### A1.4	USIM

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Dynamic Information |  |  |  |
| Temporary Key | KT | 128 bits | B |
| Random Seed User | RSU | 128 bits | c |
| Random Seed Network | RSN | 128 bits | c |
| Computed Response ( local authent.) | RES | 32-128 bits | B |
| Response to Users Challenge | RES1HEhe | 32-128 bits | B |
| Response to Network Challenge | RES2 | 32-128 bits | c |
| Expected response to network challenge | XRES1 | 32-128 bits | c |
| - and common items – section 5.1 |  |  |  |

### A1.5	 Mobile Equipment

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| See common items – section 5.1 |  |  |  |

## A2	Location of Security Functions

### A2.1	Home Environment Authentication Centre HE/AuC

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| Key Generating Function | F1 | Input: K, RSU, RSN Output: KT |
| Message Authentication Function | F2 | Input: K, RSU, RSN Output: RES1 |
| Message Authentication Function | F3 | Input: K, RSU , RSN Output: XRES1 |
| -and common items |  |  |

### A2.2	Serving Node Visited Location Register SN/VLR

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  | * May be computed at HE/AuC |
| Message Authentication Function (local authentication only) | F3 | Input: KT, RSU, RSN Output: XRES |
| Cipher Key Generating Function | F4 | Input: KT, RSU, RSN Output: CK* |
| Integrity Key Generating Function | F5 | Input: KT, RSU,RSN Output: IK* |
| and common items |  |  |

### A2.3	Radio Node Controller 	RNC

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| See common items |  |  |

### A2.4	Mobile Equipment user identity Module USIM

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| Key generating function | F1 | Input: K, RSU, RSN Output: KT |
| Message Authentication Function | F2 | Input: K, RSU, RSN Output: XRES1 |
| Message Authentication Function | F3 | Input: K, RSU, RSN Output: RES2 |
| Message Authentication Function ( for local authentication) | F3 | Input: KT, RSU, RSN Output: RES |
| Cipher Key Generating Function | F4 | Input: KT, RSU, RSN Output: CK |
| Integrity Key Generating Function | F5 | Input: KT,RSU RSU RSN Output: IK |

### A2.5	Mobile Equipment ME

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| see common items |  |  |

# Appendix 2	 Document history

| Document history |  |  |
| --- | --- | --- |
| 0.0.0 | 2nd May 1999 | Initial draft: Rapporteur- Colin Blanchard |
| 0.0.1 | 28th May 1999 | After review at TSG SA WG3 #3 Bonn 11- 12th May 1999 |
| 0.0.2 | 11th June 1999 | To incorporate comments from Takeshi Igarashi, Adam Berenzweig, Benno Tietz, Takeshi Chikazawa |
| 0.0.3 | 18th June 1999 | After review at TSG SA WG3#4 London 16th –18th June 1999 |

AUTN = (SQN xor AK) || MODE || MAC eg  AUTN = (32-64) + 1 + 64 = 97-129 bits

1

2 AUTS = SQNms  AK || MAC-S