3GPP TSG-SA3 Meeting #102-e 	S3-210264

- e-meeting, 18 - 29 January 2021, Online

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.512 | CR | DraftCR | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NAS based redirection from 5GS to EPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSCAS_5G |  |  |  |  |  |  | Date: |  | 2020-8-17 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It has been agreed by SA in SP- 200883 that for a UE in 5GMM-IDLE mode with suspend indication, the network shall always allocate a new 5G-GUTI after paging and resume of a connection, even if a NAS message is not sent, otherwise, there exists a privacy risk (identification of the UE’s presence in a particular location). SA3 has also aligned 33.501 with SA’s decision. This contribution proposed to add a test case accordingly. The Registration Reject message with an EMM cause which indicates to the UE that the UE shall not use 5GC shall be sent under protection. This is a very important behaviour at AMF. If a UE experience a RLF when using Control Plane CIoT 5GS optimisation only, the AS layer of the UE may trigger an RRCConnectionReestablishment procedure. As there is no AS security available, this procedure is protected using NAS security, a NAS container is included in RRC Reestablishment Request message, and it will be sent by the ng-eNB to the AMF to do verification, the RAN does not know the vefification result, if the RAN receives Connection Establishment Indication message, then the RAN will trigger RRC Reestablishment message to the UE. Otherwise, if the RAN receives UE Context Release Command, the RAN will not send Reestablishment message to the UE. Thus, it should be ensured that the AMF can reply right result. Otherwise, it will cause waste of resources |  |  |  |  |  |  |  |  |
| qu |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a test case that the AMF after paging a UE in idle with suspend indication shall send a new 5G-GUTI to the UE. It is proposed to add one testcase for inactive scenario security handling. Add text case related to RRCConnectionReestablsihment in CP CIoT 5GS Optimization for AMF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not complete for standard. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st Change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 41.001: "GSM Specification set".

[3]	IETF RFC 3871: "Operational Security Requirements for Large Internet Service Provider (ISP) IP Network Infrastructure".

[4]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[5]	CVE-1999-0511, http://cve.mitre.org/cgi-bin/cvename.cgi?name=CVE-1999-0511

[6] 	"Practical recommendations for securing Internet-connected Windows NT Systems", https://support2.microsoft.com/default.aspx?scid=kb;%5BLN%5D;164882.

[7]	X-Force Vulnerability Report, http://www.iss.net/security_center/static/193.php

[8]	IETF RFC 2644: "Changing the Default for Directed Broadcasts in Routers."

[9]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[10]	3GPP TS 33.501: "Security architecture and procedures for 5G system" (Release 15).

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 6749: "OAuth2.0 Authorization Framework".

[13]	3GPP TS 29.501: "Principles and Guidelines for Services Definition".

[xx] 	3GPP TS 33.501: "Security architecture and procedures for 5G system" (Release 16).

********** END OF 1st CHANGE **********

********** START OF 2nd  CHANGE **********

#### 4.2.2.5 	5G-GUTI allocation

##### 4.2.2.5.1	5G-GUTI allocation

Requirement Name: 5G-GUTI allocation

Requirement Reference: TS 33.501 [2], clause 6.12.3

Requirement Description: "A new 5G-GUTI shall be sent to a UE only after a successful activation of NAS security. The 5G-GUTI is defined in TS 23.003 [4].

Upon receiving Registration Request message of type "initial registration" or "mobility registration update" from a UE, the AMF shall send a new 5G-GUTI to the UE during the registration procedure.

Upon receiving Registration Request message of type "periodic registration update" from a UE, the AMF should send a new 5G-GUTI to the UE during the registration procedure.

Upon receiving Service Request message sent by the UE in response to a Paging message, the AMF shall send a new 5G-GUTI to the UE. This new 5G-GUTI shall be sent before the current NAS signalling connection is released.

The AMF shall after paging the UE in CM-Idle with Suspend indicator send a new 5G-GUTI to the UE.

NOTE 1:	It is left to implementation to re-assign 5G-GUTI more frequently than in cases mentioned above.

NOTE 2:	It is left to implementation to generate 5G-GUTI containing 5G-TMSI that uniquely identifies the UE within the AMF."

as specified in TS 33.501 [2], clause 6.12.3.

Threat References: TR 33.926 [6], clause K.2.7.1, Failure to allocate new 5G-GUTI

Test Case:

Test Name: TC_5G_GUTI_ALLOCATION _AMF

Purpose:

Verify that a new 5G-GUTI is allocated by the AMF under test in these scenarios accordingly.

Pre-Conditions:

For the following test case 1, 2, and 3, the following pre-conditions apply.

Test environment with a UE. The UE may be simulated.

Tester has access to the NAS signalling packets sent over N1 interface.

Tester has the knowledge of the UE’s security context used for protecting the Registration Request of type "mobility registration update" and Service Request, including the old 5G-GUTI, ngKSI, UE NR security capability, NAS security context. And the tester shall configure the UE’s security context on the AMF under test.

For the following test case 4, more pre-conditions are required. Both the UE and the AMF under test support UP CIoT 5GS Optimization. The UE has requested the use of UP CIoT 5GS Optimization during the registration procedure, and afterwards the UE has gone to CM Idle with Suspend Indicator.

Execution Steps

Test case 1:

Upon receiving Registration Request message of type "initial registration" from a UE, the AMF sends a new 5G-GUTI to the UE during the registration procedure.

Test case 2:

Upon receiving Registration Request message of type "mobility registration update" from a UE, the AMF sends a new 5G-GUTI to the UE during the registration procedure.

Test case 3:

Upon receiving Service Request message sent by the UE in response to a Paging message, the AMF sends a new 5G-GUTI to the UE.

Test case 4:

The AMF under test is triggered to page the UE in CM Idle with Suspend Indicator. After paging the UE in CM-Idle with Suspend indicator, the AMF shall send a new 5G-GUTI to the UE.

NOTE X:	Test case 4 is only applicable to AMF supporting UP CIoT 5GS Optimization.

Expected Results:

For Test case 1, 2, 3, and 4, the tester retrieves a new 5G-GUTI by accessing the NAS signalling packets sent by the AMF under test over N1 interface during registration procedure.

For Test case 1, 2, 3 and 4, the NAS message encapsulating the new 5G-GUTI is confidentiality and integrity protected by the AMF under test using the NAS security context, which is same as the UE’s NAS security context.

The new 5G-GUTI is different from the old 5G-GUTI.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

********** END OF 2nd CHANGE **********

********** START OF 3rd   CHANGE **********

##### 4.2.2.1.X	NAS based redirection from 5GS to EPS

Requirement Name: NAS based redirection from 5GS to EPS

Requirement Reference: TS 33.501 [xx], clause 6.16.4.

Requirement Description: "When a UE initiates registration procedure with the AMF, the AMF may redirect the UE from 5GC to EPC by including a EMM cause indicating to the UE that it shall not use 5GC, as described in clause 5.31.3 in TS 23.501 [2]. The following requirements apply to Registration Reject message with an EMM cause which indicates to the UE that the UE shall not use 5GC:

-	the AMF shall only send such a Registration Reject message once NAS security has been established between the AMF and the UE; and

" as specified in TS 33.501 [2], clause 6.16.4. "

Threat Reference:  TBD

Test Name: TC_AMF_REDIRCTION_5GS_EPS

Purpose:

Verify that AMF under test does not send a Registration Reject message containing an EMM cause indicating to the UE that the UE shall not use 5GC , if NAS security is not established. .

Pre-Conditions:

TBD

Execution Steps

TBD

Expected Results:

TBD

Expected format of evidence:

TBD

********** END OF 3rd CHANGE **********

********** START OF 4th CHANGE **********

##### 4.2.2.X	RRCRestablishment in Control Plane CIoT 5GS Optimization

Requirement Name: RRCRestablishment in Control Plane CIoT 5GS Optimization

Requirement Reference: TS 38.413 [yy], clause 8.3.8.2

Requirement Description: "Upon receiving the RAN CP RELOCATION INDICATION message, the AMF shall authenticate the request using the NAS-level security information received in the UL CP Security Information IE and if the authentication is successful initiate the Connection Establishment Indication procedure including NAS-level security information in the DL CP Security Information IE.

In case the AMF cannot authenticate the UE's request, the CONNECTION ESTABLISHMENT INDICATION message does not contain security information, and the NG-RAN node shall fail the RRC Re-establishment.

In case of authentication failure, the NG-RAN node and the AMF should locally release the allocated NG resources, if any." as specified in TS 38.413 [yy], clause 8.3.8.2.

Threat References: TR 33.926 [5], clause K.2.X.1 –Failed Verification of UE Identity during RRC Reestablishment Procedure for CP CIoT 5GS Optimization.

Test Case:

Test Name: TC_AMF_REEST_CP_CIOT

Purpose: To verify that the verification of RRC Reestablishment is applied correctly.

Pre-Condition:

Test environment with UE and ng-eNB, which may be simulated. The UE is using Control Plane CIoT 5GS Optimization.

Execution Steps:

A.	Test Case 1

- The UE sends the RRC Connection Reestablishment Request message to the ng-eNB.

- The ng-eNB sends RAN CP RELOCATION INDICATION message to the AMF.

B.	Test Case 2

- The UE sends the RRC Connection Reestablishment Request message to the ng-eNB.

- The ng-eNB sends RAN CP RELOCATION INDICATION message to the AMF. The ng-eNB modifies UL NAS MAC in UL CP Security Information

Expected Results:

For test case 1, the AMF sends CONNECTION ESTABLISHMENT INDICATION to the ng-eNB, and DL CP Security Information is included.

For test case 2, the AMF sends CONNECTION ESTABLISHMENT INDICATION to the ng-eNB, and DL CP Security Information is not included.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

********** END OF CHANGE **********