3GPP TSG-SA3 Meeting #104-e 	S3-212590

# e-meeting, 16 - 27 August 2021											Revision of S3-20xxxx

# Source:	Huawei, HiSilicon

# Title:	A solution preventing information leakage between mutual exclusive slices

# Document for:	Approval

Agenda Item:		5.21 FS_eNS2_SEC

# 1	Decision/action requested

Include the solution in TR33.874

# 2	References

[1]

# 3	Rationale

This contribution proposes a new solutoin to address the KI on simultaneous usage of slices.

# 4	Detailed proposal

pCR

***	BEGINNING OF CHANGES (all text are new) ***

## 6.Y	Solution #Y: Preventing information leakage between mutual exclusive slices

### 6.Y.1	Introduction

It has been introduced in TS23.501 [2] and TS23.502 [3] that a UE cannot access two network slices (S-NSSAIs) simultaneously if these S-NSSAIs do not share a Network Slice Simultaneous Registration Group (NSSRG as defined in TS23.501). In this solution, the 5G system makes sure the security context are not shared between are not shared for these slices not sharing NSSRG.

### 6.Y.2	Solution details

In order to prevent information leakage between mutual exclusing slices, e.g., the slices are not sharing NSSRG, the 5G system should make sure the security context are not shared amongst them as follows:

1. Deregistration: when a UE is derigisted, the AMF/UE should delete its security context if the UE is subscribed to mutual exclusive slices.

2. Registration: Primary authentication should be performed if a UE is requesting to register a slice mutually exclusive to the slices it had accessed previously (The UE can not access the slices at the same time as specified in the TS23.501/TS23.502)

### 6.Y.3	Evaluation

TBC

***	END OF CHANGES	***