3GPP TSG-SA3 Meeting #107-bis-e 	draft_S3-221419-r1

- e-meeting, 16 - 20 May 2022                                                 merger of S3-221330, S3-221429, S3-221447, S3-221496

- Source:	Huawei, HiSilicon

- Title:	Privacy of information over the UE-to-UE Relay

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution is accepted by the group.

# 2	References

NA

# 3	Rationale

UE-to-UE Relay was in the scope of R-17 and 3 Key issues was discussed and agreed in R-17 timeline, the clause 5.8 in TR 33.847 was copied in this contribution to the new TR33.740.

Merger of S3-221330, S3-221429, S3-221447 and S3-221496.

# 4	Detailed proposal

********* Begin 1st change*********

## 5.z	Key Issue #z: Privacy of information over the UE-to-UE Relay

### 5.z.1	Key issue details

3GPP system has to be able to protect the privacy of identities exchanged in the communications between peer UEs over a UE-to-UE Relay. Failure to protect the privacy of identities of peer UEs communicating over the UE-to-UE Relay will open vulnerability in 5GS and allow various privacy attacks including tracing and tracking of identities.

TR 23.700-33 [xx] Key Issue #1: Support of UE-to-UE Relay, has the following key issue:

‘-	How to enhance the system architecture to provide security/privacy protection for relayed connections.

...

NOTE 3:	For security/privacy protection aspects, coordination with SA WG3 is needed.’

### 5.z.2	Security threats

Failure to protect the privacy of identities exchanged in the communications between the peer UEs over the UE-to-UE Relay will open vulnerability in 5GS and allow various privacy attacks including tracing and tracking of identities.

The existing Link identifier update procedure specified in TS 33.536 [yy] provides privacy of the identities on a per unicast link basis (e.g., the link between a UE and the UE-to-UE Relay). Therefore an attacker may be able to link identities exchanged over the link between a UE and the UE-to-UE Relay to those exchanged over the corresponding link between the peer UE and the UE-to-UE Relay

Path switch between UE-to-UE Relay UEs is a new feature aiming to preserve user experience. Such preservation may be achieved by making certain elements (e.g., IP addresses) of user experience persistent across sessions and UE-to-UE Relays.  Persistent parameters may leak unique attributes associated with UEs and other ProSe entities and allow privacy attacks on these entities (e.g., UEs). Failure to protect the privacy of entities and identities during UE to UE Relay path change will open vulnerability in 5GS and allow various privacy attacks including tracing and tracking of entities and identities.

### 5.z.3	Potential security requirements

The 5G System should provide means for mitigating trackability attacks on peer UEs during communications over a UE-to-UE Relay including during the UE-to-UE Relay path switch.

The 5G System should provide means for mitigating linkability attacks on peer UEs during communications over a UE-to-UE Relay including during the UE-to-UE Relay path switch. ******** End of change*********