3GPP TSG-SA3 Meeting #108e-AdHoc 	S3-222626

e-meeting, 10 - 14 October 2022

Title:	LS on the data and analytics exchange between two NWDAFs in different PLMNs

Response to:	LS S2-2207142 on LS OUT to GSMA on the data and analytics exchange between two NWDAFs in different PLMNs from 3GPP SA WG2

Release:	18

Work Item:	FS_eNA_Ph3

Source:	SA3 Meeting #108e-AdHoc

To:	3GPP SA WG2

Cc:	GSMA

Contact person:	German Peinado

german.peinado@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

SA3 would like to thank SA2 for their LS S2-2207142 on data and analytics exchange between two NWDAFs in different PLMNs in the context of the Rel-18 study TR 23.7000-81.

S3 study TR 33.738 addresses this issue in KI#1. The study has not been concluded yet, so the provided feedback is preliminary and based on the identified security requirements so far.

Question 1:  Does SA3 have feedback to the security aspects outlined in the key issue description? For example, are there any privacy and security protection for data and/or analytics exchange between PLMNs (VPLMN and HPLMN) already defined in SA3 or are there any privacy and security issues foreseen when exchange data and/or analytics between PLMNs?

SA3 response: KI#1 of TR 33.738 describes the identified threats and potential security requirements with respect to the key issue description of SA2. The security requirements require in general the 5GS to support confidentiality, integrity, and replay protection for data and analytics exchange between PLMNs, and also to be able to control the amount of exposed data and to abstract or hide internal network aspects. There are security mechanisms available in the SA3 specifications (TS 33.501, appendix X) to enforce the authorization of the services (i.e., OAuth 2.0), and filter the data to be exposed as per TS 23.288, which may be extended for inter PLMN data and/or analytics exchange. Further mechanisms (e.g., anonymization) are still under study phase.

Question 2: Whether it is necessary to verify user consent for UE-specific data and/or analytics by the VPLMN? If so, what scenario required user consent verification? How to verify user consent in roaming scenario in VPLMN?

SA3 response: Yes, it is necessary, if analytics provided by the VPLMN requires UE-specific data and/or analytics user consent verification is required, as it is required for non-roaming scenarios. Thus, NDWAF in VPLMN needs to check the user consent flag in HPLMN UDM. If HPLMN NWDAF collects data from VPLMN the check on user consent can be done in HPLMN UDM.

Please note that SA3 has initiated a UC phase 2 study (TR 33.896) where the user consent in roaming scenarios is studied, including eNA.

# 2	Actions

To SA2

ACTION: 	Take the above information in consideration. More detailed information will be provided once the corresponding studies in S3 (TR 33.738, TR 33.896) are concluded.

# 3	Dates of next TSG SA WG 3 meetings

SA3#109	14 - 18 November 2022	Toulouse (France)

SA3#110e	16 - 20 January 2023	Online (electronic meeting)