3GPP TSG-SA3 Meeting #108e 	draft-S3-221809-r1

- e-meeting, 22 - 26 August 2022

- Source:	Nokia, Nokia Shanghai Bell, NTT Docomo

- Title:	New Ki on securing API access from subscriber to resources of other subscriber

- Document for:	Discussion

Agenda Item:	5.11

# 1	Decision/action requested

Please add to FS_SNAAPPY new Key Issue on securing API access from subscriber to resources of other subscriber.

# 2	References

[1]	3GPP TR 23.700-95: "Study on application enablement aspects for subscriber-aware northbound API access"

[2]	3GPP TR 33.884: "Study on security of application enablement aspects for subscriber-aware northbound API access"

# 3	Rationale

# The discussion paper S3-221806 summarizes the relevant status quo of security of 3GPP northbound interfaces and proposes to structure FS_SNAAPPY according to three scenarios reflected in dedicated KIs.

The key issue prosed in this contribution relates to Scenario 1 discussed in S3-221806.

# 4	Detailed proposal

It is proposed to add the Key Issue described below to [2]

****Start of 1st Change ****

# 5	Key issues

## 5.X	Securing API access from subscriber to resources of another subscriber

### 5.X.1	Key issue details

In some of the use cases relevant in the context of Subscriber Aware Northbound Access to 3GPP APIs a subscriber might use an application (API invoker) to access resources owned by a different subscriber (resource owner).

As an example, in a cooperative gaming situation one gamer might use the gaming application or gaming platform to increase the QoS of all participating gamers.

This API access may originate from an application server in the network, (i.e. an AF, cf. Key issue #S3-222249, aka Ki#2) or from an application on the UE (cf. Key issue #1)

From a security point of view the most important aspect is the ability of the resource owner to control permissions related to which subscriber using which application is allowed to access which of her resources.

In addition, the requesting subscriber needs to be able to provide or deny consent to an API invoker requesting resources owned by another subscriber on behalf of the requesting subscriber.

The analysis of the KI issue shall include the following topics :

- Authorization and Validation of API Requests (cf. KI# S3-221968, aka Ki#5)

- OAuth Grants, Scopes, and Tokens (cf. KI#new doc ki#6)

- Dynamic Resources (cf, KI#new docki#7)

### 5.X.2	Threats

Missing or erroneous authorization might allow malicious parties to misconfigure the 5G system.

### 5.X.3	Potential security requirements

API invokers acting on behalf of a subscriber shall not be able to access resources owned by another subscriber without the authorization of the resource owner.

API invokers acting on behalf of a subscriber shall not be able to access resources owned by another subscriber without the authorization of the requesting subscriber.

The 5G system shall be able to limit the scope of API requests received from an API invoker acting on behalf of a subscriber according to the authorizations granted by the resource owner to the requesting subscriber.

## 5.X	Key issue #X: <Title>

### 5.X.1	Key issue details

### 5.X.2	Threats

### 5.X.3	Potential security requirements

****End of 1st Change ****