- 3GPP TSG-SA3 Meeting #109e-AdHoc	draft_S3-230097-r1812

- e-meeting, 16th – 20th January, 2023

- Source:	Huawei, Hisilicon, Thales, Ericsson, Samsung, Xiaomi

- Title:	Conclusion on KI#2.1 Authentication and authorization of the EEC/UE by the ECS/EES

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This contribution proposes a new conclusion for EDGE2.

# 2	References

NA

# 3	Rationale

It is specified in TS 33.558 to support authentication and authorization of EEC/UE by the ECS/EES, which is aligned with the statement in Solutions #1, #2, #13 and #14. Hence, It is suggested to reuse authentication and authorization mechanism specified in TS 33.558. Besides, server side certificate-based TLS authentication can be used if the AKMA or GBA is not applicable in TS 33.558, which is recommended to be clarified.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.739.

***BEGIN OF First CHANGE***

## 7.X		Conclusions for Key Issue #2.1

Server side certificate-based TLS authentication is mandatory supported.

Editors Note: Further conclusion is FFS.

*** END OF First CHANGE ***