- 3GPP TSG-SA3 Meeting #109e-AdHoc	S3-230097-r13

- e-meeting, 16th – 20th January, 2023

- Source:	Huawei, Hisilicon, Thales, Ericsson, Samsung, Xiaomi

- Title:	Conclusion on KI#2.1 Authentication and authorization of the EEC/UE by the ECS/EES

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This contribution proposes a new conclusion for EDGE2.

# 2	References

NA

# 3	Rationale

It is specified in TS 33.558 to support authentication and authorization of EEC/UE by the ECS/EES, which is aligned with the statement in Solutions #1, #2, #13 and #14. Hence, It is suggested to reuse authentication and authorization mechanism specified in TS 33.558. Besides, server side certificate-based TLS authentication can be used if the AKMA or GBA is not applicable in TS 33.558, which is recommended to be clarified.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.739.

***BEGIN OF First CHANGE***

## 7.X		Conclusions for Key Issue #2.1

It is concluded to reuse authentication and authorization mechanism specified in TS 33.558 [4] for this key issue.

Further, it is concluded that Public key signature based authentication with server side certificate or raw public key based TLS authentication is used as the default authentication method. It is proposed to take solution#x as the basis for normative work, for default authentication between UE/EEC and ECS/EES when there is no support for common mutual authentication method.

It is concluded that token-based solution is option for EEC/UE authentication, when default authentication method is used (i.e., server side certificate/raw public key based authentication and client side token based authentication).

NOTE: Token based solution can be left for implemenation.

*** END OF First CHANGE ***