- 3GPP TSG-SA3 Meeting #109e-AdHoc	draft_S3-230097-r17

- e-meeting, 16th – 20th January, 2023

- Source:	Huawei, Hisilicon, Thales, Ericsson, Samsung, Xiaomi

- Title:	Conclusion on KI#2.1 Authentication and authorization of the EEC/UE by the ECS/EES

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This contribution proposes a new conclusion for EDGE2.

# 2	References

NA

# 3	Rationale

It is specified in TS 33.558 to support authentication and authorization of EEC/UE by the ECS/EES, which is aligned with the statement in Solutions #1, #2, #13 and #14. Hence, It is suggested to reuse authentication and authorization mechanism specified in TS 33.558. Besides, server side certificate-based TLS authentication can be used if the AKMA or GBA is not applicable in TS 33.558, which is recommended to be clarified.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.739.

***BEGIN OF First CHANGE***

## 7.X		Conclusions for Key Issue #2.1

It is concluded to reuse authentication and authorization mechanism specified in TS 33.558 [4] for this key issue. It is also recommended that server side certificate-based TLS authentication can be used

Key derived based on EEC ID and NAF key/AKMA application key of EES/ECS can be utilized to realize mutual authentication between EEC and EES/ECS.

Note: EEC authentication procedure assumes that EEC can be authenticated by the UE. And the authentication mechanism is out of scope of 3GPP.

It is concluded that token-based solution is an option for EEC/UE authentication, when default authentication method is used (i.e., server side certificate based authentication and client side token based authentication).

NOTE: Token based solution can be left for implemenation.

It is agreed that AKMA and UE ID API are options to be used by the ECS/EES to verify the GPSI if necessary.

*** END OF First CHANGE ***