3GPP TSG-SA3 Meeting #109Adhoc-e 	draft S3-230235 r1

- e-meeting, 16th – 20th January, 2023

- Source:	ChinaTelecom

- Title:	Update to conclusion of KI#2 in TR 33.740

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes to add a Solution for UE-to-UE Relay discovery security.

# 2	References

[1]	3GPP TR 33.740: "Study on security aspects of Proximity Based Services (ProSe) in the 5G System (5GS); Phase 2".

# 3	Rationale

This contribution proposes to update the conclusion of  Key issue #2 in TR 33.740 [1].

# 4	Detailed proposal

**** 1st change****

## 7.2	Key Issue #2: Security of UE-to-UE Relay

For Key Issue #2, the following statements are agreed:

Regarding End-to-End security:

- For L2 relay, the source UE and the target UE can establish a secure PC5 link over the L2 relay after the per hop link is established.

Regarding hop-by-hop security:

- For L3 relay,

- when UE-to-UE Relay is in coverage, the source/target UE and the UE-to-UE Relay can establish a secure PC5 link over the L3 relay with network assistance or without network assistance.

- When UE-to-UE Relay is out of coverage, the source/target UE and the UE-to-UE Relay can establish a secure PC5 link over the L3 relay without network assistance.

- With network assistance, the similar security procedure as PC5 security for 5G ProSe Communication via 5G ProSe Layer-3 UE to-Network Relay as defined in TS33.503 [x] can be reused. In this procedure, the source/target UE takes the role of the Remote UE and the U2U Relay takes the role of the U2N Relay as described in clause 6.3.3 of TS33.503 [x].

- Without network assistance, the similar security procedure as PC5 security for unicast mode 5G ProSe Direct Communication as defined in TS33.503 [x] can be reused.

NOTE: The selection or negotiation methods between the PC5 security procedure (between the source/target UE and U2U relay) with and without network assistance can be determined in the normative work phase.

Editor’s Note: further conclusion is to be updated.

**** End of change****