3GPP TSG-SA3 Meeting #109AdHoc-e 	draft_S3-230390-r4

- Electronic meeting, 16 - 20 January 2023

- Source:	Xiaomi

- Title:	KI#2, New Sol on User authorization revocation for API invocation procedure

- Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

It is requested to approve the new solution for KI #2 of 3GPP TR 33.884.

# 2	References

[1]	3GPP TR 33.884: "Study on security of application enablement aspects for subscriber-aware northbound API access"

# 3	Rationale

This pCR proposes to add a new solution for KI #2 in TR 33.884 [1].

# 4	Detailed proposal

*************** Start of the Change ****************

## 6.Y	Solution #Y: User authorization revocation for API invocation procedure

### 6.Y.1	Introduction

This solution addresses the key issue #2 in terms of authorization revocation.

In this solution, UE controlled by the resource owner can revoke authorization information (e.g., access token, refresh token) that is stored in the CAPIF core function/authorization function.

UE controlled by the resource owner can trigger CAPIF core function/authorization function to inform API exposing function to store the information about revoked authorization information.

UE controlled by the resource owner can trigger CAPIF core function/authorization function to revoke authorization information that is stored in the API invoker.

### 6.Y.2	Solution details

The target UE is the UE controlled by the resource owner.

It is assume that API invoker is an UE/AF in SNA scenarios.

Figure 6.Y.2-x: User authorization revocation for API invocation procedure.

- The API invoker obtains the service API invocation authorization from the the resource owner. Specifically, the resource indicates the location information of the UE, QoS information of the UE, etc. And the API invoker is an UE/AF. Resource owner agent is part of the UE.

- The CAPIF core function/authorization function and the UE should do the mutual authentication.

For the case of CAPIF core function, the UE can authenticate the CAPIF core function via a certificate. Then the CAPIF core function can authenticate the UE using GBA-based authentication mechanism, AKMA-based authentication mechanism or certificate-based authentication mechanism, etc. And the CAPIF core function may generate a certificate for the UE after the API invoker onboarding procedure.

For the case of authorization function, the UE can authenticate the authorization function via a certificate. Then the authorization function can authenticate the UE using GBA-based authentication mechanism, AKMA-based authentication mechanism or certificate-based authentication mechanism, in which the certificate can be assigned by the CAPIF core function. UE and authorization function may do the mutual authentication based on methods that are defined in clause 6.5.2 of TS 33.122[5].

A secure connection between the UE and the CAPIF core function/authorization function is established after the mutual authentication. The secure connection can be established via TLS.

- The UE sends a service API authorization revocation request to the CAPIF core function/authorization function. The revocation is triggered by the resource owner. CAPIF core function/authorization identifies tokens that need to be revoked by checking the token type and token claim against the revocation request. The revocation request includes the information about the authorization information that needs to be revoked. Specifically, the revocation request includes the type of the token to be be revoked (e.g., refresh token, access token), the identity (e.g., GPSI, IMPI) of the API invoker, the identity (e.g., GPSI, IMPI) of the resource owner, the target resource (e.g., location of UE, QoS of the UE), and the target service API. Details on token, refresh token, and access token are given in RFC 6749 [4].

Editor's Note: How can the CCF ensure that the revocation request is originated from the user is FFS

- The CAPIF core function/authorization function sends the service API authorization revocation request to the API exposing function. The CAPIF core function/authorization function may delete locally stored tokens that are related to the service API authorization revocation request. If the request intends to revoke access tokens, the service API authorization revocation request is identical to the revocation request that is sent by the resource owner. If the request intends to revoke the refresh tokens, the CAPIF core function/authorization function should first identify access tokens that are related to the refresh tokens. Then the CAPIF core function/authorization function sends the API exposing function with identified access tokens via the service API authorization revocation request.

- Upon receiving the service API authorization revocation request, the API exposing function stores the token information in the message that is sent by the CAPIF core function/authorization function. The API exposing function should reject the request sent by the API invoker if the local stored token information indicates the API invoker is not allowed to request the resource.

- The API exposing function sends the service API invocation authorization revocation response to the CAPIF core function/authorization function. The response indicates that the API exposing function has stored the revocation request.

- Then CAPIF core function/authorization function sends the Service API invocation authorization revocation response to the UE. The response indicates that the authorization revocation has completed.

Editor's Note: token type in the revocation request is FFS.

### 6.Y.3	Evaluation

TBD

*************** End of the Change ****************